CECS 440 –MIPS Instruction Set Architecture

Benjamin Santos - 015780126

Naoaki Takatsu - 015746144

November 27, 2018

This page intentionally left blank

Table of Contents

Introduction ………...………………………………………………………………………	6

A. Memory Structure ………………………………………………………………………	7

B. Processor Register Set …………………………………………………………………..	9

C. Data Types …...………………………………………………………………………….	10

D. Addressing Modes ………………………………………………………………………	11

E. Instruction Set …………………………………………………………………………..	14
	1. R-Type Instructions ……………………………………………………………..	14

a. sll ………………………………………………………………………...	14

b. srl ………………………………………………………………………....16

c. sra ………………………………………………………………………..	18

d. jr .………………………………………………………………………...	20

e. mfhi ……………………………………………………………………...	22

f. mflo ……………………………………………………………………... 24

g. mult ……………………………………………………………………….26

h. div ……………………………………………………………………….	28

i. add ………………………………………………………………………..	30

j. addu ………………………………………………………………………	32

k. sub ……………………………………………………………………….	34

l. subu ………………………………………………………………………	36

m. and ………………………………………………………………………	38

n. or ………………………………………………………………………...	40

o. xor ……………………………………………………………………….	42

p. nor ……………………………………………………………………….	44

q. slt ………………………………………………………………………..	46

r. sltu ……………………………………………………………………….	48

s. break ……………………………………………………………………..	50

t. setie ………………………………………………………………………	51

2. I-Type Instructions ………………………………………………………………	52

a. beq ………………………………………………………………………	52

b. bne ………………………………………………………………………	54

c. blez ………………………………………………………………………	56

d. bgtz ………………………………………………………………………	58

e. addi ………………………………………………………………………	60

f. slti ………………………………………………………………………..	62

g. sltiu ………………………………………………………………………	64

h. andi ………………………………………………………………………	66

i. ori ………………………………………………………………………...	68

j. xori ……………………………………………………………………….	70

k. lui ………………………………………………………………………..	72

l. lw ………………………………………………………………………...	74

m. sw ……………………………………………………………………….	76

3. J-Type Instructions ……………………………………………………………...	78

a. j ………………………………………………………………………….	83

b. jal ………………………………………………………………………..	80

4. Enhanced Instructions …………………………………………………..…….....	82

a. input ……………………………………………………………………..	82

b. output ……………………………………………………………………	84

c. reti ……………………………………………………………………….	86

d. rotleft .. …………………….…………………………………………….	87

e. rotright ………………………………………………………………….. 89

F. Verilog Source Code …………………………………………………………..……......	91

a. Top Level Module ……………….………………………………………………	91

b. CPU ……………...………………………………………………………………	94

c. MIPS Control Unit (MCU)………………………………………………………	97

d. Instruction Unit (IU) …………………………………………………………….	121

e. Instruction Memory (iM) ………………………………………………………..	123

f. Integer Datapath (IDP) …………………………………………………………..	124

g. Regfile_32 ……………………………………………………………….	……...	126

h. alu_32 ……………………………………………………………………………	127

i. MIPS_32 …………………………………………………………………………	130

j. MPY_32 ………………………………………………………………………….	135

k. DIV_32 ………………………………………………………………………….	136

l. Barrel Shift ……………………………………………………………….	……...	137

m. Rotate_32 ……………………………………………………………………….	139

n. DataMemory …………………………………………………………………….	141

o. InputOutput ……………………………………………………………………...	142

G. Verilog Annotation ………………………………………………………..……............	143

a. Instruction Memory Module 1 …………………………………………………..	143

b. Instruction Memory Module 2 …………………………………………………..	144

c. Instruction Memory Module 3 …………………………………………………..	145

d. Instruction Memory Module 4 …………………………………………………..	146

e. Instruction Memory Module 5 …………………………………………………..	147

f. Instruction Memory Module 6 …………………………………………………..	148

g. Instruction Memory Module 7 …………………………………………………..	149

h. Instruction Memory Module 8 …………………………………………………..	150

i. Instruction Memory Module 9 …………………………………………………...	152

j. Instruction Memory Module 10 ………………………………………………….	154

k. Instruction Memory Module 11 …………………………………………………	156

l. Instruction Memory Module 12 ………………………………………………….	158

m. Instruction Memory Module 13 ………………………………………………...	159

n. Instruction Memory Module 14 …………………………………………………	161

o. Instruction Memory Module Enhanced …………………………………………	163

H. Hardware Block Diagram ………………………………………………..…….……….	165

a. CPU_test3 ……………………………………………………………………….	165

b. CPU ……………………………………………………………………………..	166

1. Integer Datapath ………………………………………………………… 167

2. Instruction Unit …………………………………………………………. 168

c. ALU_32 …………………………………………………………………………	169

d. Regfile_32 ……………………………………………………………………….	170

e.barrelSHIFT_32 ………………………………………………………………….	171

f. rotate_32 ………………………………………………………………………...	172

Introduction

This MIPS reduced instruction set architecture is specifically designed and created for educational purposes. The MIPS includes the following characteristics:

- non-pipeline multicycle data path

- 32 general purpose registers and 4 flag registers

- Harvard memory architecture granting bidirectional communication through 3 separate memory blocks

- 32-bit size instructions

- 5 major addressing modes to fetch operands at runtime

- Original enhanced key instructions

A. 	Memory Structure

This MIPS uses a 32-bit Harvard memory architecture. The Central Processing Unit (CPU) establishes separate bus communication between three memory units: Instruction Memory, Data Memory, and Input Output Memory. All memory sizes are set to 4098 x 8. Addresses are byte addressable (4-bit) with each address containing 8-bit data values, creating 32-bit data per address byte. The memory placement follows a big-endian format.

Ex.

0x1234_ABCD stored at 0x2258

| Address | Content |
| --- | --- |
| … | … |
| 0x225B | 0x12 |
| 0x226A | 0x34 |
| 0x2259 | 0xAB |
| 0x2258 | 0xCD |
| … | … |

B. 	Processor Register Set

32-bit Register File

| Name | Number | Use | Preserved Across a Call |
| --- | --- | --- | --- |
| $zero | 0 | Constant 0 | N/A |
| $at | 1 | Assembler Temporary | No |
| $v0-$v1 | 2-3 | Values for function results and expression evaluation | No |
| $a0-$a3 | 4-7 | Arguments | No |
| $t0-$t7 | 8-15 | Temporary | No |
| $s0-$s7 | 16-23 | Saved Temporary | Yes |
| $t8-$t9 | 24-25 | Temporary | No |
| $k0-$k1 | 26-27 | Reserved for OS Kernel | No |
| $gp | 28 | Global Pointer | Yes |
| $sp | 29 | Stack Pointer | Yes |
| $fp | 30 | Frame Pointer | Yes |
| $ra | 31 | Return Address | No |

Other Registers

| Register Name | Symbol | Size (bit) |
| --- | --- | --- |
| Carry | C | 1 |
| Negative | N | 1 |
| Zero | Z | 1 |
| Overflow | V | 1 |
| Program Counter | PC | 32 |
| Instruction Register | IR | 32 |
| rt Register | RT | 32 |
| rs Register | RS | 32 |
| Higher Register | HI | 32 |
| Lower Register | LO | 32 |
| Arithmetic Logic Unit Register | ALU_reg | 32 |
| Data In | D_in | 32 |

C. 	Data Types

Single Word Integer

- 16-bit integer for register and immediate type operands/results

Double Word Integer

- 32-bit integer for or immediate, multiplication, and division, results

D. 	Addressing Modes

Register

Use:

Register type instructions

Operand:

rs and rt

Ex.

If r2’s content is 0x0080 and 
r3’s content is 0x000F

r1 gets 0x0080 – 0x000F = 0x0071

Use:

Register type instructions

Operand:

rs and rt

Ex.

If r2’s content is 0x0080 and 
r3’s content is 0x000F

r1 gets 0x0080 – 0x000F = 0x0071

|  | Memory Addr | Instruction |  | Addr | Contents |
| --- | --- | --- | --- | --- | --- |
|  | 0x084C | sub r1, r2, r3 |  | 0x0844 | ----- |
|  |  |  |  | 0x0848 | ----- |
|  |  |  |  | 0x084C | opc r1 r9 r3 |
|  |  |  |  | 0x0850 | ----- |
|  |  |  |  | 0x0854 | ----- |
|  |  |  |  | 0x0858 | ----- |

Base (Indexed)

|  | Memory Addr | Instruction |  | Addr | Contents |
| --- | --- | --- | --- | --- | --- |
|  | 0x1108 | load r6, 0x0328(r2) |  | 0x1100 | ----- |
|  |  |  |  | 0x1104 | ----- |
|  |  |  |  | 0x1108 | opc r6 r2 base |
|  |  | Use: Load and store Operand: Result of adding 16-bit trailing word offset and content of rs register Ex. If r2’s content is 0x73C4: 0x0328 + 0x73C4 = 0x76EC So r6 gets 0x0021 Use: Load and store Operand: Result of adding 16-bit trailing word offset and content of rs register Ex. If r2’s content is 0x73C4: 0x0328 + 0x73C4 = 0x76EC So r6 gets 0x0021 |  | 0x110C | 0x0328 |
|  |  |  |  | 0x1110 | ----- |
|  |  |  |  | 0x1114 | ----- |
|  |  |  |  | … | … |
|  |  |  |  | 0x73C0 | ----- |
|  |  |  |  | 0x76EC | 0x0021 |
|  |  |  |  | 0x73C8 | ----- |

PC Direct

|  | Memory Addr | Instruction |  | Addr | Contents |
| --- | --- | --- | --- | --- | --- |
|  | 0x0928 | j 0x1000 |  | 0x0920 | ----- |
| Use: Jump type instructions Operand: 26-bit trailing word Ex. PC gets 0x1000 Use: Jump type instructions Operand: 26-bit trailing word Ex. PC gets 0x1000 |  |  |  | 0x0924 | ----- |
|  |  |  |  | 0x0928 | opc PC_dir |
|  |  |  |  | 0x092C | 0x1000 |
|  |  |  |  | 0x0930 | ----- |
|  |  |  |  | 0x0934 | ----- |
|  |  |  |  | … | … |
|  |  |  |  | 0x0FFC | ----- |
|  |  |  |  | 0x1000 | instr we'll jump to |
|  |  |  |  | 0x1004 | ----- |

PC Relative

|  | Memory Addr | Instruction |  | Addr | Contents |
| --- | --- | --- | --- | --- | --- |
|  | 0x0330 | beq r3, r0, 0xFF00 |  | 0x0224 | ----- |
| Use: Branch instructions If condition is true: PC PC + 4 + 16-bit address Otherwise: PC PC + 4 Ex. If r3’s content is 0x0000 PC gets PC + 4 + 0xFF00 = 0x0238 If r3’s content is 0x0001 PC gets PC + 4 = 0x0338 Use: Branch instructions If condition is true: PC PC + 4 + 16-bit address Otherwise: PC PC + 4 Ex. If r3’s content is 0x0000 PC gets PC + 4 + 0xFF00 = 0x0238 If r3’s content is 0x0001 PC gets PC + 4 = 0x0338 |  |  |  | 0x0238 | instr we'll jump to |
|  |  |  |  | 0x023C | ----- |
|  |  |  |  | … | … |
|  |  |  |  | 0x0328 | ----- |
|  |  |  |  | 0x032C | ----- |
|  |  |  |  | 0x0330 | opc r3 r0 PC_rel |
|  |  |  |  | 0x0334 | 0xFFF0 |
|  |  |  |  | 0x0338 | next instr |
|  |  |  |  | 0x033C | ----- |

Immediate

|  | Memory Addr | Instruction |  | Addr | Contents |
| --- | --- | --- | --- | --- | --- |
|  | 0x0230 | addi r0, r8, 0xA21C |  | 0x0228 | ----- |
|  |  |  |  | 0x022C | ----- |
|  |  | Use: Immediate type instructions Operand: 16-bit trailing word Ex. If r8’s content is 0x3019: r0 gets 0x3019 + 0xA21C = 0xD235 Use: Immediate type instructions Operand: 16-bit trailing word Ex. If r8’s content is 0x3019: r0 gets 0x3019 + 0xA21C = 0xD235 |  | 0x0230 | opc r0 r8 imm |
|  |  |  |  | 0x0234 | 0xA21C |
|  |  |  |  | 0x0238 | ----- |
|  |  |  |  | 0x023C | ----- |

E. 	Instruction Set

Shift Left Logical                                            sll

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | rt | rd | shtamt | 0x00 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	sll    $rd, $rt, shtamt

Purpose:

Execute logical shift left by specified shift amount.

Description: 	rd  rt << shtamt

This operation will execute a zero-fill logical shift left to a register $rt by shift amount shtamt and store the result into a destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rt << shtamt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Shift Left Logical - Example                                 sll

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0_0000 | 1_0010 | 1_0001 | 0_0010 | 00_0000 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s1 = 0x0000_0000

$s2 = 0x1234_5678

sll   $s1, $s2, 2

0001_0010_0011_0100_0101_0110_0111_1000 00

$s1  0x48D1_59E0

Shift Right Logical                                           srl

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | rt | rd | shtamt | 0x02 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	srl    $rd, $rt, shtamt

Purpose:

Execute logical shift right by specified shift amount

Description: 	rd  rt >> shtamt

This operation will execute a zero-fill logical shift right to a register $rt by shift amount shtamt and store the result into a destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rt >> shtamt

Exceptions:

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Shift Right Logical - Example                               srl

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0_0000 | 1_0001 | 1_0100 | 0_0001 | 00_0001 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s4 = 0x0000_0000

$s1 = 0x1010_1010

srl   $s4, $s1, 1

0 0001_0000_0001_0000_0001_0000_0001_0000

$s4  0x0808_0808

Shift Right Arithmetic                                      sra

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | rt | rd | shtamt | 0x03 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	sra    $rd, $rt, shtamt

Purpose:

Execute arithmetic shift right by specified shift amount

Description: 	rd  ( rt >> shtamt ) || 0x10000

This operation will execute an arithmetic logical shift right to a register $rt by shift amount shtamt and store the result into a destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

at  rt;

for( int i = 0; i < shtamt; i++ )

at  { rt(31), rt(31), at(30:1) };

rd  at;

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Shift Right Arithmetic - Example                           sra

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0_0000 | 1_0010 | 1_0011 | 0_0011 | 00_0011 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s3 = 0x0000_0000

$s2 = 0xFEFA_1934

sra   $s3, $s2, 3

1 111 111_1110_1111_1010_0001_1001_0011_0100

$s3  0xFFDF_4326

Jump Register                                                 jr

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | 0x00 | 0x00 | 0x00 | 0x08 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	jr    $rs

Purpose:

Jump to specified address

Description: 	PC  rs

This operation will execute a jump with its target address set to $rs.

Restrictions:

- None

Operation:

PC  rs

Exceptions:

- None

Programming Notes:

None

Jump Register - Example                                      jr

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0110 | 0_0000 | 0_0000 | 0_0000 | 00_1000 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s6 = 0xABBA_CCDD

jr   $s6

PC  0xABBA_CCDD

Move from HI                                            mfhi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | 0x00 | rd | 0x00 | 0x10 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	mfhi    $rd

Purpose:

Copy content of special register HI and store it to a register

Description: 	rd  HI

This operation will copy data from special register $HI and store it into destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  HI

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Move from HI - Example                                 mfhi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0_0000 | 0_0000 | 0_1111 | 0_0000 | 01_0000 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t7 = 0x0000_0000

$HI = 0x0095_FFE5

mfhi   $t7

$t7  0x0095_FFE5

Move from LO                                           mflo

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | 0x00 | rd | 0x00 | 0x12 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	mflo    $rd

Purpose:

Copy content of special register LO and store it to a register

Description: 	rd  LO

This operation will copy data from special register $LO and store it into destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  LO

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Move from LO - Example                                mflo

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0_0000 | 0_0000 | 1_0011 | 0_0000 | 01_0010 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s3 = 0x0000_0000

$LO = 0xCD12_8876

mflo   $s3

$s3  0xCD12_8876

Multiply                                                      mult

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | 0x00 | 0x00 | 0x18 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	mult     $rs, $rt

Purpose:

Multiply two registers with each other and store result

Description: 	{ HI, LO }  rs * rt

This operation will multiply $rs by $rt and store the upper 32 bit result to special register $HI and lower 32 bit result to special register $LO.

Restrictions:

- None

Operation:

{ HI, LO }  rs * rt

Exceptions:

- Negative flag will be affected by $HI’s parity bit

- Zero flag will be set if $HI and $LO are 0x0

Programming Notes:

Performs multiplication on signed values

Multiply - Example                                           mult

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0001 | 1_0101 | 0_0000 | 0_0000 | 01_1000 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s1 = 0x3D65_9900

$s5 = 0x113B_0008

mult   $s1, $s5

$s1 * $s5 = 0x0421_E595_2E2C_C800

$HI  0x0421_E595

$LO  0x2E2C_C800

Divide                                                          div

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | 0x00 | 0x00 | 0x1A |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	div    $rs, $rt

Purpose:

Divide a dividend register by a divisor register and store the result to a register

Description: 	{ HI, LO }  rs / rt

This operation will divide $rs by $rt and store the result into register $rd.

Restrictions:

- rt cannot be 0x0

Operation:

LO  rs / rt

HI  rs % rt

Exceptions:

- Negative flag will be affected by $LO’s parity bit

- Overflow flag will be set if rt is 0x0

- Zero flag will be set if $HI and $LO are 0x0

Programming Notes:

Performs division on signed values. Division by zero will set an overflow flag and may result in unpredictable value for HI and LO.

Divide - Example                                               div

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0001 | 1_0111 | 0_0000 | 0_0000 | 01_1010 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s1 = 0x0228_1FBC

$s7 = 0x000C_1FA4

div   $s1, $s7

$s1 % $s7 = 0x0006_8FE8

$s1 / $s7 = 0x0000_002D

$HI  0x0006_8FE8

$LO  0x0000_002D

Signed Add - Example                                         add

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x20 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	add    $rd, $rs, $rt

Purpose:

Signed add two registers with each other and store the result to a register

Description: 	rd  rs + rt

This operation will signed add $rs to $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

{ cf, rd }  rs + rt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Carry flag will be set if cf is a 1’b1

- Overflow flag will be set for the following two cases:

- positive + positive = negative

- negative + negative = positive

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Signed Add - Example                                        add

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0010 | 1_0100 | 0_1000 | 0_0000 | 10_0000 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t0 = 0x0000_0001

$s2 = 0x7111_0943

$s4 = 0x39BB_3A23

add   $t0, $s2, $s4

$s2 + $s4 = 0xAACC_4366

$t0  0xAACC_4366

Unsigned Add                                                addu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x21 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	addu    $rd, $rs, $rt

Purpose:

Unsigned add two registers with each other and store the result to a register

Description: 	rd  rs + rt

This operation will unsigned add $rs to $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rs + rt

Exceptions:

- Carry flag will be set if cf is a 1’b1

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Unsigned Add – Example                                    addu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0001 | 1_0010 | 1_0000 | 0_0000 | 10_0001 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s0 = 0x0001_0001

$s1 = 0x9900_891D

$s2 = 0x00FD_AA51

addu   $s0, $s1, $s2

$s1 + $s2 = 0x99FE_336E

$s0  0x99FE_336E

Signed Subtract                                                sub

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x22 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	sub    $rd, $rs, $rt

Purpose:

Signed subtract two registers with each other and store the result to a register

Description: 	rd  rs - rt

This operation will signed subtract $rs by $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

{ cf, rd }  rs - rt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Carry flag will be set if cf is a 1’b1

- Overflow flag will be set for the following two cases:

- positive - negative = negative

- negative - positive = positive

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Signed Subtract – Example                                    sub

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0011 | 1_0100 | 0_1000 | 0_0000 | 10_0010 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t0 = 0x0000_0000

$s3 = 0x0111_341B

$s4 = 0x8149_33EF

sub   $t0, $s3, $s4

$s3 - $s4 = 0x7FC8_002C

$t0  0x7FC8_002C

Unsigned Subtract                                            subu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x23 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	subu    $rd, $rs, $rt

Purpose:

Unsigned subtract two registers with each other and store the result to a register

Description: 	rd  rs - rt

This operation will unsigned subtract $rs by $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rs - rt

Exceptions:

- Carry flag will be set if cf is a 1’b1

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Unsigned Subtract - Example                            subu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0000 | 1_0101 | 0_1011 | 0_0000 | 10_0011 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t3 = 0x0000_0000

$s0 = 0xFFFF_FFFF

$s5 = 0x341F_DD89

subu   $t3, $s0, $s5

$s0 - $s5 = 0xCBE0_2276

$t3  0xCBE0_2276

And                                                                                        and

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x24 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	and    $rd, $rs, $rt

Purpose:

Bitwise and two registers with each other and store the result to a register

Description: 	rd  rs and rt

This operation will bitwise and $rs with $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rs & rt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

And – Example                                                                     and

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0000 | 1_0010 | 1_0110 | 0_0000 | 10_0100 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s6 = 0x0000_0000

$s0 = 0x0123_FFFF

$s2 = 0x6745_34AB

and   $s6, $s0, $s2

$s0 & $s2 = 0x0101_34AB

$s6  0x0101_34AB

Or                                                                                              or

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x25 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	or    $rd, $rs, $rt

Purpose:

Bitwise or two registers with each other and store the result to a register

Description: 	rd  rs or rt

This operation will bitwise or $rs with $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rs | rt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Or – Example                                                                            or

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0001 | 1_0010 | 1_0100 | 0_0000 | 10_0101 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s4 = 0x0000_0000

$s1 = 0xFFFF_76BE

$s2 = 0x9234_5377

or   $s4, $s1, $s2

$s1 | $s2 = 0x9234_5256

$s4  0x9234_5256

Exclusive Or                                                                          xor

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x26 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	xor    $rd, $rs, $rt

Purpose:

Exclusive or two registers with each other and store the result to a register

Description: 	rd  rs xor rt

This operation will exclusive or $rs with $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rs ^ rt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Exclusive Or - Example                                                         xor

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0011 | 1_0101 | 1_0000 | 0_0000 | 10_0110 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s0 = 0x0000_0000

$s3 = 0xFFFF_76BE

$s5 = 0x9234_5377

xor   $s0, $s3, $s5

$s3 ^ $s5 = 0x6DCB_25C9

$s0  0x6DCB_25C9

Not Or                                                                                    nor

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x27 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	nor    $rd, $rs, $rt

Purpose:

Not or two registers with each other and store the result to a register

Description: 	rd  rs nor rt

This operation will not or $rs with $rt and store the result into register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  ! ( rs | rt )

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Not Or – Example                                                                  nor

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0010 | 1_0011 | 1_0010 | 0_0000 | 10_0111 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s2 = 0x0126_FEF3

$s3 = 0x9234_5377

nor   $s2, $s2, $s3

~($s2 | $s3) = 0x6CC9_0008

$s2  0x6CC9_0008

Signed Set if Less Than                                                           slt

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x2A |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	slt    $rd, $rs, $rt

Purpose:

Check if a register is less than another register and store the result to a register (signed)

Description: 	rd  rs < rt

This operation will check if $rs is less than $rt and store the result into register $rd (signed).

Restrictions:

- rd cannot be a zero register

Operation:

if ( ( rs(31) == 1’b1 ) && ( rt(31) == 1’b0 ) )

rd  32’h1

if ( ( rs(31) == 1’b0 ) && ( rt(31) == 1’b1 ) )

rd  32’h0

else ( rs(31) == rt(31) )

rd   rs(30:0) < rt(30:0)

Exceptions:

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Signed Set if Less Than - Example                                          slt

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0000 | 1_0100 | 0_1001 | 0_0000 | 10_1010 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t1 = 0x0000_0000

$s0 = 0x7563_FABB

$s4 = 0xF444_0000

slt   $t1, $s0, $s4

$s0 < $s4 = 0x0

$t1  0x0000_0000

Unsigned Set if Less Than                                                    sltu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | rs | rt | rd | 0x00 | 0x2B |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	sltu    $rd, $rs, $rt

Purpose:

Check if a register is less than another register and store the result to a register (unsigned)

Description: 	rd  rs < rt

This operation will check if $rs is less than $rt and store the result into register $rd (unsigned).

Restrictions:

- rd cannot be a zero register

Operation:

rd   rs < rt

Exceptions:

- Zero flag will be set if $rd is 0x0

Programming Notes:

None

Unsigned Set if Less Than - Example                                   sltu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 1_0010 | 1_0100 | 0_1010 | 0_0000 | 10_1011 |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t2 = 0x0000_0000

$s2 = 0x4412_0098

$s4 = 0x993B_AABC

sltu   $t1, $s0, $s4

$s2 < $s3 = 0x1

$s2  0x0000_0001

Break                                                                                      break

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x0D |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	break

Purpose:	The halt operation of the MIPS Processor

Description:

The instruction stops all the operations of the processor.

Restrictions:

None

Operation:

When the control unit detects 0x0D on the last 6 bits of the Instruction Register, all operation of the Processor stops.

Exceptions:

None

Programming Notes:

None

Set Interrupt Enable                                                            setie

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | 0x00 | 0x00 | 0x00 | 0x1F |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	setie

Purpose:	Enables the processor to check for Interrupts.

Description:

The instruction sets the processor to check for Interrupts

Restrictions:

None

Operation:

When the control unit detects 0x1F on the last 6 bits of the Instruction Register, the processor is activated to detect Interrupts.

Exceptions:

None

Programming Notes:

None

Branch on Equal                                                                                                    beq

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0100 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	beq    $rt, $rs, ‘branch address’

Purpose:	Conditional Branch that jumps to the 16-bit immediate value address.

Description: 	if(rs == rt), branch

If the contents of register $rs and register $rt are equal, branch to the target address ’16-bit immediate value’.

Restrictions:

None

Operation:

if(rs == rt)
		PC  PC + 4 + ’16-bit immediate value’

Exceptions:

None

Programming Notes:

None

Branch on Equal – Example					                      beq

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0100 | 0_0111 | 0_0110 | 1111_1111_1111_1100 |
| --- | --- | --- | --- |

6		   5		     5			   16

beq	$r7, $r6, Label

R[r7] =  0x7070_7070

R[r6] =  0x7070_7070

Label = 0xFFFC

Since $rs is equal to $rt, PC  0xFFFC

Branch not Equal                                                                                                    bne

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0101 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	bne    $rt, $rs, ‘branch address’

Purpose:	Conditional Branch that jumps to the 16-bit immediate value address.

Description: 	if(rs != rt), branch

If the contents of register $rs and register $rt are not equal, branch to the target address ’16-bit immediate value’.

Restrictions:

None

Operation:

if(rs != rt)
		PC ’16-bit immediate value’

Exceptions:

None

Programming Notes:

None

Branch not Equal – Example				  	                      bne

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0101 | 0_0010 | 0_0011 | 0000_0000_0111_1100 |
| --- | --- | --- | --- |

6		   5		     5			   16

bne	$r2, $r3, Label

R[r7] =  0x7070_7070

R[r6] =  0xFFFF_ABCD

Label = 0x007C

Since $rs is not equal to $rt, PC  0x007C

Branch on Less Than or Equal to Zero                                                blez

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0110 | rs | 0_0000 | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	blez		$rs, ‘branch address’

Purpose:	Conditional Branch that jumps to the 16-bit immediate value address.

Description: 	if(rs  0), branch

If the value of register $rs is less than or equal to zero, branch to the target address 
’16-bit immediate value’.

Restrictions:

None

Operation:

if(rs  0)
		PC ’16-bit immediate value’

Exceptions:

None

Programming Notes:

None

Branch on Less Than or Equal to Zero – Example  	        blez

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0110 | 0_0111 | 0_0000 | 0000_0111_0000_0101 |
| --- | --- | --- | --- |

6		   5		     5			   16

blez	$r1, Label

R[r1] =  0xF010_1010

Label = 0x0705

Since $r1 is less than zero, PC  0x0705

Branch on Greater Than Zero                                                                    bgtz

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0111 | rs | 0x00 | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	bgtz    $rs, ‘branch address’

Purpose:	Conditional Branch that jumps to the 16-bit immediate value address.

Description: 	if(rs  0), branch

If the value of register $rs is greater than zero, branch to the target address 
’16-bit immediate value’.

Restrictions:

None

Operation:

if(rs  0)
		PC ’16-bit immediate value’

Exceptions:

None

Programming Notes:

None

Branch on Greater than Zero – Example			       bgtz

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0111 | 0_0111 | 0_0000 | 1111_1111_1111_1100 |
| --- | --- | --- | --- |

6		   5		     5			   16

bgtz	$r7, Label

R[r7] =  0x0727_2727

Label = 0x0027

Since $rs is greater than zero, PC  0x0027

Add Immediate                                                                                                      addi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1000 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	addi    $rt, $rs, ’16-bit immediate value’

Purpose:	Add instruction that takes a 16-bit immediate value, add it with register $rs and store to register $rt

Description: 	rt  rs + ‘16-bit immediate’

The operation adds the register $rt with the 16-bit immediate value and store the result 
into the register $rs.

Restrictions:

None

Operation:

rt = rs + ’16-bit immediate value’

Exceptions:

- Integer Overflow Flag is affected if result is wrong.

- Carry Flag is affected if result exceeds the max 32-bit value (0xFFFF_FFFF)

Programming Notes:

None

Add Immediate – Example				                               addi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_0100 | 0_0011 | 0_0101 | 1111_1111_1110_1110 |
| --- | --- | --- | --- |

6		   5		     5			   16

addi	$r5, $r3, 0x0002

R[r5] =  …

R[r3] = 0xFEEB_1234

Immediate Value = 0x0002

R3  -> FEEB_1234
Imm -> 0000_0002 +
R5  -> FEEB_1236

Register $r5 gets the value 0xFEEB_1236

R[r5] = 0xFEEB_1236

Set Less Than Immediate                                                                                   slti

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1010 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	slti    $rt, $rs, ’16-bit immediate value’

Purpose:	Boolean operation that checks if the register $rs is less than the ’16-bit immediate value’ and sets register $rt to 1 if it is and 0 otherwise.

Description: 	rt  if(rs < ’16-bit immediate value’)

If register $rs is less than the ’16-bit immediate value’, register $rt is 1, otherwise 0.

Restrictions:

None

Operation:

if(rs < ’16-bit immediate value’)

rt = 1

else

rt = 0

Exceptions:

None

Programming Notes:

None

Set Less Than Immediate – Example			                        slti

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1010 | 0_0010 | 0_0001 | 0111_1111_1111_1100 |
| --- | --- | --- | --- |

6		   5		     5			   16

slti	$r2, $r1, 0x7FFC

R[r2] = …

R[r1] =  0x0000_2070

Immediate Value = 0x7FFC

Since value of register $r1 is than the immediate value, register $r2 gets 32-bit value of 1

R[r2] = 0x0000_0001

Set Less Than Immediate Unsigned                                                      sltiu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1011 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	sltiu    $rt, $rs, ’16-bit immediate value’

Purpose:	Boolean operation that checks if the register $rs is less than the ’16-bit immediate value’ and sets register $rt to 1 if it is and 0 otherwise. (UNSIGNED)

Description: 	rt  if(rs < ’16-bit immediate value’)

If register $rs is less than the ’16-bit immediate value’, register $rt is 1, otherwise 0.

Restrictions:

None

Operation:

if(rs < ’16-bit immediate value’)

rt = 1

else

rt = 0

Exceptions:

None

Programming Notes:

None

Set Less Than Immediate Unsigned – Example		       sltiu

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1011 | 0_0011 | 0_0001 | 0111_0111_0111_0111 |
| --- | --- | --- | --- |

6		   5		     5			   16

sltiu $r3, $r1, 0x7777

R[r3] = …

R[r1] =  0x0000_2070

Immediate Value = 0x7777

Since value of register $r1 is than the immediate value, register $r3 gets 32-bit value of 1

R[r3] = 0x0000_0001

AND Immediate                                                                                                   andi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1100 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	andi    $rt, $rs, ’16-bit immediate value’

Purpose:	Logical AND between register $rs and the ’16-bit immediate value’

Description: 	rt  rs AND ’16-bit immediate value’

The operation does a bitwise logical AND between register $rs and the zero extended to the left ’16-bit immediate value’ and stores the result into register $rt

Restrictions:

None

Operation:

rt = rs & {16’h0, ’16-bit immediate value’}

Exceptions:

None

Programming Notes:

None

AND Immediate – Example                                                                        andi

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1100 | 0_0001 | 0_0111 | 0111_0010_0111_0010 |
| --- | --- | --- | --- |

6		   5		     5			   16

andi	$r7, $r1, 0x7272

R[r7] = …

R[r1] =  0x5A5A_5A5A

Immediate Value = 0x7272

R1  -> 5A5A_5A5A
Imm -> 0000_7272 AND
R7  -> 0000_5252

Register $r7 gets the value 0x0000_5252

R[r7] = 0x0000_5252

OR Immediate                                                                                                              ori

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1101 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	ori    $rt, $rs, ’16-bit immediate value’

Purpose:	Logical OR between register $rs and the ’16-bit immediate value’

Description: 	rt  rs OR ’16-bit immediate value’

The operation does a bitwise logical OR between register $rs and the ’16-bit immediate 	value’ and stores the result into register $rt

Restrictions:

None

Operation:

rt = rs | {16’h0, ’16-bit immediate value’}

Exceptions:

None

Programming Notes:

None

OR Immediate – Example                                                                                  ori

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1101 | 0_0010 | 0_0101 | 0100_1011_0100_1011 |
| --- | --- | --- | --- |

6		   5		     5			   16

ori	$r5, $r2, 0x2B2B

R[r5] = …

R[r2] =  0xB2B2_B2B2

Immediate Value = 0x2B2B

R2  -> B2B2_B4B4
Imm -> 0000_4B4B OR
R5  -> 0000_FFFF

Register $r5 gets the value 0x0000_FFFF

R[r5] = 0x0000_FFFF

Exclusive OR Immediate                                                                              xori

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1110 | Rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	xori    $rt, $rs, ’16-bit immediate value’

Purpose:	Logical XOR between register $rs and the ’16-bit immediate value’

Description: 	rt  rs XOR ’16-bit immediate value’

The operation does a bitwise logical XOR between register $rs and the ’16-bit immediate 	value’ and stores the result into register $rt

Restrictions:

None

Operation:

rt = rs ^ {16’h0, ’16-bit immediate value’}

Exceptions:

None

Programming Notes:

None

Exclusive OR Immediate – Example                                                    xori

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1110 | 0_1000 | 0_0010 | 0000_0000_1111_1111 |
| --- | --- | --- | --- |

6		   5		     5			   16

xori    $r2, $r8, 0x00FF

R[r2] = …

R[r8] =  0x2F2F_1111

Immediate Value = 0x00FF

R8  -> 2F2F_3333
Imm -> 0000_00FF XOR
R2  -> 2F2F_33CC

Register $r2 gets the value 0x2F2F_33CC

R[r2] = 0x2F2F_33CC

Load Upper Immediate                                                                                        lui

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1111 | 0_0000 | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	lui    $rt, ’16-bit immediate value’

Purpose:	Loads register $rt with the 16-bit immediate value on the upper 16 bits

Description: 	rt  {’16-bit immediate value, 16’h0}

The operation stores the 16-bit immediate value and 16 bits of zeros into the register $rt.

Restrictions:

None

Operation:

rt = rs & {’16-bit immediate value’, 16’h0}

Exceptions:

None

Programming Notes:

None

Load Upper Immediate – Example                                                             lui

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 00_1111 | 0_0000 | rt | 0111_1111_1111_0111 |
| --- | --- | --- | --- |

6		   5		     5			   16

lui    $r3, 0x7FF7

R[r3] = …

Immediate Value = 0x7FF7

Register $r3 loads the immediate value as the upper bits and zero fill.

R[r3] = 0x7FF7_0000

Load Word                                                     lw

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 10_0011 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	lw    $rt, ’16-bit immediate value’($rs)

Purpose:	Loads word from memory into a register

Description: 	rs is the base address

rt  M[rs + 16-bit immediate value]

The operation loads the value from a specific memory address specified by the base(register $rs) + the offset(’16-bit immediate value’) into the register $rt

Restrictions:

None

Operation:

rt = M[rs + 16-bit immediate value]

Exceptions:

None

Programming Notes:

None

Load Word – Example                                       lw

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 10_0011 | 0_0010 | 0_0001 | 0000_0000_0000_0100 |
| --- | --- | --- | --- |

6		   5		     5			   16

lw	$r1, 0x0004($r2)

R[r1] = …

R[r2] = 0x2111_FBC0

Immediate Value = 0x0004

r2  -> 2111_FBC0
Imm -> 0000_0004 +
Adr -> 2111_FBC4

| 2111_FBC0 | ……… |
| --- | --- |
| 2111_FBC4 | 1234_5678 |
| 2111_FBC8 | ……... |

Register $r1 loads the value from memory of address [0x2111_FBC4]

R[r1] = 0x1234_5678

Store Word                                                   sw

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 10_1011 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	sw    $rt, ’16-bit immediate value’($rs)

Purpose:	Stores word from register into memory

Description: 	rs is the base address

M[rs + 16-bit immediate value] rt

The operation stores the value from a register $rt into a specific memory address specified by the base(register $rs) + the offset(’16-bit immediate value’)

Restrictions:

None

Operation:

M[rs + 16-bit immediate value] = rt

Exceptions:

None

Programming Notes:

None

Store Word – Example                                       sw

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 10_1011 | 0_0111 | 0_0110 | 0000_0000_0000_1000 |
| --- | --- | --- | --- |

6		   5		     5			   16

sw	$r6, 0x1234($r7)

R[r6] = 0x2222_7777

R[r7] = 0xB0B0_1234

Immediate Value = 0x0008

r7  -> B0B0_1234
Imm -> 0000_0008 +
Adr -> B0B0_123C

Register $r6 is stored into the Memory Address (B0B0_123C)

| B0B0_1238 | ……… |
| --- | --- |
| B0B0_123C | 2222_7777 |
| B0B0_123F | ……... |

M[Adr] = 0x2222_7777

Jump                                                               j

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| J J 00_0010 | 26-bit Jump Address |
| --- | --- |

6						     26

Format: 	j    ’26-bit Jump Address’

Purpose:	Unconditional Jump to the 26-bit Jump Address

Description: 	PC  ’26-bit Jump Address’

Jumps to the target address specified by the ’26-bit Jump Address’

Restrictions:

None

Operation:

PC = ’26-bit Jump Address’

Exceptions:

None

Programming Notes:

None

Jump  - Example                                                j

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| J J 00_0010 | 10_1110_1101_1100_1011_1010_1000 |
| --- | --- |

6						     26

j	0x02ED_CBA8

Program Counter changes

Program Counter = 0x02ED_CBA8

|  | 02ED_CBA4 | ……… |
| --- | --- | --- |
| PC | 02ED_CBA8 | ……… |
|  | 02ED_CBAC | ……… |

Jump and Link                                                jal

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| J J 00_0011 | 26-bit Jump Address |
| --- | --- |

6						     26

Format: 	jal    ’26-bit Jump Address’

Purpose:	Unconditional Jump to the 26-bit Jump Address

Description: 	PC  ’26-bit Jump Address’

Jumps to the target address specified by the ’26-bit Jump Address’

Restrictions:

None

Operation:

PC = ’26-bit Jump Address’

Exceptions:

None

Programming Notes:

None

Jump and Link – Example                                    jal

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| J J 00_0011 | 26-bit Jump Address |
| --- | --- |

6						     26

jal	0x02ED_CBA8

Program Counter changes

Program Counter = 0x02ED_CBA8

|  | 02ED_CBA4 | ……… |
| --- | --- | --- |
| PC | 02ED_CBA8 | ……… |
|  | 02ED_CBAC | ……… |

Input	                                                        input

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 01_1100 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	input    $rt, ’16-bit immediate value’($rs)

Purpose:	Loads word from I/O into a register

Description: 	rs is the base address

rt  IO[rs + 16-bit immediate value]

The operation loads the value from a specific IO memory address specified by the base(register $rs) + the offset(’16-bit immediate value’) into the register $rt

Restrictions:

None

Operation:

rt = IO[rs + 16-bit immediate value]

Exceptions:

None

Programming Notes:

None

Input – Example                                         input

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 01_1100 | 1_0001 | 1_0000 | 0000_0000_0000_1000 |
| --- | --- | --- | --- |

6		   5		     5			   16

input	$s0, 0x0008($s1)

R[s1] = 0x1111_2224

Immediate Value = 0x0008

s1  -> 1111_2224
Imm -> 0000_0008 +
Adr -> 1111_222C

| 1111_2228 | ……… |
| --- | --- |
| 1111_222C | FFFE_ABCD |
| 1111_2230 | ……... |

Register $s0 loads the value from the IO memory of address [0x1111_222C]

R[s0] IO[0x1111_222C]

R[s0] 0xFFFE_ABCD

Output  									     output

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 01_1101 | rs | rt | 16-bit immediate |
| --- | --- | --- | --- |

6		   5		     5			   16

Format: 	output    $rt, ’16-bit immediate value’($rs)

Purpose:	Stores word from register into the I/O

Description: 	rs is the base address

IO[rs + 16-bit immediate value] rt

The operation stores the value from a register $rt into a specific IO memory address specified by the base(register $rs) + the offset(’16-bit immediate value’)

Restrictions:

None

Operation:

IO[rs + 16-bit immediate value] = rt

Exceptions:

None

Programming Notes:

None

Output – Example                                     output

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| I I 01_1101 | 0_1000 | 0_1100 | 0000_0000_0001_0000 |
| --- | --- | --- | --- |

6		   5		     5			   16

output	$t4, 0x0010($t0)

R[t0] = 0x001F_EEDC

R[t4] = 0xB0B0_B0B0

Immediate Value = 0x0010

t0  -> 001F_EEDC
Imm -> 0000_0010 +
Adr -> 001F_EEEC

Register $r6 is stored into the Memory Address (B0B0_123C)

| 001F_EEE8 | ……… |
| --- | --- |
| 001F_EEEC | B0B0_B0B0 |
| 001F_EEF0 | ……... |

M[0x001F_EEEC]  0xB0B0_B0B0

Return Interrupt                                                                                                        reti

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| J J 01_1110 | 00_0000_0000_0000_0000_0000_0000 |
| --- | --- |

6						     26

Format: 	reti

Purpose:	Jumps back to the original PC before the jump to the Interrupt Service Routine

Description:

The operation takes the value from the top of stack (M[0x404]), and updates the status flags to the original flags before the Interrupt, decrements stack pointer by 4, then takes the value from the new top of stack (M[0x400]), and jumps to that address, and decrements one more time by 4, and update the stack pointer to 0x3FC.

It takes 6 clock cycles to complete the operation.

Restrictions:

None

Operation:

- ALU_Out  RS(0x404)

- D_in  dM[ALU_Out(0x404)] ,

- I, C, V, N, Z  D_in(28’b0, intr, C, V, N, Z),  
ALU_Out  RS(0x404) – 4, 
RS  RS(0x404) - 4

- D_in  dM[ALU_Out(0x400)] ,
ALU_Out RS(0x400) – 4,

- PC  D_in (dM[0x400])

- $sp ALU_Out(0x3FC)

Exceptions:

None

Programming Notes:

None

Rotate Left                                                   rotleft

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | rt | rd | shtamt | 0x2C |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	rotleft    $rd, $rt, shtamt

Purpose:

Execute logical rotate left by specified shift amount.

Description: 	rd  rt << shtamt

This operation will execute a rotate left to a register $rt by shift amount shtamt and store the result into a destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rt << shtamt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x00

- Compared to Shift Left Logical (SLL), this operation does not affect the carry flag

Programming Notes:

None

Rotate Left – Example                                 rotleft

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0000_0 | 0_1001 | 0_1001 | 0_0010 | 0x2C |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t1 = 0xFEDC_BA98

rotleft 	$t1, $t1, 2

1111_1110_1101_1100_1011_1010_1001_1000 11

$t1  0xFB72_EA63

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0000_0 | 0_1100 | 0_1000 | 0_0110 | 0x2C |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$t4 = 0xABCD_EF01

rotleft 	$t0, $t4, 6

1010_1011_1100_1101_1110_1111_0000_0001 1010_10

$t0  0xF37B_C06A

Rotate Right                                                rotright

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 0x00 | 0x00 | rt | rd | shtamt | 0x2D |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

Format: 	rotright    $rd, $rt, shtamt

Purpose:

Execute logical rotate right by specified shift amount.

Description: 	rd  rt >> shtamt

This operation will execute a rotate right to a register $rt by shift amount shtamt and store the result into a destination register $rd.

Restrictions:

- rd cannot be a zero register

Operation:

rd  rt >> shtamt

Exceptions:

- Negative flag will be affected by $rd’s parity bit

- Zero flag will be set if $rd is 0x00

- Compared to Shift Right Logical (SRL), this operation does not affect the carry flag

Programming Notes:

None

Rotate Right – Example                              rotright

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0000_0 | 1_0000 | 1_0001 | 0_0100 | 0x2D |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s0 = 0x0000_00CD

rotright 	$s1, $s0, 4

1101_0000_0000_0000_0000_0000_0000_1100_1101

$s1  0xD000_000C

31 30 29 28 27 26 25 24 23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1 0

| R R 00_0000 | 0000_0 | 1_0100 | 1_0000 | 0_0110 | 0x2D |
| --- | --- | --- | --- | --- | --- |

6		   5		     5		 5	     5        6

$s4 = 0x1234_5678

rotright 	$s0, $s4, 8

0111_1000_0001_0010_0011_0100_0101_0110_0111_1000

$s0  0x7812_3456

III. Verilog Implementation

- Top Level Module

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  CPU_test.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.3

* Rev. Date:  11/8/2018

*

* Purpose:    This is the testbench that will instantiate and verify the

*             correctness of the Central Processing Unit(CPU) with the Data

*             Memory module and the InputOutput.v module.

*

* Notes:      This module will set an interrupt at 300ns in the middle of

*             the Finite State Machine to test the validity of the

*             interrupt, and goes back to the original Program Counter

*             before the interrupt occurs.

*

* Revision Notes:

*             1.2: (Revised from CPU_test1.v testbench module)

*                  Added IO module for testing the Input and Output

*                  instructions

*             1.3: Changed to work for iMem 14module CPU_test1;

*

****************************************************************************/ 
module CPU_test3;

// Inputs

reg clk, reset;               //Clock and Reset

reg intr;                     //Interrupt Request

// Outputs

wire int_ack;                 //Interrupt Acknowledge Flag

wire [31:0] D_MemToInt;       //Data from DM to Datapath

wire [31:0] D_IOToInt;        //Data from IO to Datapath

wire [31:0] D_MemOrIO, Addr;  //Data Output from Datapath

wire dm_cs, dm_wr, dm_rd;     //Data Memory flags

wire io_cs, io_wr, io_rd;     //IO module flags

// Instantiate the Unit Under Tests

CPU            cpu(

.clk(clk),  .reset(reset),   //Clock and Reset

.intr(intr),                 //Interrupt Request

.int_ack(int_ack),           //Interrupt Acknowledge

.D_MemToInt(D_MemToInt),     //Data Input from Memory

.D_IOToInt(D_IOToInt),       //Data Input from IO

.Addr(Addr),                 //Data Address output of CPU

.D_OUT(D_MemOrIO),           //Data Output for Memory or IO

.dm_cs(dm_cs),               //  \

.dm_rd(dm_rd),               // --> Data Memory Flags

.dm_wr(dm_wr),               //  /

.io_cs(io_cs),               //  \

.io_rd(io_rd),               // --> I/O Module Flags

.io_wr(io_wr)                //  /

);

DataMemory  data_mem (

.clk(clk),                   //Clock

.Address( { 20'b0,           //Only takes first 12 bits

Addr[11:0]}),    // of Address

.D_In(D_MemOrIO),            //Data Input from CPU

.D_Out(D_MemToInt),          //Data Output to CPU

.dm_cs(dm_cs),               //  \

.dm_rd(dm_rd),               // --> Data Memory Flags

.dm_wr(dm_wr)                //  /

);

InputOutput in_out (

.clk(clk),                   //Clock

.Address( { 20'b0,           //Only takes first 12 bits

Addr[11:0]}),    //of Address

.D_In(D_MemOrIO),            //Data Input from CPU

.D_Out(D_IOToInt),           //Data Output to CPU

.io_cs(io_cs),               //  \

.io_rd(io_rd),               // --> Data Memory Flags

.io_wr(io_wr),               //  /

.int_ack(int_ack)            //Interrupt Acknowledge

);

// Create a 10 ns clock

always #5 clk = ~clk;

initial begin

// Initialize Inputs

clk = 0;

reset = 1;

intr = 0;

$timeformat( -9, 1, " ps", 9 );  //Display time in nanoseconds

// store .dat files into their respective memories (Data and Instruction)

//	$readmemh( "dMem_1.dat", data_mem.M ); 	//Test 1

//    	$readmemh( "iMem_1.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_2.dat", data_mem.M ); 	//Test 2

//    	$readmemh( "iMem_2.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_3.dat", data_mem.M ); 	//Test 3

//   	$readmemh( "iMem_3.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_4.dat", data_mem.M ); 	//Test 4

//    	$readmemh( "iMem_4.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_5.dat", data_mem.M ); 	//Test 5

//   	$readmemh( "iMem_5.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_6.dat", data_mem.M ); 	//Test 6

//    	$readmemh( "iMem_6.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_7.dat", data_mem.M ); 	//Test 7

//    	$readmemh( "iMem_7.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_8.dat", data_mem.M ); 	//Test 8

//    	$readmemh( "iMem_8.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_9.dat", data_mem.M ); 	//Test 9

//   	$readmemh( "iMem_9.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_10.dat", data_mem.M );	//Test 10

//    	$readmemh( "iMem_10.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_11.dat", data_mem.M );	//Test 11

//   	$readmemh( "iMem_11.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_12.dat", data_mem.M );	//Test 12

//	$readmemh( "iMem_12.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_13.dat", data_mem.M );	//Test 13

//	$readmemh( "iMem_13.dat", cpu.IU.IM.IMem );

$readmemh( "dMem_14.dat", data_mem.M );	//Test 14

$readmemh( "iMem_14.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_rotleft.dat", data_mem.M ); //ROTLEFT Instruction

//	$readmemh( "iMem_rotleft.dat", cpu.IU.IM.IMem );

//	$readmemh( "dMem_rotright.dat", data_mem.M );//ROTRIGHT Instruction

//	$readmemh( "iMem_rotright.dat", cpu.IU.IM.IMem );

// Wait 100 ns for global reset to finish

#100;

reset = 0;

//Set interrupt to 1 at 150ns

#300;

intr = 1;

@(posedge int_ack)

intr = 0;

$display(" "); $display(" ");
   $display("******************************************************************");

$display("       CECS 440 - MIPS ISA Control Unit (Test for iM 1-14)        ");

$display("******************************************************************");

$display(" ");

end //initial

endmodule

CPU

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  CPU.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.2

* Rev. Date:  11/8/2018

*

* Purpose:    The Central Processing Unit(CPU) module has the Control

*             Unit, Instruction Unit and the Datpath modules.

*

* Notes:      The CPU can be connected to the DataMemory module and the

*             InputOutput module that can be written to and read from

*             according to the Instruction from the Instruction Memory

*             The Instruction Memory is instantiated using $readmemh on the

*             testbench module (CPU_test1.v)

*

* Revision Notes:

*             1.2:  Added IO flag outputs (io_cs, io_rd, io_wr)

*                   Added wires M_sel  - Determines data going into Datapath

*                               DY     - Input for DY reg of the Datapath

*             1.3:  Added wires D_val  - Value from D_OUT of Datapath

*                               stat_sel

*                                      - Determines the Data type being

*                                        output by the CPU

*                               S_sel  - Determines where register RS in the

*                                        Datapath is getting Data from

*                               D_in   - Register D_in from the Datapath now

*                                        connects to the Control Unit to

*                                        acquire the status flags from the

*                                        stack *

*

****************************************************************************/

module CPU(clk, reset,            //Clock and Reset

intr,  int_ack,        //Interrupt Flags

D_MemToInt, D_IOToInt, //Data Into IDP

Addr,  D_OUT,          //Data going to Memory

dm_cs, dm_rd, dm_wr,   //Data Memory Flags

io_cs, io_rd, io_wr    //IO Module Flags

);

input         clk, reset, intr;

input  [31:0] D_MemToInt, D_IOToInt;

output        int_ack;

output [31:0] Addr, D_OUT;

output        dm_cs, dm_rd, dm_wr;

output        io_cs, io_rd, io_wr;

//Wires

wire        N, Z, C, V;

wire [31:0] IR_out;

wire [31:0] PC_IUtoID, SE_16;

wire [31:0] DY;               //Added for Rev 1.2

//MCU control words(control words of state machine)

wire [4:0] FS;                //5-bit control word

wire [2:0] Y_Sel;             //3-bit control word

wire [1:0] pc_sel, DA_sel;    //2-bit control words

wire pc_ld, pc_inc, ir_ld, im_cs, im_rd, im_wr,

D_En, T_sel, HILO_LD;

//Added for Rev 1.2

wire M_sel;

//Added for Rev 1.3

wire [31:0] D_val;      //D_OUT of Datapath

wire  [1:0] stat_sel;   //0 - D_val from IDP, 1 - PC, 2 - Status Flags

wire        S_sel;      //0 - S from RegFile, 1 - Y_lo from ALU

wire [31:0] D_in;       //D_in register from IDP to the CU

/****************************************************************

*  Instantiate Control Unit, Instruction Unit and Datapath

***************************************************************/

//Control Unit

MCU      mips_ctrl(

.sys_clk(clk), .reset(reset),//Clock and Reset

.intr(intr),                 //Interrupt Request

.c(C), .n(N), .z(Z), .v(V),  //ALU status Flags

.IR(IR_out),                 //Instruction Register input

.int_ack(int_ack),           //output to I/O subsystem

.FS(FS),                     //Function Select

.ir_ld(ir_ld),               //IR Laod Enable

.pc_sel(pc_sel),             //  \

.pc_ld(pc_ld),               // --> PC Flags

.pc_inc(pc_inc),             //  /

.im_cs(im_cs),               //  \

.im_rd(im_rd),               // --> Instruction Memory Flags

.im_wr(im_wr),               //  /

.D_En(D_En),                 //  \

.DA_sel(DA_sel),             //   \

.T_sel(T_Sel),               // ---> Datapath Flags

.HILO_ld(HILO_LD),           //   /

.Y_sel(Y_Sel),               //  /

.dm_cs(dm_cs),               //  \

.dm_rd(dm_rd),               // --> Data Memory Flags

.dm_wr(dm_wr),               //  /

.io_cs(io_cs),               //  \

.io_rd(io_rd),               // --> I/O Module Flags

.io_wr(io_wr),               //  /

.M_sel(M_sel),               //DM or IO Select

.stat_sel(stat_sel),         //CPU Output type select

.S_sel(S_sel)		    //Select for register RS

.D_in(D_in)                  //Register D_in from IDP

);

//Instruction Unit

Instruction_Unit IU(

.clk(clk),  .reset(reset),   //Clock and Reset

.PC_in(Addr),                //Value to change PC

.ir_ld(ir_ld),               //IR Load Enable

.pc_sel(pc_sel),             //  \

.pc_ld(pc_ld),               // --> PC Flags

.pc_inc(pc_inc),             //  /

.im_cs(im_cs),               //  \

.im_rd(im_rd),               // --> Instruction Memory Flags

.im_wr(),                    //  /

.PC_out(PC_IUtoID),          //Current PC sending to Datapath

.IR_out(IR_out),             //Current IR sending to Datapath

.SE_16(SE_16)                //Sign-Extended IR[15:0]

);

//Datapath

Integer_Datapath

IDP (

.clk(clk),    .reset(reset), //Clock and Reset

.D_En(D_En),                 //Write Enable

.D_Addr(IR_out[15:11]),      //D - Address

.S_Addr(IR_out[25:21]),      //S - Address

.T_Addr(IR_out[20:16]),      //T - Address

.DT(SE_16),                  //Sign Extended IR[15:0]

.DA_sel(DA_sel),             //Register File Write Selector

.T_Sel(T_Sel),               //RT value Selector

.FS(FS),                     //Function Select

.N(N), .Z(Z), .C(C), .V(V),  //ALU Status Flags

.HILO_LD(HILO_LD),           //Enable for HI and LO

.DY(DY),                     //Value of Memory to D_in register

.PC_in(PC_IUtoID),           //PC value into the Datapath

.Y_Sel(Y_Sel),               //ALU_Out Selector

.ALU_OUT(Addr),              //Data Address output of CPU

.D_OUT(D_val),               //Data Value output of Datapath

.shift_val(IR_out[10:6]),    //Shift value for Barrel Shift

.S_sel(S_sel)                //Select for register RS

.D_in(D_in)                  //Register D_in to CU

);

// Select between Data Memory and IO Module

// (0 - Data Memory, 1 - I/O)

assign DY    = (M_sel == 0) ? D_MemToInt : D_IOToInt;

// Type of Data to send to memory

assign D_OUT = (stat_sel == 2'b00) ? D_val      :                 //RT

(stat_sel == 2'b01) ? PC_IUtoID  :                 //PC

(stat_sel == 2'b10) ? {28'b0, intr, C, V, N, Z} :  //Status Flags

D_OUT;

endmodule

MIPS Control Unit (MCU)

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  MCU.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.4

* Rev. Date:  11/20/2018

*

* Purpose:    A state machine implementing the MIPS Control Unit (MCU) for the

*             major cycles of fetch, execute and some MIPS instructions from

*             memory, including checking for interrupts.

*

* Notes:      MCU Control Word:

*

* {pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

* {im_cs, im_rd, im_wr}                  = 3'b0_0_0;

* {D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;   FS = 5'h0;

* {dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;          int_ack = 0;

*

* Revision Notes:

*             1.2: Added Instructions: BEQ, BNE, ADDI, SRL, JMP, SRA, SLL,

*                                      SLT, SLTI, JAL, MULT, MFLO, MFHI,

*                                      XOR, SLTU, DIV, XORI, SUB, ANDI,

*                                      SLTIU, BLEZ, BGTZ

*                  Added present and next state registers for the Interrupt

*                  and ALU status flags

*             1.3: Added Instructions: SETIE, INPUT, OUTPUT

*             1.4: Added Enhanced Instructions: ROTLEFT, ROTRIGHT

*

****************************************************************************/

module MCU  (sys_clk, reset, intr,                 //system inputs

c, n, z, v,                           //ALU status inputs

IR,                                   //Instruction Register input

int_ack,                              //Output to I/O subsystem

FS,                                   //Function Select

pc_sel, pc_ld, pc_inc, ir_ld,         //PC and IR control words

im_cs, im_rd, im_wr,                  //Inst. Memory control words

D_En, DA_sel, T_sel, HILO_ld, Y_sel,  //Datapath Control Words

dm_cs, dm_rd, dm_wr,                  //Data memory control words

io_cs, io_rd, io_wr, M_sel,           //IO module control words

stat_sel,                             //Select for CPU output

S_sel                                 //RS read select

D_in                                  //D_in reg (return flags)

);

input sys_clk, reset, intr;

input c, n, z, v;

input [31:0] IR;

output reg int_ack;

//control words

output reg [4:0] FS;                //5-bit control word

output reg [2:0] Y_sel;             //3-bit control word

output reg [1:0] pc_sel, DA_sel;    //2-bit control words

output reg pc_ld, pc_inc, ir_ld, im_cs, im_rd, im_wr,

D_En, T_sel, HILO_ld, dm_cs, dm_rd, dm_wr;

//Added for Rev 1.3

output reg io_cs, io_rd, io_wr, M_sel;

//Added for Rev 1.4

output reg [1:0] stat_sel;

output reg       S_sel;

input [31:0]     D_in;

//****************************

// Internal Data Structures

//****************************

//state assignments

parameter

RESET      = 00,  FETCH  = 01,  DECODE = 02,

ADD        = 10,  ADDU   = 11,  AND    = 12, OR    = 13, NOR = 14,

ORI        = 20,  LUI    = 21,  LW     = 22, LW2   = 23, SW  = 24,

WB_alu     = 30,  WB_imm = 31,  WB_Din = 32,

WB_hi      = 33,  WB_lo  = 34,  WB_mem = 35,

JR1        = 40,  JR2    = 41,

//      INTR_1     = 501, INTR_2 = 502, INTR_3 = 503,

BREAK      = 510,

ILLEGAL_OP = 511,

//added

BEQ        = 50,  BEQ2   = 51,               //Test 1

BNE        = 60,  BNE2   = 61,

ADDI       = 70,  SRL    = 71,  JMP    = 72, //Test 2

SRA        = 80,                             //Test 3

SLL        = 90,  SLT    = 91,               //Test 4

SLTI       = 92,                             //Test 5

JAL        = 100, JAL2   = 101,              //Test 7

MULT       = 110, MFLO   = 111, MFHI   = 112,//Test 8

XOR        = 120, SLTU   = 121,              //Test 9

DIV        = 130,                            //Test 10

XORI       = 140, SUB    = 141, ANDI   = 142,//Test 11

SLTIU      = 143,

BLEZ       = 150, BLEZ2  = 151,              //Test 12

BGTZ       = 152, BGTZ2  = 153,

SETIE      = 160,                            //Test 13

INPUT      = 161, INPUT2 = 162,

OUTPUT     = 163, OUTPUT2= 164,

INTR_1     = 201, INTR_2 = 202, INTR_3 = 203,//Test 14

INTR_4     = 204, INTR_5 = 205, INTR_6 = 206,

INTR_7     = 207, INTR_8 = 208,

RETI1      = 171, RETI2  = 172, RETI3  = 173,

RETI4      = 174, RETI5  = 175, RETI6  = 176,

//Enhanced Instructions

ROTLEFT    = 180, ROTRIGHT = 181;

//Registers

reg [8:0] state;              //State Register (up to 512 states)

reg psi, psc, psv, psn, psz,  //Present state Status Flags

nsi, nsc, nsv, nsn, nsz;  //Next state Status Flags

always @(posedge sys_clk, posedge reset)

if(reset)

{psi, psc, psv, psn, psz} = 5'b0;

else

{psi, psc, psv, psn, psz} = {nsi, nsc, nsv, nsn, nsz};

//***********************************************

// 440 MIPS Control Unit (Finite State Machine)

//***********************************************

//Next State Logic

always @(posedge sys_clk, posedge reset)

if(reset)

begin

// PC <- 32'b0, R31($ra) <- 32'b0

// NS <- RESET

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b10_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_10_0_0_100;   FS = 5'h15;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;          int_ack = 0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

state = RESET;

end

else

case(state)

FETCH:   @(negedge sys_clk)   begin

if(int_ack == 0 & intr == 1)  begin

// control word assignments for 'deasserting' everything

// NS <- INTR_1

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

FS = 5'h00;        int_ack = 0;

{stat_sel, S_sel}                      = 3'b00_0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_1;

end

else

begin

if(int_ack == 1 & intr == 0) int_ack = 0;

// IR <- iM[PC], PC <- PC + 4

// NS <- DECODE

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_1_1;

{im_cs, im_rd, im_wr}                  = 3'b1_1_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = DECODE;

end

end //fetch

RESET:   @(negedge sys_clk)

begin

// R29($sp) <- ALU_Out(32'h3FC)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_11_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //reset

DECODE:  @(negedge sys_clk)

begin

if( IR[31:26] == 6'h0 ) //check for MIPS format

begin    //R-type format

// RS <- $rs  RT <- $rt (default)

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           ={psi, c, v, n, z};

//Decides which state to go to according to IR[5:0]

case ( IR[5:0] )

6'h0D :  state = BREAK;

6'h20 :  state = ADD;

6'h08 :  state = JR1;

6'h02 :  state = SRL;      //iMem 2 test

6'h03 :  state = SRA;      //iMem 3 test

6'h00 :  state = SLL;      //iMem 4 test

6'h2A :  state = SLT;

6'h18 :  state = MULT;     //iMem 8 test

6'h10 :  state = MFHI;

6'h12 :  state = MFLO;

6'h26 :  state = XOR;      //iMem 9 test

6'h24 :  state = AND;

6'h25 :  state = OR;

6'h27 :  state = NOR;

6'h2B :  state = SLTU;

6'h1A :  state = DIV;      //iMem 10 test

6'h22 :  state = SUB;      //iMem 11 test

6'h1F :  state = SETIE;    //iMem 13 test

6'h2C :  state = ROTLEFT;  //Enhanced Instructions

6'h2D :  state = ROTRIGHT;

default: state = ILLEGAL_OP;

endcase

end //end of R - type

else

begin    //I-type or J-type format

// RS <- $rs, RT <- DT

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           ={psi, c, v, n, z};

//Decides which state to go to according to IR[31:26]

case ( IR[31:26] )

6'h0D :  state = ORI;

6'h0F :  state = LUI;

6'h2B :  state = SW;

6'h04 :  {T_sel, state} = {1'b0, BEQ}; //iMem 1 test

6'h05 :  {T_sel, state} = {1'b0, BNE}; //iMem 1 test

6'h08 :  state = ADDI;                 //iMem 2 test

6'h02 :  state = JMP;                  //iMem 2 test

6'h0A :  state = SLTI;                 //iMem 5 test

6'h23 :  state = LW;                   //iMem 6 test

6'h03 :  state = JAL;                  //iMem 7 test

6'h0E :  state = XORI;                 //iMem 11 test

6'h0C :  state = ANDI;

6'h0B :  state = SLTIU;

6'h06 :  state = BLEZ;                 //iMem 12 test

6'h07 :  state = BGTZ;

6'h1C :  state = INPUT;                //iMem 13 test

6'h1D :  state = OUTPUT;

6'h1E :  state = RETI1;                //iMem 14 test

default: state = ILLEGAL_OP;

endcase

end //end of I - type

end //end of DECODE

ADD:     @(negedge sys_clk)   //Add (R-type)

begin

// ALU_Out <- RS($rs) + RT($rt)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //ADD

ORI:     @(negedge sys_clk)   //Or Immediate (I-type)

begin

// ALU_Out <- RS($rs) or {16'h0, RT[15:0]}

// NS <- WB_imm

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h17;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //ORI

LUI:     @(negedge sys_clk)   //Load Upper Immediate (I-type)

begin

// ALU_Out <- { RT[15:0], 16'h0 }

// NS <- WB_imm

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h18;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //LUI

SW:     @(negedge sys_clk)   //Store Word (I-type)

begin

// ALU_Out <- RS($rs) + RT(se_16), RT <- $rt

// NS <- WB_mem

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_mem;

end //SW

WB_alu:  @(negedge sys_clk)   //Write Back to D_Addr

begin

// R[rd] <- ALU_Out

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //WB_alu

WB_imm:  @(negedge sys_clk)   //Write Back to T_Addr

begin

// R[rt] <- ALU_Out

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_01_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //WB_imm

WB_mem:  @(negedge sys_clk)   //Write Back to Memory

begin

// M[ ALU_Out($rs + se_16) ] <- RT($rt)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_0_1;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //WB_mem

BREAK:   @(negedge sys_clk)

begin

$display("BREAK INSTRUCTION FETCHED %t", $time);

//control word assignments for 'deasserting' everything

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

FS = 5'h00;        int_ack = 0;

$display(" R E G I S T E R ' S   A F T E R   B R E A K");

$display(" ");

Reg_Dump();       //Dumps Register Values

//Dump_DMEM();

Dump_DMEM_INTR(); //Dumps Values in Memory (0x3F0 - 0x404)

Dump_IO();        //Dumps Values in IO Memory

Dump_PCIR();

$display(" ");

$finish;          //ends simulation

end //BREAK

ILLEGAL_OP:

@(negedge sys_clk)

begin //ILLEGAL_OP

$display("ILLEGAL OPCODE FETCHED %t", $time);

// control word assignments for 'deasserting everything'

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

FS = 5'h00;        int_ack = 0;

Reg_Dump();

Dump_PCIR();        //Dumps Value of PC and IR

$finish;          //ends simulation

end //ILLEGAL_OP

INTR_1:  @(negedge sys_clk)

begin

//PC gets address of interrupt vector, Save PC in stack

// RS <- 0x3FC, ALU_Out <- 0x3FC, R[$ra] <- PC

// NS <- INTR_2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h15;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_2;

end //INTR_1

INTR_2:  @(negedge sys_clk)

begin

// ALU_Out <- RS(0x3FC) + 4, RS <- RS(0x3FC) + 4

// NS <- INTR_3

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h11;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_3;

end //INTR_2

INTR_3:  @(negedge sys_clk)

begin

// M[ ALU_Out(0x400) ] <- PC,  RS <- ALU_Out(0x400)

// NS <- INTR_4

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_0_1;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b01_1;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_4;

end //INTR_3

INTR_4:  @(negedge sys_clk)

begin

// ALU_Out <- RS(0x400) + 4

// NS <- INTR_5

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h11;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_5;

end //INTR_4

INTR_5:  @(negedge sys_clk)

begin

// M[ ALU_Out(0x404) ] <- status flags(psi, c, v, n, z)

// $sp <- ALU_Out(0x404)

// NS <- INTR_6

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_0_1;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b10_0;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_6;

end //INTR_5

INTR_6:  @(negedge sys_clk)

begin

// ALU_Out <- 0x3FC, RS <- 0x3FC

// NS <- INTR_7

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h15;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_7;

end //INTR_6

INTR_7:  @(negedge sys_clk)

begin

//Read address of ISR into D_in

// D_in <- dM[ALU_Out(0x3FC)]

// NS <- INTR_8

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_1_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INTR_8;

end //INTR_7

INTR_8:  @(negedge sys_clk)

begin

//Reload PC with address of ISR; ack the intr; goto FETCH

// PC <- D_in( dM[0x3FC] ), int_ack <- 1

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b10_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_011;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;          int_ack = 1;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end  //INTR_8

JR1:	@(negedge sys_clk)   //Jump Register (R-type)

begin

// ALU_Out <- RS($rs)

// NS <- JR2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = JR2;

end //JR1

JR2:	@(negedge sys_clk)

begin

// PC <- ALU_Out($rs)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b10_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //JR2

//Added States for iMem 1-12

//test 1

BEQ:  @(negedge sys_clk)   //Branch on Equal (I-type)

begin

// ALU_Out <- RS($rs) - RT($rt)

// NS <- BEQ2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h03;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = BEQ2;

end //BEQ

BEQ2: @(negedge sys_clk)

begin

// if(z==1)    PC <- ALU_Out(se_16), NS <- FETCH

// else        NS <- FETCH

if(z == 1)  begin    //BEQ pass

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h01;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

else  begin          //BEQ fail

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

end //BEQ2

BNE:  @(negedge sys_clk)   //Branch Not Equal (I-type)

begin

// ALU_Out <- RS($rs) - RT($rt)

// NS <- BRANCH(pass) OR NS <- FETCH(fail)

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h03;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = BNE2;

end //BNE

BNE2:	 @(negedge sys_clk)

begin

// if(z==1)    PC <- PC + ALU_Out(se_16), NS <- FETCH

// else        NS <- FETCH

if(z != 1)  begin    //BNE pass

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h01;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

else  begin          //BNE fail

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

end //BNE2

//test 2

ADDI: @(negedge sys_clk)   //Add Immediate (I-type)

begin

// ALU_Out <- RS($rs) + RT(se_16), RT <- RT($rt)

// NS <- WB_imm

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //ADDI

SRL:  @(negedge sys_clk)   //Shift Right Logical (R-type)

begin

// ALU_Out <- RT($rt) >> IR[10:6]

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h0D;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SRL

JMP:  @(negedge sys_clk)   //Jump (J-type)

begin

// PC <- {PC[31:28], IR[25:0], 2'b00}

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b01_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //JMP

//test 3

SRA:  @(negedge sys_clk)   //Shift Right Arithmetic (R-type)

begin

// ALU_Out <- {$rt[31], %rt[31:1]}

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h0E;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SRA

//test 4

SLL:  @(negedge sys_clk)   //Shift Left Logical (I-type)

begin

// ALU_Out <- RT($rt) << IR[10:6]

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h0C;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SLL

SLT:  @(negedge sys_clk)   //Set Less Than (R-type)

begin

// if( RS($rs) < RT($rt) )    ALU_Out <- 1

// else                       ALU_Out <- 0

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h06;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SLT

//test 5

SLTI: @(negedge sys_clk)   //Set Less Than Immediate (I-type)

begin

// if( RS($rs) < RT(se_16) )  ALU_Out <- 1

// else                       ALU_Out <- 0

// NS <- WB_imm

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h06;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //SLTI

//test 6

LW: @(negedge sys_clk)   //Load Word (I-type)

begin

// ALU_Out <- RS($rs) + RT(se_16)

// NS <- LW2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = LW2;

end //LW

LW2: @(negedge sys_clk)

begin

// D_in <- M[rs + se_16]

// NS <- WB_Din

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_1_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_Din;

end //LW2

WB_Din: @(negedge sys_clk)   //Write Back to Address of D_in

begin

// R[rt] <- D_in( M[rs + se_16] )

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_01_1_0_011;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //WB_Din

//test 7

JAL:  @(negedge sys_clk)   //Jump and Link (J-type)

begin

// R31 <- PC

// NS <- JAL2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_10_0_0_100;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = JAL2;

end //JAL

JAL2: @(negedge sys_clk)

begin

// PC <- {PC[31:28], IR[25:0], 2'b00}

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b01_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //JAL2

//test 8

MULT: @(negedge sys_clk)   //Multiply (R-type)

begin

// {HI, LO} <- RS($rs) * RT($rt)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_1_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h1E;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //MULT

MFLO: @(negedge sys_clk)   //Move from LO (R-type)

begin

// R[rd] <- LO

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_00_0_0_010;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //MFLO

MFHI: @(negedge sys_clk)   //Move from HI (R-type)

begin

// R[rd] <- HI

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_00_0_0_001;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //MFHI

//test 9

XOR:  @(negedge sys_clk)   //Exclusive OR (R-type)

begin

// ALU_Out <- RS($rs) ^ RT($rt)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h0A;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //XOR

AND:  @(negedge sys_clk)   //AND (R-type)

begin

// ALU_Out <- RS($rs) & RT($rt)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h08;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //AND

OR:  @(negedge sys_clk)   //OR (R-type)

begin

// ALU_Out <- RS($rs) | RT($rt)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h09;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //OR

NOR:  @(negedge sys_clk)   //Not OR (R-type)

begin

// ALU_Out <- ~( RS($rs) | RT($rt) )

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h0B;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //NOR

SLTU: @(negedge sys_clk)   //Set Less Than Unsigned (R-type)

begin

// if( RS($rs) < RT(%rt) )    ALU_Out <- 1

// else                       ALU_Out <- 0

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h07;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SLTU

//test 10

DIV: @(negedge sys_clk)   //Divide (R-type)

begin

// {HI, LO} <- RS($rs) / RT($rt)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_1_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h1F;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //DIV

//test 11

XORI: @(negedge sys_clk)   //Exclusive OR Immediate (I-type)

begin

// ALU_Out <- RS($rs) ^ RT(se_16)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h19;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //XORI

SUB: @(negedge sys_clk)   //Subtract (R-type)

begin

// ALU_Out <- RS($rs) - RT($rt)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h03;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end //SUB

ANDI: @(negedge sys_clk)   //AND Immediate (I-type)

begin

// ALU_Out <- RS($rs) & RT(se_16)

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h16;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //ANDI

SLTIU: @(negedge sys_clk)   //Set Less Than Immediate Unsigned (I-type)

begin

// if( RS($rs) < RT(%se_16) )    ALU_Out <- 1

// else                          ALU_Out <- 0

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h07;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_imm;

end //SLTIU

//test 12  BLEZ, BGTZ

BLEZ: @(negedge sys_clk)//Branch on Less than or Equal to Zero (I type)

begin

// ALU_Out <- RS($rs) - RT($rt), RT <- DT(se_16)

// NS <- BLEZ2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h03;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = BLEZ2;

end //BLEZ

BLEZ2: @(negedge sys_clk)

begin

// if(z==1) or (n==1) PC <- ALU_Out(se_16), NS <- FETCH

// else               NS <- FETCH

if( (z == 1) | (n == 1) )  begin    //BLEZ pass

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h01;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

else  begin                         //BLEZ fail

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

end //BLEZ2

BGTZ: @(negedge sys_clk)   //Branch on Greater Than Zero (I-type)

begin

// ALU_Out <- RS($rs) - RT($rt), RT <- DT(se_16)

// NS <- BGTZ2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h03;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = BGTZ2;

end //BGTZ

BGTZ2: @(negedge sys_clk)

begin

// if(z==1)    PC <- ALU_Out(se_16), NS <- FETCH

// else        NS <- FETCH

if( (z == 1) | (n == 1) ) begin  //BGTZ fail

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

else  begin                      //BGTZ pass

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end

end //BGTZ2

//test 13

SETIE: @(negedge sys_clk)   //Set Interrupt Enable

begin

// NSI <- 1'b1

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {1'b1, c, v, n, z};

state = FETCH;

end //SETIE

INPUT: @(negedge sys_clk)   //INPUT (similar to LW)

begin

// ALU_Out <- RS($rs) + RT(se_16)

// NS <- INPUT2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = INPUT2;

end //INPUT

INPUT2: @(negedge sys_clk)   //      (similar to LW2)

begin

// D_in <- I_O[rs + se_16]

// NS <- WB_Din

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b1_1_0_1;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_Din;

end //INPUT2

OUTPUT: @(negedge sys_clk)   //OUTPUT (similar to SW)

begin

// ALU_Out <- RS($rs) + RT(se_16), RT <- $rt

// NS <- OUTPUT2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_1_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h02;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = OUTPUT2;

end //OUTPUT

OUTPUT2: @(negedge sys_clk)   //       (similar to WB_mem)

begin

// I_O[ ALU_Out($rs + se_16) ] <- RT($rt)

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b1_0_1_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h00;        int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end //OUTPUT2

//test 14

RETI1: @(negedge sys_clk)   //Return Interrupt

begin

// ALU_Out <- RS(0x404)

// NS <- RETI2

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = RETI2;

end //RETI1

RETI2: @(negedge sys_clk)

begin

// D_in <- dM[ALU_Out(0x404)],

// NS <- RETI3

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_1_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = RETI3;

end  //RETI2

RETI3: @(negedge sys_clk)

begin

// I, C, V, N, Z <- D_in(28'b0, intr, C, V, N, Z),

// ALU_Out <- RS(0x404) - 4, RS <- 0x404 - 4

// NS <- RETI4

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h12;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = D_in[4:0];

state = RETI4;

end //RETI3

RETI4: @(negedge sys_clk)

begin

// D_in <- dM[ALU_Out(0x400)],

// ALU_Out <- RS(0x400) - 4, RS <- 0x400 - 4

// NS <- RETI5

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b1_1_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h12;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = RETI5;

end //RETI4

RETI5: @(negedge sys_clk)

begin

// PC <- D_in( dM[0x400] ),

// NS <- RETI6

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b10_1_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_011;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h00;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = RETI6;

end //RETI5

RETI6: @(negedge sys_clk)

begin

// $sp <- ALU_Out(0x3FC), int_ack = 1,

// NS <- FETCH

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b1_11_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_1;

FS = 5'h00;          int_ack = 1;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = FETCH;

end  //RETI6

//Enhanced Instructions

ROTLEFT: @(negedge sys_clk)

begin

// ALU_Out <- RT($rt) (rotate left) IR[10:6]

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h1C;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end  //ROTLEFT

ROTRIGHT: @(negedge sys_clk)

begin

// ALU_Out <- RT($rt) (rotate right) IR[10:6]

// NS <- WB_alu

{pc_sel, pc_ld, pc_inc, ir_ld}         = 5'b00_0_0_0;

{im_cs, im_rd, im_wr}                  = 3'b0_0_0;

{D_En, DA_sel, T_sel, HILO_ld, Y_sel}  = 8'b0_00_0_0_000;

{dm_cs, dm_rd, dm_wr}                  = 3'b0_0_0;

{io_cs, io_rd, io_wr, M_sel}           = 4'b0_0_0_0;

{stat_sel, S_sel}                      = 3'b00_0;

FS = 5'h1D;          int_ack = 0;

#1 {nsi, nsc, nsv, nsn, nsz}           = {psi, c, v, n, z};

state = WB_alu;

end  //ROTRIGHT

endcase  //end of FSM logic

// Simulation Tasks

integer i;

//Task to dump values of RegFile

task Reg_Dump;

//Output register values

for( i = 0; i < 16; i = i + 1 )

@(negedge sys_clk)

begin

$display( "t=%t, $r%0d = 0x%h  ||   $r%0d = 0x%h",

$time, i,    CPU_test3.cpu.IDP.regfile.regs[i],

i+16, CPU_test3.cpu.IDP.regfile.regs[i+16]);

end

endtask

//Task to dump values of Memory Locations 0xC0 to 0xFF

task Dump_DMEM;   begin

$display(" "); $display("Memory Locations 0xC0 to 0xFF");

for(i = 12'h0C0; i < 12'h100; i = i + 4)

@(negedge sys_clk)

$display( "t=%t, M[%h] = 0x%h%h%h%h",

$time, i, CPU_test3.data_mem.M[i],

CPU_test3.data_mem.M[i+1],

CPU_test3.data_mem.M[i+2],

CPU_test3.data_mem.M[i+3]);

end

endtask

//Task to dump value of PC and IR

task Dump_PCIR;

@(negedge sys_clk)

$display( "t=%t, PC = 0x%h,   IR = 0x%h",

$time, CPU_test3.cpu.IU.PC, CPU_test3.cpu.IU.IR);

endtask

//Task to dump values from IO Location 0xC0 to 0xFF

task Dump_IO;  begin

$display(" "); $display("IO Memory Locations 0xC0 to 0xFF");

for(i = 12'h0C0; i < 12'h100; i = i + 4)

@(negedge sys_clk)

$display( "t=%t, IO[%h] = 0x%h%h%h%h",

$time, i, CPU_test3.in_out.IO[i],

CPU_test3.in_out.IO[i+1],

CPU_test3.in_out.IO[i+2],

CPU_test3.in_out.IO[i+3]);

end

endtask

//Task to dump value from Memory Location 0x3F0

task Dump_DMEM_3F0; begin

$display(" "); $display("Memory Location 0x3F0");

i = 12'h3F0;

@(negedge sys_clk)

$display( "t=%t, M[%h] = 0x%h%h%h%h",

$time, i, CPU_test3.data_mem.M[i],

CPU_test3.data_mem.M[i+1],

CPU_test3.data_mem.M[i+2],

CPU_test3.data_mem.M[i+3]);

end

endtask

//Task to dump values from Memory Locations 0x3F0 to 0x404

task Dump_DMEM_INTR; begin

$display(" "); $display("Memory Location 0x3F0 to 0x404");

for(i = 12'h3F0; i < 12'h408; i = i + 4)

@(negedge sys_clk)

$display( "t=%t, M[%h] = 0x%h%h%h%h",

$time, i, CPU_test3.data_mem.M[i],

CPU_test3.data_mem.M[i+1],

CPU_test3.data_mem.M[i+2],

CPU_test3.data_mem.M[i+3]);

end

endtask

endmodule

Instruction Unit (IU)

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  InstructionUnit.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.2

* Rev. Date:  11/8/2018

*

* Purpose:    This is the Instruction Unit that holds and controls the

*             Program Counter, Instruction Memory and the Instruction Register.

* Notes:      This module holds the 32-bit Program Counter, the Instruction

*             Memory following the MIPS Instruction Formats, and the

*             32-bit Instruction Register for the Datapath.

*                IR_out[25:21] -> S_Addr for the Datapath

*                IR_out[20:16] -> T_Addr for the Datapath

*                IR_out[15:11] -> D_Addr for the Datapath

*

****************************************************************************/

module Instruction_Unit(clk, reset, PC_in, pc_sel, pc_ld, pc_inc, PC_out,

im_cs, im_wr, im_rd,

ir_ld, IR_out, SE_16);

input clk, reset;          //Clock and Reset

input [31:0] PC_in;        //Value to change PC

input  [1:0] pc_sel;       //Select for new PC value

input pc_ld, pc_inc;       //PC write enable and increment flag

input         ir_ld;       //Instruction Register Load Enable

input im_cs, im_wr, im_rd; //Instruction Memory Flags

output [31:0] PC_out;      //Output current PC

output [31:0] IR_out,      //Output current IR

SE_16;      //Output sign extended IR

reg [31:0] PC, IR;         //PC and IR register

wire [31:0] IMem_to_IR;    //Data from I-Memory to IR if ir_ld is 1

InstructionMemory IM (

.clk(clk),                   //Clock

.Address( { 20'b0,           //Only takes first 12 bits

PC_out[11:0]}),  //of Address

.D_In(32'h0),                //Handled by $readmemh in TB

.im_cs(im_cs),               //  \

.im_rd(im_rd),               // --> Instruction Memory Flags

.im_wr(),                    //  /

.D_Out(IMem_to_IR)

);

always @(posedge clk, posedge reset)

if(reset)

{PC, IR} <= 64'b0;      //Reset PC and IR values

else

begin

//Program Counter Input Logic

if(pc_ld)

case(pc_sel)

2'b00:   PC <= PC + { SE_16[29:0], 2'b00 };     //for Branches

2'b01:   PC <= { PC[31:28], IR[25:0], 2'b00 };  //for Jumps

2'b10:   PC <= PC_in;                           //for Returns

default: PC <= PC;

endcase

else

if(pc_inc)     //Increment PC

PC <= PC + 32'd4;

else

PC <= PC;

//IR

if(ir_ld)   IR <= IMem_to_IR;

else        IR <= IR;

end

// Asynchronous Outputs of PC, IR and SE

assign PC_out = PC;

assign IR_out = IR;

assign SE_16 = { {16{IR[15]}} , IR[15:0] };  //sign extended Instruction Register

endmodule

Instruction Memory (iM)

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  InstructionMemory.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/9/2018

*

* Purpose:    This is the 4098x8 bit readable/writable instruction memory module.

*

* Notes:      This module will be used to read and store memory data.

*             To read or write data, chip select "im_cs" must be set to 1.

*             To read, "im_rd" must be set to 1. To write, "im_wr" must be

*             set to 1. Data will be accessed through big endian format.

*

****************************************************************************/

module InstructionMemory(clk, Address, D_In,

im_cs, im_wr, im_rd, D_Out);

input clk;

input [31:0] Address, D_In;

input im_cs, im_wr, im_rd;

output [31:0] D_Out;

//use 1024x32 'big endian' MSB goes to smaller address

reg [7:0] IMem [0:4095];  //4098x8 memory

always @(posedge clk)   begin

//write

if(im_cs && im_wr)   begin

IMem[Address]     <= D_In[31:24];

IMem[Address + 1] <= D_In[23:16];

IMem[Address + 2] <= D_In[15:8];

IMem[Address + 3] <= D_In[7:0];

end

end

//asynchronous continuous output READ

assign D_Out = (im_cs && im_rd) ?

{IMem[Address],IMem[Address + 1],IMem[Address + 2],IMem[Address+3]}:

32'hZZZZ_ZZZZ;          //high impedance

endmodule

Integer Datapath (IDP)

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  Integer_Datapath.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.2

* Rev. Date:  11/8/2018

*

* Purpose:    This module instantiates two modules:

*             regfile_32 and alu_32. This module contains two MUX and two

*             32 bit register blocks for HI and LO.

*

* Notes:      This is the integer datapath module. It will connect the 32 bit

*             regfile module with the 32 bit alu module. Within the T-Mux,

*             T_Sel will be used to select T data from two choices: regfile T

*             and immediate 32 bit value DT. Multiplication and Division

*             outputs will be stored in HI LO register blocks. Within the

*             Y-Mux, Y_Sel will select ALU_OUT from five choices: HI, LO,

*             Y_lo, 32 bit immediate DY, and 32 bit immediate PC_in. ALU_OUT

*             will be stored in register specified by address D_Addr when

*             D_En is enabled.

*

* Revision Notes:

*             1.2:  Added input shift_val - Shift Value for Barrel Shift

*             1.3:  Added input S_sel     - Select for register RS

*                        output D_in      - For the MCU to acquire flags

*

****************************************************************************/

module Integer_Datapath( clk, reset, S_Addr, T_Addr, D_Addr, D_En, DT, DA_sel,

T_Sel, HILO_LD, FS, DY, PC_in, Y_Sel,

C, V, N, Z, ALU_OUT, D_OUT,

shift_val,

S_sel

D_in);

input        clk, reset, D_En, T_Sel, HILO_LD;

input  [1:0] DA_sel;

input  [2:0] Y_Sel;

input  [4:0] S_Addr, T_Addr, D_Addr, FS;

input [31:0] DT, DY, PC_in;

input  [4:0] shift_val; //Added for Rev 1.2

input        S_sel;     //Added for Rev 1.3

output            C, V, N, Z;

output reg [31:0] ALU_OUT, D_OUT;

//Wires and Registers

wire [31:0] S, T, Y_hi, Y_lo;

wire  [4:0] DA_Addr;

reg  [31:0] HI, LO, RS, RT, ALU_reg, D_in;

output reg [31:0] D_in; 	   //D_in is output to write back the status flags

//Register File module Instantiation

regfile_32 regfile (

.clk(clk),       .reset(reset),	.D(ALU_OUT),  .D_En(D_En),

.D_Addr(DA_Addr),.S_Addr(S_Addr),	.T_Addr(T_Addr),

.S(S), .T(T)

);

//ALU module Instantiation

alu_32 alu (

.FS(FS),     .S(RS),    .T(RT),

.N(N),       .Z(Z),     .V(V),   .C(C), 
               .Y_hi(Y_hi), .Y_lo(Y_lo),
               .shift_val(shift_val)

);

//2-to-1 MUX that determines the D_OUT output of the CPU

always @(*)
   if(T_Sel)   D_OUT <= DT;

else        D_OUT <= T;

//4-to-1 MUX that determines the address of RegFile to write to

assign DA_Addr = (DA_sel == 2'b00)? D_Addr : //IR[15:11]

(DA_sel == 2'b01)? T_Addr : //IR[20:16]

(DA_sel == 2'b10)? 5'h1F  : //Scratch PC register R31

(DA_sel == 2'b11)? 5'h1D  : //Stack Pointer register R29

DA_Addr;

//Synchronous Register Assignments

always @(posedge clk, posedge reset)

if(reset)

{RS, RT, HI, LO, ALU_reg, D_in, ALU_OUT, D_OUT} <= 256'h0;

else  begin

RT <= D_OUT;

{ALU_reg, D_in} <= {Y_lo, DY};

if(S_sel) RS <= Y_lo;

else      RS <= S;

if(HILO_LD) {HI, LO} <= {Y_hi, Y_lo};

else        {HI, LO} <= {HI, LO};

end

//5-to-1 MUX that determines the address

always @ (*)

begin

case( Y_Sel )

3'b000  : ALU_OUT = ALU_reg; // pass SA

3'b001  : ALU_OUT = HI;      // pass SB

3'b010  : ALU_OUT = LO;      // pass SA

3'b011  : ALU_OUT = D_in;    // pass SB

3'b100  : ALU_OUT = PC_in;   // pass SB

default : ALU_OUT = 32'b0;   // pass 32 bits of 0s

endcase

end

endmodule

RegFile_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  regfile_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2018

*

* Purpose:    This is the 32 bit register file module.

*

* Notes:      This 32 bit register file module will store 32 registers of

*             size 32 bit. D is the incoming 32 bit data. S and T are 32 bit

*             data outputs of the module. D_Addr, S_Addr, and T_Addr

*             will address which register to select respectively for D, S,

*             and T. D_En will act as a write enable. S and T are

*             asynchronous outputs while D will be a synchronous input.

*

****************************************************************************/

module regfile_32(clk, reset, D, D_En, D_Addr, S_Addr, T_Addr, S, T);

input clk, reset, D_En;

input [4:0] S_Addr, D_Addr, T_Addr;

input [31:0] D;

output reg [31:0] S, T;

//32-bit 32 registers

reg [31:0] regs [31:0];

//Asynchronous Reading

always @ (*)

begin	
       S <= regs[S_Addr];

T <= regs[T_Addr];

end

//Synchronous Writing

always @ ( posedge clk, posedge reset )

begin

if ( reset )

regs[0] <= 32'h0;

else if ( D_En && ( D_Addr != 5'h0 ) )

regs[D_Addr] <= D;

else

regs[D_Addr] <= regs[D_Addr];

end

endmodule

alu_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  alu_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.3

* Rev. Date:  11/20/2018

*

* Purpose:    This module will instantiate three modules:

*             MIPS_32, DIV_32, and MPY_32.

*

* Notes:      This is the Arithmetic Logic Unit module. It will act as a

*             wrapper instantiating MIPS_32, DIV_32, and MPY_32 modules.

*             5 bit function select "FS" will select which wires to use for

*             this module's outputs.

*

* Revision Notes:

*             1.2:  Added Barrel Shift module

*                   Added input shift_val - Shift Value for Barrel Shift

*             1.3:  Added enhancement Rotate module

*

****************************************************************************/

module alu_32( FS, S, T, N, Z, V, C, Y_hi, Y_lo, shift_val );

input wire [4:0] FS;          // function select

input wire [31:0] S, T;       // two 32 bit inputs

input [4:0] shift_val;        //Added for Rev 1.2

output reg N, Z, V, C;        // negative, zero, overflow, carry flags

output reg [31:0] Y_hi, Y_lo; // two 32 bit outputs

//Temporary Wires

wire [63:0] Y_mips, Y_mul, Y_div;   //Y values of MIPS, MPY and DIV

wire N_mps, Z_mps, V_mps, C_mps;    //All flags of the MIPS module

wire N_mul, Z_mul, V_mul, C_mul;    //All flags of the MPY module

wire N_div, Z_div, V_div, C_div;    //All flags of the DIV module

//Added Wires for Rev 1.2

wire [31:0] Y_BS;

wire        C_BS;

//Added Wires for Rev 1.3

wire [31:0] Y_ROT;

// instantiate MIPS, DIV, and MPY modules

MIPS_32       MIPS(

.S(S),                 //32-bit Input S

.T(T),                 //32-bit Input T

.FS(FS),               //Function Select

.Y_hi(Y_mips[63:32]),  //32-bit upper output from MIPS

.Y_lo(Y_mips[31:0]),   //32-bit lower output from MIPS

.N(N_mps),             //Negative flag output of MIPS

.Z(Z_mps),             //Zero     flag output of MIPS

.V(V_mps),             //Overflow flag output of MIPS

.C(C_mps)              //Carry    flag output of MIPS

);

MPY_32         MPY(

.S(S),                 //32-bit Input S

.T(T),                 //32-bit Input T

.Y_hi(Y_mul[63:32]),   //32-bit upper output from MPY

.Y_lo(Y_mul[31:0]),    //32-bit lower output from MPY

.N(N_mul),             //Negative flag output of MPY

.Z(Z_mul),             //Zero     flag output of MPY

.V(V_mul),             //Overflow flag output of MPY

.C(C_mul)              //Carry    flag output of MPY

);

DIV_32         DIV(

.S(S),                 //32-bit Input S

.T(T),                 //32-bit Input T

.Y_hi(Y_div[63:32]),   //32-bit upper output from MPY

.Y_lo(Y_div[31:0]),    //32-bit upper output from MPY

.N(N_div),             //Negative flag output of DIV

.Z(Z_div),             //Zero     flag output of DIV

.V(V_div),             //Overflow flag output of DIV

.C(C_div)              //Carry    flag output of DIV

);

barrelSHIFT_32  BS(

.D(T),                 //32-bit Input T

.shift_val(shift_val), //Shift value

.LRRA(FS[1:0]),        //Select for shift type

.Y(Y_BS),              //32-bit Output of Barrel Shift

.C_BS(C_BS)            //Carry of Barrel Shift

);

rotate_32      ROT(

.D(T),                 //32-bit Input T

.shift_val(shift_val), //Rotate Value

.LorR(FS[0]),          //Rotate Direction

.Y(Y_ROT)              //32-bit Output of Rotate

);

//sets the ALU Y value to the Y value from its needed module.

always @(*) begin

//Barrel Shift

if( (FS == 5'h0C) | (FS == 5'h0D) | (FS == 5'h0E) ) begin

{Y_hi, Y_lo} = {32'b0, Y_BS};

{N, Z, V, C} = {

(Y_BS[31]) ? 1'b1:1'b0,       //Negative

(Y_BS == 32'h0) ? 1'b1:1'b0,  //Zero

1'b0,                         //Overflow

C_BS                          //Carry

};

end

//Rotate

else if( (FS == 5'h1C) | (FS == 5'h1D) ) begin

{Y_hi, Y_lo} = {32'b0, Y_ROT};

{N, Z, V, C} = {

(Y_ROT[31]) ? 1'b1:1'b0,      //Negative

(Y_ROT == 32'h0) ? 1'b1:1'b0, //Zero

1'b0,                         //Overflow

1'b0                          //Carry

};

end

//Multiply

else if(FS == 5'h1E)      begin

{Y_hi, Y_lo} = Y_mul;

{N, Z, V, C} = {N_mul, Z_mul, 1'bx, 1'bx};

end

//Divide

else if(FS == 5'h1F) begin

{Y_hi, Y_lo} = Y_div;

{N, Z, V, C} = {N_div, Z_div, V_div, 1'bx};

end

//MIPS

else                 begin

{Y_hi, Y_lo} = Y_mips;

{N, Z, V, C} = {N_mps, Z_mps, V_mps, C_mps};

end

end //always

endmodule

MIPS_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  MIPS_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.2

* Rev. Date:  11/8/2018

*

* Purpose:    This module will perform the following operations for each of

*             its corresponding 5 bit function select hex value:

*

*                0 - Pass_S   7 - SLTU   E  - SRA     15 - SP_INIT

*                1 - Pass_T   8 - AND    F  - INC     16 - ANDI

*                2 - ADD      9 - OR     10 - DEC     17 - ORI

*                3 - SUB      A - XOR    11 - INC4    18 - LUI

*                4 - ADDU     B - NOR    12 - DEC4    19 - XORI

*                5 - SUBU     C - SLL    13 - ZEROS   default - all x's

*                6 - SLT      D - SRL    14 - ONES

*

*             This module will detect if the result: is a negative value,

*             is a zero, has overflow, or has carry. The results of the

*             operation will be sent out as outputs.

*

* Notes:      This is the 32 bit MIPS operation module. It will take a 5 bit

*             function selet input "FS" to determine which operation it

*		  should execute. I will also take two 32 bit values for its

*             input. Flags 'N', 'Z', 'V', and 'C' (negative, zero, overflow,

*             and carry) will be outputted along with the resulting two

*             32 bit values "Y_hi" and "Y_lo".

*

* Revision Notes:

*             1.2: SLL, SRL and SRA are commented out and are handled by

*                  the Barrel Shift module

*

****************************************************************************/

module MIPS_32( FS, S, T, N, Z, V, C, Y_hi, Y_lo );

input [31:0] S, T;

input [4:0] FS;

output reg [31:0] Y_hi, Y_lo;

output reg N, Z, V, C;

reg neg, zero, ovf, carry;

//cast S and T as integers for the SLT op

integer inta, intb;

//always @(S, T, FS)   //any change of the inputs(S, T or FS)

always @(*)

begin

case(FS)

5'h00: begin //PASS_S   (Y_lo = S)

Y_lo = S;

if (S[31] == 1'b1)  neg  = 1; else neg  = 0;

if (S     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h01: begin //PASS_T   (Y_lo = T)

Y_lo = T;

if (T[31] == 1'b1)  neg  = 1; else neg  = 0;

if (T     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h02: begin //ADD      (Y_lo = S + T)

{carry, Y_lo} = S + T;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) & (Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h03: begin //SUB      (Y_lo = S - T)

{carry, Y_lo} = S - T;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) & (Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h04: begin //ADDU     (Y_lo = S + T) unsigned

{carry, Y_lo} = S + T;

if (Y_lo == 32'b0) zero = 1; else zero = 0;

ovf = carry;

{N, Z, V, C} = {1'b0, zero, ovf, carry};

end

5'h05: begin //SUBU     (Y_lo = S - T) unsigned

Y_lo = S - T;

if (S < T)         neg  = 1;  else neg  = 0;

if (Y_lo == 32'b0) zero = 1;  else zero = 0;

if (T > S)         carry = 1; else carry = 0;

ovf = carry;

{N, Z, V, C} = {1'b0, zero, ovf, carry};

end

5'h06: begin //SLT      (if S < T -> Y_lo = 1, else Y_lo = 0)

//S and T are casted as integers to compare

//signed integers

inta = S;

intb = T;

if(inta < intb)  Y_lo = 32'd1;

else             Y_lo = 32'b0;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h07: begin //SLTU     (if S < T -> Y_lo = 1, else Y_lo = 0) unsigned

if(S < T)  Y_lo = 32'd1;

else       Y_lo = 32'b0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {1'b0, zero, 2'bxx};

end

5'h08: begin //AND      (Y_lo = S & T)

Y_lo = S & T;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h09: begin //OR       (Y_lo = S | T)

Y_lo = S | T;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h0A: begin //XOR      (Y_lo = S ^ T)

Y_lo = S ^ T;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h0B: begin //NOR      (Y_lo = ~(S | T))

Y_lo = ~(S | T);

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

/*

* Shift Instructions are handled by Barrel Shift module *

5'h0C: begin //SLL      (Y_lo = T < 1) with 0 fill

Y_lo = {T[30:0], 1'b0};

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 1'bx, T[31]};

end

5'h0D: begin //SRL      (Y_lo = T > 1) with 0 fill

Y_lo = {1'b0, T[31:1]};

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 1'bx, T[0]};

end

5'h0E: begin //SRA(keeps MSB and shifts to the right)

Y_lo = {T[31], T[31:1]};

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 1'bx, T[0]};

end

*/

5'h0F: begin //INC      (Y_lo = S + 1)

{carry, Y_lo} = S + 1'b1;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) & (Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h10: begin //DEC      (Y_lo = S - 1)

Y_lo = S - 1'b1;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if (S == 32'b0)        carry = 1; else carry = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) & (Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h11: begin //INC4     (Y_lo = S + 4)

{carry, Y_lo} = S + 3'd4;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) & (Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h12: begin //DEC4     (Y_lo = S - 4)

Y_lo = S - 3'd4;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

if (S > 32'hFFFF_FFFB) carry = 1; else carry = 0;

if ( (S[31] == 1'b0) & (T[31] == 1'b0) &

(Y_lo[31] == 1'b1) )

ovf = 1'b1;

else if ( (S[31] == 1'b1) & (T[31] == 1'b1) &

(Y_lo[31] == 1'b0) )

ovf = 1'b1;

else

ovf = 1'b0;

{N, Z, V, C} = {neg, zero, ovf, carry};

end

5'h13: begin //ZEROS    (Y_lo = 0)

Y_lo = 32'h0;

{N, Z, V, C} = 4'b01xx;

end

5'h14: begin //ONES     (Y_lo = FFFF_FFFF)

Y_lo = 32'hFFFF_FFFF;

{N, Z, V, C} = 4'b10xx;

end

5'h15: begin //SP_INIT  (Y_lo = 32'h3FC)

Y_lo = 32'h3FC;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h16: begin //ANDI     (Y_lo = ( S & {16'h0, T[15:0]} ))

Y_lo = ( S & {16'h0, T[15:0]} );

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h17: begin //ORI      (Y_lo = ( S | {16'h0, T[15:0]} ))

Y_lo = ( S | {16'h0, T[15:0]} );

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h18: begin //LUI      (Y_lo = {T[15:0], 16'h0})

Y_lo = {T[15:0], 16'h0};

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

5'h19: begin //XORI     (Y_lo = ( S ^ {16'h0, T[15:0]} ))

Y_lo = ( S ^ {16'h0, T[15:0]} );

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

default: begin //default set to the PASS_S function

Y_lo = S;

if (Y_lo[31] == 1'b1)  neg  = 1; else neg  = 0;

if (Y_lo     == 32'b0) zero = 1; else zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};

end

endcase

//since Y_hi is never used in MIPS, set it to 0

Y_hi = 32'b0;

end

endmodule

MPY_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  MPY_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2018

*

* Purpose:    This module will take two 32 bit inputs and perform a mutlipy.

*             The result will be sent out as a 64 bit output.

*

* Notes:      This is the 32 bit multiplication module. It will multiply

*             32 bit A and B inputs, then send the resulting 64 bit Y as its

*             output. Prior to its multiplication step, this module will

*             change all negative inputs to positive values. After

*             multiplying, it will check the two inputs' sign bit and change

*             the result to a negative value if both sign bits are not equal

*             to each other.

*

****************************************************************************/

module MPY_32(S, T, Y_hi, Y_lo, N, Z, V, C);

input [31:0] S, T;

output reg [31:0] Y_hi, Y_lo;

output reg N, Z, V, C;

//cast S and T into the following integers

integer inta, intb;

reg neg, zero;

reg [63:0] mul;

always @(*)  begin

inta = S;

intb = T;

mul = inta * intb;

{Y_hi, Y_lo} = mul;                             //Product

if (mul[31] == 1'b1) neg  = 1;                  //Negative flag

else                 neg  = 0;

if ( (S == 32'b0) & (T == 32'b0) )              //Zero flag

zero = 1;

else

zero = 0;

{N, Z, V, C} = {neg, zero, 2'bxx};              //Set the output flags

end

endmodule

DIV_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  DIV_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2018

*

* Purpose:    This module will take two 32 bit inputs and perform a divide.

*             The result will be sent out as a 64 bit output.

*

* Notes:      This is the 32 bit division module. It will divide

*             32 bit A and B inputs, then send the resulting 64 bit Y as its

*             output. Y[63:32] will hold the remainder value while Y[31:0]

*             will hold the quotient value. Prior to its division step, this

*             module will change all negative inputs to positive values.

*             After dividing, it will check the two inputs' sign bit and

*             change the results to a negative quotient and remainder value

*             depending on the inputs' (divisor and dividend's) sign bit.

*             Overflow bit "Vt" will be set to 1 if the operation is a

*             division by zero.

*

****************************************************************************/

module DIV_32(S, T, Y_hi, Y_lo, N, Z, V, C);

input [31:0] S, T;

output reg [31:0] Y_hi, Y_lo;

output reg N, Z, V, C;

reg neg, zero;

reg [31:0] quot, rem;

//cast S and T into the following integers

integer inta;

integer intb;

always @(*) begin

inta = S;

intb = T;

quot = inta / intb;

rem  = inta % intb;

{Y_hi, Y_lo} = {rem, quot};                        //Quotient

if (quot[31] == 1'b1)   neg  = 1; else neg  = 0;   //Negative flag

if (Y_lo     == 32'b0)  zero = 1; else zero = 0;   //Zero flag

{N, Z, V, C} = {neg, zero, 2'bxx};                 //Set the output flags

end

endmodule

Barrel Shift

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  barrelSHIFT_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  11/20/2018

*

* Purpose:    Provides the ALU the ability to shift up to 32 times.

*             The Barrel Shift can Shift Left Logical(SLL), Shift Right

*             Logical(SRL) and Shift Right Arithmetic(SRA).

*

****************************************************************************/

module barrelSHIFT_32(D, shift_val, LRRA, Y, C_BS);

input  [31:0] D;           //32-bit input to be shifted

input   [4:0] shift_val;   //Amount of Shifts to be done

input   [1:0] LRRA;        //Type of Shift

output reg [31:0] Y;       //32-bit output of the Barrel Shift

output reg    C_BS;        //Carry Flag from the Shifts

always @(*)

if(LRRA == 2'b00) begin       //Left Shift Logical

case(shift_val)

5'd1:  Y = { D[30:0],  1'b0 };    5'd17: Y = { D[14:0], 17'b0 };

5'd2:  Y = { D[29:0],  2'b0 };    5'd18: Y = { D[13:0], 18'b0 };

5'd3:  Y = { D[28:0],  3'b0 };    5'd19: Y = { D[12:0], 19'b0 };

5'd4:  Y = { D[27:0],  4'b0 };    5'd20: Y = { D[11:0], 20'b0 };

5'd5:  Y = { D[26:0],  5'b0 };    5'd21: Y = { D[10:0], 21'b0 };

5'd6:  Y = { D[25:0],  6'b0 };    5'd22: Y = {  D[9:0], 22'b0 };

5'd7:  Y = { D[24:0],  7'b0 };    5'd23: Y = {  D[8:0], 23'b0 };

5'd8:  Y = { D[23:0],  8'b0 };    5'd24: Y = {  D[7:0], 24'b0 };

5'd9:  Y = { D[22:0],  9'b0 };    5'd25: Y = {  D[6:0], 25'b0 };

5'd10: Y = { D[21:0], 10'b0 };    5'd26: Y = {  D[5:0], 26'b0 };

5'd11: Y = { D[20:0], 11'b0 };    5'd27: Y = {  D[4:0], 27'b0 };

5'd12: Y = { D[19:0], 12'b0 };    5'd28: Y = {  D[3:0], 28'b0 };

5'd13: Y = { D[18:0], 13'b0 };    5'd29: Y = {  D[2:0], 29'b0 };

5'd14: Y = { D[17:0], 14'b0 };    5'd30: Y = {  D[1:0], 30'b0 };

5'd15: Y = { D[16:0], 15'b0 };    5'd31: Y = {    D[0], 31'b0 };

5'd16: Y = { D[15:0], 16'b0 };

default: Y = Y;

endcase

C_BS = D[31];

end

else if(LRRA == 2'b01) begin  //Right Shift Logical

case(shift_val)

5'd1:  Y = {  1'b0,   D[31:1] };    5'd17: Y = { 17'b0,  D[31:17] };

5'd2:  Y = {  2'b0,   D[31:2] };    5'd18: Y = { 18'b0,  D[31:18] };

5'd3:  Y = {  3'b0,   D[31:3] };    5'd19: Y = { 19'b0,  D[31:19] };

5'd4:  Y = {  4'b0,   D[31:4] };    5'd20: Y = { 20'b0,  D[31:20] };

5'd5:  Y = {  5'b0,   D[31:5] };    5'd21: Y = { 21'b0,  D[31:21] };

5'd6:  Y = {  6'b0,   D[31:6] };    5'd22: Y = { 22'b0,  D[31:22] };

5'd7:  Y = {  7'b0,   D[31:7] };    5'd23: Y = { 23'b0,  D[31:23] };

5'd8:  Y = {  8'b0,   D[31:8] };    5'd24: Y = { 24'b0,  D[31:24] };

5'd9:  Y = {  9'b0,   D[31:9] };    5'd25: Y = { 25'b0,  D[31:25] };

5'd10: Y = { 10'b0,  D[31:10] };    5'd26: Y = { 26'b0,  D[31:26] };

5'd11: Y = { 11'b0,  D[31:11] };    5'd27: Y = { 27'b0,  D[31:27] };

5'd12: Y = { 12'b0,  D[31:12] };    5'd28: Y = { 28'b0,  D[31:28] };

5'd13: Y = { 13'b0,  D[31:13] };    5'd29: Y = { 29'b0,  D[31:29] };

5'd14: Y = { 14'b0,  D[31:14] };    5'd30: Y = { 30'b0,  D[31:30] };

5'd15: Y = { 15'b0,  D[31:15] };    5'd31: Y = { 31'b0,     D[31] };

5'd16: Y = { 16'b0,  D[31:16] };

default: Y = Y;

endcase

C_BS = D[0];

end

else if (LRRA == 2'b10) begin //Right Shift Arithmetic

case(shift_val)

5'd1:  Y = {       D[31],  D[31:1] };

5'd2:  Y = { { 2{D[31]}},  D[31:2] };

5'd3:  Y = { { 3{D[31]}},  D[31:3] };

5'd4:  Y = { { 4{D[31]}},  D[31:4] };

5'd5:  Y = { { 5{D[31]}},  D[31:5] };

5'd6:  Y = { { 6{D[31]}},  D[31:6] };

5'd7:  Y = { { 7{D[31]}},  D[31:7] };

5'd8:  Y = { { 8{D[31]}},  D[31:8] };

5'd9:  Y = { { 9{D[31]}},  D[31:9] };

5'd10: Y = { {10{D[31]}}, D[31:10] };

5'd11: Y = { {11{D[31]}}, D[31:11] };

5'd12: Y = { {12{D[31]}}, D[31:12] };

5'd13: Y = { {13{D[31]}}, D[31:13] };

5'd14: Y = { {14{D[31]}}, D[31:14] };

5'd15: Y = { {15{D[31]}}, D[31:15] };

5'd16: Y = { {16{D[31]}}, D[31:16] };

5'd17: Y = { {17{D[31]}}, D[31:17] };

5'd18: Y = { {18{D[31]}}, D[31:18] };

5'd19: Y = { {19{D[31]}}, D[31:19] };

5'd20: Y = { {20{D[31]}}, D[31:20] };

5'd21: Y = { {21{D[31]}}, D[31:21] };

5'd22: Y = { {22{D[31]}}, D[31:22] };

5'd23: Y = { {23{D[31]}}, D[31:23] };

5'd24: Y = { {24{D[31]}}, D[31:24] };

5'd25: Y = { {25{D[31]}}, D[31:25] };

5'd26: Y = { {26{D[31]}}, D[31:26] };

5'd27: Y = { {27{D[31]}}, D[31:27] };

5'd28: Y = { {28{D[31]}}, D[31:28] };

5'd29: Y = { {29{D[31]}}, D[31:29] };

5'd30: Y = { {30{D[31]}}, D[31:30] };

5'd31: Y = { {31{D[31]}},    D[31] };

default: Y = Y;

endcase

C_BS = D[0];

end

endmodule

Rotate_32

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  rotate_32.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  11/20/2018

*

* Purpose:    Provides the ALU the ability to rotate up to 31 times.

*             The Rotate module can rotate left and right.

*

****************************************************************************/

module rotate_32(D, shift_val, LorR, Y);

input  [31:0] D;           //32-bit input to be rotated

input   [4:0] shift_val;   //Amount of Shifts to be done

input         LorR;        //Rotate rule

output reg [31:0] Y;       //32-bit output of the Barrel Shift

always @(*) begin

if(LorR == 1'b0)   //Rotate Left

case(shift_val)

5'd1:  Y = { D[30:0],    D[31] };  5'd17: Y = { D[14:0], D[31:15] };

5'd2:  Y = { D[29:0], D[31:30] };  5'd18: Y = { D[13:0], D[31:14] };

5'd3:  Y = { D[28:0], D[31:29] };  5'd19: Y = { D[12:0], D[31:13] };

5'd4:  Y = { D[27:0], D[31:28] };  5'd20: Y = { D[11:0], D[31:12] };

5'd5:  Y = { D[26:0], D[31:27] };  5'd21: Y = { D[10:0], D[31:11] };

5'd6:  Y = { D[25:0], D[31:26] };  5'd22: Y = {  D[9:0], D[31:10] };

5'd7:  Y = { D[24:0], D[31:25] };  5'd23: Y = {  D[8:0],  D[31:9] };

5'd8:  Y = { D[23:0], D[31:24] };  5'd24: Y = {  D[7:0],  D[31:8] };

5'd9:  Y = { D[22:0], D[31:23] };  5'd25: Y = {  D[6:0],  D[31:7] };

5'd10: Y = { D[21:0], D[31:22] };  5'd26: Y = {  D[5:0],  D[31:6] };

5'd11: Y = { D[20:0], D[31:21] };  5'd27: Y = {  D[4:0],  D[31:5] };

5'd12: Y = { D[19:0], D[31:20] };  5'd28: Y = {  D[3:0],  D[31:4] };

5'd13: Y = { D[18:0], D[31:19] };  5'd29: Y = {  D[2:0],  D[31:3] };

5'd14: Y = { D[17:0], D[31:18] };  5'd30: Y = {  D[1:0],  D[31:2] };

5'd15: Y = { D[16:0], D[31:17] };  5'd31: Y = {    D[0],  D[31:1] };

5'd16: Y = { D[15:0], D[31:16] };

default: Y = Y;

endcase

else              //Rotate Right

case(shift_val)

5'd1:  Y = {    D[0],  D[31:1] };  5'd17: Y = { D[16:0], D[31:17] };

5'd2:  Y = {  D[1:0],  D[31:2] };  5'd18: Y = { D[17:0], D[31:18] };

5'd3:  Y = {  D[2:0],  D[31:3] };  5'd19: Y = { D[18:0], D[31:19] };

5'd4:  Y = {  D[3:0],  D[31:4] };  5'd20: Y = { D[19:0], D[31:20] };

5'd5:  Y = {  D[4:0],  D[31:5] };  5'd21: Y = { D[20:0], D[31:21] };

5'd6:  Y = {  D[5:0],  D[31:6] };  5'd22: Y = { D[21:0], D[31:22] };

5'd7:  Y = {  D[6:0],  D[31:7] };  5'd23: Y = { D[22:0], D[31:23] };

5'd8:  Y = {  D[7:0],  D[31:8] };  5'd24: Y = { D[23:0], D[31:24] };

5'd9:  Y = {  D[8:0],  D[31:9] };  5'd25: Y = { D[24:0], D[31:25] };

5'd10: Y = {  D[9:0], D[31:10] };  5'd26: Y = { D[25:0], D[31:26] };

5'd11: Y = { D[10:0], D[31:11] };  5'd27: Y = { D[26:0], D[31:27] };

5'd12: Y = { D[11:0], D[31:12] };  5'd28: Y = { D[27:0], D[31:28] };

5'd13: Y = { D[12:0], D[31:13] };  5'd29: Y = { D[28:0], D[31:29] };

5'd14: Y = { D[13:0], D[31:14] };  5'd30: Y = { D[29:0], D[31:30] };

5'd15: Y = { D[14:0], D[31:15] };  5'd31: Y = { D[30:0],    D[31] };

5'd16: Y = { D[15:0], D[31:16] };

default: Y = Y;

endcase

end

endmodule

DataMemory

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  DataMemory.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2018

*

* Purpose:    This is the 4098x8 bit readable/writable data memory module.

*

* Notes:      This module will be used to read and store memory data.

*             To read or write data, chip select "dm_cs" must be set to 1.

*             To read, "dm_rd" must be set to 1. To write, "dm_wr" must be

*             set to 1. Data will be accessed through big endian format.

*

****************************************************************************/

module DataMemory(clk, Address, D_In, dm_cs, dm_wr, dm_rd, D_Out);

input             clk, dm_cs, dm_wr, dm_rd;

input      [31:0] Address, D_In;

output     [31:0] D_Out;

//use 1024x32 'big endian' MSB goes to smaller address

reg [7:0] M [0:4095];  //4098x8 memory

always @(posedge clk)   begin

//write

if(dm_cs && dm_wr)   begin

M[Address]     <= D_In[31:24];

M[Address + 1] <= D_In[23:16];

M[Address + 2] <= D_In[15:8];

M[Address + 3] <= D_In[7:0];

end

end

//asynchronous continuous output READ

assign D_Out = (dm_cs && dm_rd) ?

{M[Address], M[Address + 1], M[Address + 2], M[Address + 3]} :

32'hZZZZ_ZZZZ;   //high impedance

endmodule

InputOutput

`timescale 1ns / 1ps

/****************************** C E C S  4 4 0 ******************************

*

* File Name:  InputOutput.v

* Project:    CECS 440 Senior Project - Enhanced MIPS Processor

* Designer:   Benjamin Santos and Naoaki Takatsu

* Email:      benjaminsantos@gmx.com

*             naoaki.takatsu@student.csulb.edu

* Rev. No.:   Version 1.0

* Rev. Date:  10/7/2018

*

* Purpose:    This is the 4098x8 bit readable/writable I/O memory module.

*

* Notes:      This module will be used to read and store I/O data.

*             To read or write data, chip select "io_cs" must be set to 1.

*             To read, "io_rd" must be set to 1. To write, "io_wr" must be

*             set to 1. Data will be accessed through big endian format.

*

****************************************************************************/

module InputOutput(clk, Address, D_In, io_cs, io_wr, io_rd, D_Out, int_ack, intr);

input             clk, io_cs, io_wr, io_rd;

input             int_ack;

input      [31:0] Address, D_In;

output     [31:0] D_Out;

output reg        intr;

//use 1024x32 'big endian' MSB goes to smaller address

reg [7:0] IO [0:4095];  //4098x8 memory

always @(posedge clk)   begin

//write

if(io_cs && io_wr)   begin

IO[Address]     <= D_In[31:24];

IO[Address + 1] <= D_In[23:16];

IO[Address + 2] <= D_In[15:8];

IO[Address + 3] <= D_In[7:0];

end

end

//asynchronous continuous output READ

assign D_Out = (io_cs && io_rd) ?

{IO[Address], IO[Address + 1], IO[Address + 2], IO[Address + 3]} :

32'hZZZZ_ZZZZ;   //high impedance

//Deassert Interrupt when Interrupt is acknowledged by the CPU

always @(posedge int_ack)

intr <= 1'b0;

endmodule

Instruction Memory Module 1

Testing Instructions: LUI, ORI, ADD, BEQ, BNE, SW, BREAK

@0

3c 01 12 34  // main:   lui  $01, 0x1234

34 21 56 78  //         ori  $01, 0x5678      # LI   R01,  0x12345678

3c 02 87 65  //         lui  $02, 0x8765

34 42 43 21  //         ori  $02, 0x4321      # LI   R02,  0x87654321

00 01 18 20  //         add  $03, $00, $01    # COPY R03, R01

10 22 00 01  //         beq  $01, $02, no_eq  # should not branch

10 23 00 03  //         beq  $01, $03, yes_eq # should branch

3c 0e ff ff  // no_eq:  lui  $14, 0xFFFF

35 ce ff ff  //         ori  $14, 0xFFFF # LI R14,0xFFFFFFFF"fail flag"

00 00 00 0d  //         break

00 00 70 20  // yes_eq: add  $14, $0, $0      # CLR  R14  "pass flag"

14 23 00 01  //         bne  $01, $03, no_ne  # should not branch

14 22 00 03  //         bne  $01, $02, yes_ne # should branch

3c 0f ff ff  // no_ne:  lui  $15, 0xFFFF

35 ef ff ff  //         ori  $15, 0xFFFF # LI R15 0xFFFFFFFF"fail flag"

00 00 00 0d  //         break

00 00 78 20  // yes_ne: add  $15, $0, $0      # CLR  R15  "pass flag"

3c 0d 10 01  //         lui  $13, 0x1001

35 ad 00 c0  //         ori  $13, 0x00C0      # LI   R13,  0x100100C0

ad a1 00 00  //         sw   $01, 0($13)      # ST  [R13], R01

00 00 00 0d  //         break

- Registers $r1 and $r2 gets their respective values from the LUI and ORI instructions where 
  LUI fills the upper 16 bits and ORI fills the lower 16 bits of the data.

- Register $r3 gets its value from the ADD instruction adding $r1 and $zero

- Since register $r14 has 0x00000000, this proves that the BEQ instruction is working as 
  intended

- Since register $r15 has 0x00000000, this proves that the BNE instruction is working 
  correctly

- Since data in Memory Location 0x0C0 has 0x12345678, this proves that the SW instruction 
  is working correctly

- Since the final value of IR has 0x0000000d, this proves the functionality of the BREAK
   instruction

BREAK INSTRUCTION FETCHED  700.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t= 710.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t= 720.0 ps, $r1  = 0x12345678  ||   $r17 = 0xxxxxxxxx

t= 730.0 ps, $r2  = 0x87654321  ||   $r18 = 0xxxxxxxxx

t= 740.0 ps, $r3  = 0x12345678  ||   $r19 = 0xxxxxxxxx

t= 750.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t= 760.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t= 770.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t= 780.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t= 790.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t= 800.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t= 810.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t= 820.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t= 830.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t= 840.0 ps, $r13 = 0x100100c0  ||   $r29 = 0x000003fc

t= 850.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t= 860.0 ps, $r15 = 0x00000000  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t= 870.0 ps, M[000000c0] = 0x12345678

t= 880.0 ps, M[000000c4] = 0xxxxxxxxx

t= 890.0 ps, M[000000c8] = 0xxxxxxxxx

t= 900.0 ps, M[000000cc] = 0xxxxxxxxx

t= 910.0 ps, M[000000d0] = 0xxxxxxxxx

t= 920.0 ps, M[000000d4] = 0xxxxxxxxx

t= 930.0 ps, M[000000d8] = 0xxxxxxxxx

t= 940.0 ps, M[000000dc] = 0xxxxxxxxx

t= 950.0 ps, M[000000e0] = 0xxxxxxxxx

t= 960.0 ps, M[000000e4] = 0xxxxxxxxx

t= 970.0 ps, M[000000e8] = 0xxxxxxxxx

t= 980.0 ps, M[000000ec] = 0xxxxxxxxx

t= 990.0 ps, M[000000f0] = 0xxxxxxxxx

t=1000.0 ps, M[000000f4] = 0xxxxxxxxx

t=1010.0 ps, M[000000f8] = 0xxxxxxxxx

t=1020.0 ps, M[000000fc] = 0xxxxxxxxx

t=1030.0 ps, PC = 0x00000054,   IR = 0x0000000d

Instruction Memory Module 2

Testing Instructions:  ADDI, SRL, J

@0

3c 01 ff ff  // main: lui  $01, 0xFFFF

34 21 ff ff  //       ori  $01, 0xFFFF     # LI   R01,  0xFFFFFFF

20 02 00 10  //       addi $02, $00, 0x10  # LI   R02,  0x10

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0     # LI   R15,  0x100100C0

00 01 08 42  // top:  srl  $01, $01, 1     # logical shift right 1 bit

ad e1 00 00  //       sw   $01, 0($15)     # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4     # inc memory pointer by 4

20 42 ff ff  //       addi $02, $02, -1    # decrement the loop counter

14 40 ff fb  //       bne  $02, $00, top   # and jmp to top if not done

08 10 00 0c  //       j    exit            # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C     # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

$r2 is decremented by 1, sixteen times until it reaches 0 and JUMPS to the exit.

$r15 from 0x100100C0 incremented by 4, sixteen times and resulted into 0x10010100
- Register $r2 having the value 0x00000000 proves that the ADDI function works correctly.

- Since register $r14 has 0x5A5A3C3C, this proves that the J instruction is working as 
  intended

- Values inside the Memory Locations 0xC0 to 0xFF proves that both SRL and SW functions 
  work correctly.

BREAK INSTRUCTION FETCHED 3650.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=3660.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=3670.0 ps, $r1  = 0x0000ffff  ||   $r17 = 0xxxxxxxxx

t=3680.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=3690.0 ps, $r3  = 0xxxxxxxxx  ||   $r19 = 0xxxxxxxxx

t=3700.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=3710.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=3720.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=3730.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=3740.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=3750.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=3760.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=3770.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=3780.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=3790.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=3800.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=3810.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=3820.0 ps, M[000000c0] = 0x7fffffff

t=3830.0 ps, M[000000c4] = 0x3fffffff

t=3840.0 ps, M[000000c8] = 0x1fffffff

t=3850.0 ps, M[000000cc] = 0x0fffffff

t=3860.0 ps, M[000000d0] = 0x07ffffff

t=3870.0 ps, M[000000d4] = 0x03ffffff

t=3880.0 ps, M[000000d8] = 0x01ffffff

t=3890.0 ps, M[000000dc] = 0x00ffffff

t=3900.0 ps, M[000000e0] = 0x007fffff

t=3910.0 ps, M[000000e4] = 0x003fffff

t=3920.0 ps, M[000000e8] = 0x001fffff

t=3930.0 ps, M[000000ec] = 0x000fffff

t=3940.0 ps, M[000000f0] = 0x0007ffff

t=3950.0 ps, M[000000f4] = 0x0003ffff

t=3960.0 ps, M[000000f8] = 0x0001ffff

t=3970.0 ps, M[000000fc] = 0x0000ffff

t=3980.0 ps, PC = 0x0040003c,   IR = 0x0000000d

Instruction Memory Module 3

Testing Instructions: SRA

@0

3c 01 80 00  // main: lui  $01, 0x8000

34 21 ff ff  //       ori  $01, 0xFFFF    # LI   R01,  0x8000FFFF

20 02 00 10  //       addi $02, $00, 0x10 # LI   R02,  0x10

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0    # LI   R15,  0x100100C0

00 01 08 43  // top:  sra  $01, $01, 1    # logical shift right 1 bit

ad e1 00 00  //       sw   $01, 0($15)    # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4    # inc memory pointer by 4

20 42 ff ff  //       addi $02, $02, -1   # decrement the loop counter

14 40 ff fb  //       bne  $02, $00, top  # and jmp to top if not done

08 10 00 0c  //       j    exit           # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C    # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

- SRA instruction is done 16 times on register $r1 and changes original value from
  0x8000FFFF to 0xFFFF8000

- Values inside the Memory Locations 0xC0 to 0xFF proves that both SRA and SW functions 
  work correctly.

- Since register $r14 has 0x5A5A3C3C, this proves that the all the instructions are working as

intended

BREAK INSTRUCTION FETCHED 3650.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=3660.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=3670.0 ps, $r1  = 0xffff8000  ||   $r17 = 0xxxxxxxxx

t=3680.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=3690.0 ps, $r3  = 0xxxxxxxxx  ||   $r19 = 0xxxxxxxxx

t=3700.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=3710.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=3720.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=3730.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=3740.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=3750.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=3760.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=3770.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=3780.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=3790.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=3800.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=3810.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=3820.0 ps, M[000000c0] = 0xc0007fff

t=3830.0 ps, M[000000c4] = 0xe0003fff

t=3840.0 ps, M[000000c8] = 0xf0001fff

t=3850.0 ps, M[000000cc] = 0xf8000fff

t=3860.0 ps, M[000000d0] = 0xfc0007ff

t=3870.0 ps, M[000000d4] = 0xfe0003ff

t=3880.0 ps, M[000000d8] = 0xff0001ff

t=3890.0 ps, M[000000dc] = 0xff8000ff

t=3900.0 ps, M[000000e0] = 0xffc0007f

t=3910.0 ps, M[000000e4] = 0xffe0003f

t=3920.0 ps, M[000000e8] = 0xfff0001f

t=3930.0 ps, M[000000ec] = 0xfff8000f

t=3940.0 ps, M[000000f0] = 0xfffc0007

t=3950.0 ps, M[000000f4] = 0xfffe0003

t=3960.0 ps, M[000000f8] = 0xffff0001

t=3970.0 ps, M[000000fc] = 0xffff8000

t=3980.0 ps, PC = 0x0040003c,   IR = 0x0000000d

Instruction Memory Module 4

Testing Instructions: SLL, SLT

@0

3c 01 ff ff  // main: lui  $01, 0xFFFF

34 21 ff ff  //       ori  $01, 0xFFFF     # LI   R01,  0xFFFFFFF

20 02 00 10  //       addi $02, $00, 0x10  # LI   R02,  0x10

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0     # LI   R15,  0x100100C0

00 01 08 40  // top:  sll  $01, $01, 1     # logical shift left 1 bit

ad e1 00 00  //       sw   $01, 0($15)     # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4     # inc memory pointer by 4

20 42 ff ff  //       addi $02, $02, -1    # decrement the loop counter

00 02 18 2a  //       slt  $03, $00, $02   # r3 <--1 if r0 < r2

14 60 ff fa  //       bne  $03, $00, top   # jmp if r3==1

08 10 00 0d  //       j    exit            # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C     # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

- SLL instruction is done 16 times on register $r1 and changes original value from
  0xFFFFFFFF to 0xFFFF0000

- SLT instruction sets register $r3 to 1 if register $r0 is less than $r2 and is used with  
  conjunction with the ADDI and BNE instructions to loop 16 times.

- Values inside the Memory Locations 0xC0 to 0xFF proves that both SLL and SW functions 
  work correctly.

- Since register $r14 has 0x5A5A3C3C, this proves that the all the instructions are working as

intended

BREAK INSTRUCTION FETCHED 4290.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=4300.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=4310.0 ps, $r1  = 0xffff0000  ||   $r17 = 0xxxxxxxxx

t=4320.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=4330.0 ps, $r3  = 0x00000000  ||   $r19 = 0xxxxxxxxx

t=4340.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=4350.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=4360.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=4370.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=4380.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=4390.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=4400.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=4410.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=4420.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=4430.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=4440.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=4450.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=4460.0 ps, M[000000c0] = 0xfffffffe

t=4470.0 ps, M[000000c4] = 0xfffffffc

t=4480.0 ps, M[000000c8] = 0xfffffff8

t=4490.0 ps, M[000000cc] = 0xfffffff0

t=4500.0 ps, M[000000d0] = 0xffffffe0

t=4510.0 ps, M[000000d4] = 0xffffffc0

t=4520.0 ps, M[000000d8] = 0xffffff80

t=4530.0 ps, M[000000dc] = 0xffffff00

t=4540.0 ps, M[000000e0] = 0xfffffe00

t=4550.0 ps, M[000000e4] = 0xfffffc00

t=4560.0 ps, M[000000e8] = 0xfffff800

t=4570.0 ps, M[000000ec] = 0xfffff000

t=4580.0 ps, M[000000f0] = 0xffffe000

t=4590.0 ps, M[000000f4] = 0xffffc000

t=4600.0 ps, M[000000f8] = 0xffff8000

t=4610.0 ps, M[000000fc] = 0xffff0000

t=4620.0 ps, PC = 0x00400040,   IR = 0x0000000d

Instruction Memory Module 5

Testing Instructions: SLTI

@0

3c 01 ff ff  // main: lui  $01, 0xFFFF

34 21 ff ff  //       ori  $01, 0xFFFF     # LI   R01,  0xFFFFFFF

20 02 ff f0  //       addi $02, $00, -16   # LI   R02,  -16

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0     # LI   R15,  0x100100C0

00 01 08 40  // top:  sll  $01, $01, 1     # logical shift left 1 bit

ad e1 00 00  //       sw   $01, 0($15)     # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4     # inc memory pointer by 4

20 42 00 01  //       addi $02, $02, 1     # increment the loop counter

28 43 00 00  //       slti $03, $02, 0     # r3 <--1 if r2 < 0

14 60 ff fa  //       bne  $03, $00, top   # jmp if r3==1

08 10 00 0d  //       j    exit            # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C     # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

- SLL instruction is done 16 times on register $r1 and changes original value from
  0xFFFFFFFF to 0xFFFF0000

- SLTI instruction sets register $r3 to 1 if register $r0 is less than $r2 and is used with  
  conjunction with the ADDI and BNE instructions to loop 16 times.

- Values inside the Memory Locations 0xC0 to 0xFF proves that both SLTI and SW functions 
  work correctly.

- Since register $r14 has 0x5A5A3C3C, this proves that the all the instructions are working as

intended

BREAK INSTRUCTION FETCHED 4290.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=4300.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=4310.0 ps, $r1  = 0xffff0000  ||   $r17 = 0xxxxxxxxx

t=4320.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=4330.0 ps, $r3  = 0x00000000  ||   $r19 = 0xxxxxxxxx

t=4340.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=4350.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=4360.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=4370.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=4380.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=4390.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=4400.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=4410.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=4420.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=4430.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=4440.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=4450.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=4460.0 ps, M[000000c0] = 0xfffffffe

t=4470.0 ps, M[000000c4] = 0xfffffffc

t=4480.0 ps, M[000000c8] = 0xfffffff8

t=4490.0 ps, M[000000cc] = 0xfffffff0

t=4500.0 ps, M[000000d0] = 0xffffffe0

t=4510.0 ps, M[000000d4] = 0xffffffc0

t=4520.0 ps, M[000000d8] = 0xffffff80

t=4530.0 ps, M[000000dc] = 0xffffff00

t=4540.0 ps, M[000000e0] = 0xfffffe00

t=4550.0 ps, M[000000e4] = 0xfffffc00

t=4560.0 ps, M[000000e8] = 0xfffff800

t=4570.0 ps, M[000000ec] = 0xfffff000

t=4580.0 ps, M[000000f0] = 0xffffe000

t=4590.0 ps, M[000000f4] = 0xffffc000

t=4600.0 ps, M[000000f8] = 0xffff8000

t=4610.0 ps, M[000000fc] = 0xffff0000

t=4620.0 ps, PC = 0x00400040,   IR = 0x0000000d

Instruction Memory Module 6

Testing Instructions: LW

@0

3c 0f 10 01  // lui  $15, 0x1001

35 ef 00 00  // ori  $15, 0x0000        # LI R15,0x10010000 dest ptr

3c 0e 10 01  // lui  $14, 0x1001

35 ce 00 c0  // ori  $14, 0x00C0        # LI R14,0x100100C0 dest ptr

20 0d 00 10  // addi $13, $00, 16       # LI R13, 16  loop counter

8d e1 00 04  // lw   $01, 04($15)       # Load

8d e2 00 08  // lw   $02, 08($15)       #   R01

8d e3 00 0c  // lw   $03, 12($15)       #     to

8d e4 00 10  // lw   $04, 16($15)       #      R12

8d e5 00 14  // lw   $05, 20($15)

8d e6 00 18  // lw   $06, 24($15)

8d e7 00 1c  // lw   $07, 28($15)

8d e8 00 20  // lw   $08, 32($15)

8d e9 00 24  // lw   $09, 36($15)

8d ea 00 28  // lw   $10, 40($15)

8d eb 00 2c  // lw   $11, 44($15)

8d ec 00 30  // lw   $12, 48($15)

// mem2mem:

8d f1 00 00  // lw   $17, 00($15)       # do mem to

ad d1 00 00  // sw   $17, 00($14)       #   mem transfer

21 ef 00 04  // addi $15, $15, 04       # bump both source

21 ce 00 04  // addi $14, $14, 04       #   and dest pointers

21 ad ff ff  // addi $13, $13, -1       # dec the loop counter

15 a0 ff fa  // bne  $13, $00, mem2mem  #   and continue till done

00 00 00 0d  // break

- Registers $r1 to $r12 are loaded in data from dM[0x04 to 0x33] using LW

- Registers from dM[0x00 to 0x3F] are transferred to dM[0xC0 to 0xFF] using LW and SW in 
  a loop

- Loops 16 times using ADDI and BNE

- Values inside the Memory Locations 0xC0 to 0xFF proves that all the instructions are 
   working as intended

BREAK INSTRUCTION FETCHED 4940.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=4950.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=4960.0 ps, $r1  = 0x12345678  ||   $r17 = 0x000075cc

t=4970.0 ps, $r2  = 0x89abcdef  ||   $r18 = 0xxxxxxxxx

t=4980.0 ps, $r3  = 0xa5a5a5a5  ||   $r19 = 0xxxxxxxxx

t=4990.0 ps, $r4  = 0x5a5a5a5a  ||   $r20 = 0xxxxxxxxx

t=5000.0 ps, $r5  = 0x2468ace0  ||   $r21 = 0xxxxxxxxx

t=5010.0 ps, $r6  = 0x13579bdf  ||   $r22 = 0xxxxxxxxx

t=5020.0 ps, $r7  = 0x0f0f0f0f  ||   $r23 = 0xxxxxxxxx

t=5030.0 ps, $r8  = 0xf0f0f0f0  ||   $r24 = 0xxxxxxxxx

t=5040.0 ps, $r9  = 0x00000009  ||   $r25 = 0xxxxxxxxx

t=5050.0 ps, $r10 = 0x0000000a  ||   $r26 = 0xxxxxxxxx

t=5060.0 ps, $r11 = 0x0000000b  ||   $r27 = 0xxxxxxxxx

t=5070.0 ps, $r12 = 0x0000000c  ||   $r28 = 0xxxxxxxxx

t=5080.0 ps, $r13 = 0x00000000  ||   $r29 = 0x000003fc

t=5090.0 ps, $r14 = 0x10010100  ||   $r30 = 0xxxxxxxxx

t=5100.0 ps, $r15 = 0x10010040  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=5110.0 ps, M[000000c0] = 0xc3c3c3c3

t=5120.0 ps, M[000000c4] = 0x12345678

t=5130.0 ps, M[000000c8] = 0x89abcdef

t=5140.0 ps, M[000000cc] = 0xa5a5a5a5

t=5150.0 ps, M[000000d0] = 0x5a5a5a5a

t=5160.0 ps, M[000000d4] = 0x2468ace0

t=5170.0 ps, M[000000d8] = 0x13579bdf

t=5180.0 ps, M[000000dc] = 0x0f0f0f0f

t=5190.0 ps, M[000000e0] = 0xf0f0f0f0

t=5200.0 ps, M[000000e4] = 0x00000009

t=5210.0 ps, M[000000e8] = 0x0000000a

t=5220.0 ps, M[000000ec] = 0x0000000b

t=5230.0 ps, M[000000f0] = 0x0000000c

t=5240.0 ps, M[000000f4] = 0x0000000d

t=5250.0 ps, M[000000f8] = 0xfffffff8

t=5260.0 ps, M[000000fc] = 0x000075cc

t=5270.0 ps, PC = 0x00000060,   IR = 0x0000000d

Instruction Memory Module 7

Testing Instructions: JAL, JR

@0

3c 0f 10 01  // main:    lui  $15, 0x1001

35 ef 00 00  //          ori  $15, 0x0000   # LI R15, 0x10010000 dest

3c 0e 10 01  //          lui  $14, 0x1001

35 ce 00 c0  //          ori  $14, 0x00C0   # LI R14, 0x100100C0 dest

20 0d 00 10  //          addi $13, $00, 16  # LI R13, 16 loop counter

8d e1 00 04  //          lw   $01, 04($15)  # Load

8d e2 00 08  //          lw   $02, 08($15)  #   R01

8d e3 00 0c  //          lw   $03, 12($15)  #     to

8d e4 00 10  //          lw   $04, 16($15)  #      R12

8d e5 00 14  //          lw   $05, 20($15)

8d e6 00 18  //          lw   $06, 24($15)

8d e7 00 1c  //          lw   $07, 28($15)

8d e8 00 20  //          lw   $08, 32($15)

8d e9 00 24  //          lw   $09, 36($15)

8d ea 00 28  //          lw   $10, 40($15)

8d eb 00 2c  //          lw   $11, 44($15)

8d ec 00 30  //          lw   $12, 48($15)

0c 10 00 15  //          jal  mem2mem

3c 0f ff ff  //          lui  $15, 0xFFFF

35 ef ff ff  //          ori  $15, 0xFFFF#LI R15,0xFFFFFFFF "pass flag"

00 00 00 0d  //          break

8d f1 00 00  // mem2mem: lw   $17, 00($15)       # do mem to

ad d1 00 00  //          sw   $17, 00($14)       #   mem transfer

21 ef 00 04  //          addi $15, $15, 04       # bump both source

21 ce 00 04  //          addi $14, $14, 04       #   and dest pointers

21 ad ff ff  //          addi $13, $13, -1       # dec the loop counter

15 a0 ff fa  //          bne  $13, $00, mem2mem  #  and continue til

03 e0 00 08  //          jr   $31                # return to old PC

00 00 00 0d  //          break                   # safety net

- The JAL instruction writes current PC to $r31 and jumps to mem2mem subroutine

- The JR instruction jumps back to the old saved PC in register $r31

- Registers $r1 to $r12 are loaded in data from dM[0x04 to 0x33] using LW

- Registers from dM[0x00 to 0x3F] are transferred to dM[0xC0 to 0xFF] using LW and SW in 
  a loop

- Loops 16 times using ADDI and BNE

BREAK INSTRUCTION FETCHED 5100.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=5110.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=5120.0 ps, $r1  = 0x12345678  ||   $r17 = 0x000075cc

t=5130.0 ps, $r2  = 0x89abcdef  ||   $r18 = 0xxxxxxxxx

t=5140.0 ps, $r3  = 0xa5a5a5a5  ||   $r19 = 0xxxxxxxxx

t=5150.0 ps, $r4  = 0x5a5a5a5a  ||   $r20 = 0xxxxxxxxx

t=5160.0 ps, $r5  = 0x2468ace0  ||   $r21 = 0xxxxxxxxx

t=5170.0 ps, $r6  = 0x13579bdf  ||   $r22 = 0xxxxxxxxx

t=5180.0 ps, $r7  = 0x0f0f0f0f  ||   $r23 = 0xxxxxxxxx

t=5190.0 ps, $r8  = 0xf0f0f0f0  ||   $r24 = 0xxxxxxxxx

t=5200.0 ps, $r9  = 0x00000009  ||   $r25 = 0xxxxxxxxx

t=5210.0 ps, $r10 = 0x0000000a  ||   $r26 = 0xxxxxxxxx

t=5220.0 ps, $r11 = 0x0000000b  ||   $r27 = 0xxxxxxxxx

t=5230.0 ps, $r12 = 0x0000000c  ||   $r28 = 0xxxxxxxxx

t=5240.0 ps, $r13 = 0x00000000  ||   $r29 = 0x000003fc

t=5250.0 ps, $r14 = 0x10010100  ||   $r30 = 0xxxxxxxxx

t=5260.0 ps, $r15 = 0xffffffff  ||   $r31 = 0x00000048

Memory Locations 0xC0 to 0xFF

t=5270.0 ps, M[000000c0] = 0xc3c3c3c3

t=5280.0 ps, M[000000c4] = 0x12345678

t=5290.0 ps, M[000000c8] = 0x89abcdef

t=5300.0 ps, M[000000cc] = 0xa5a5a5a5

t=5310.0 ps, M[000000d0] = 0x5a5a5a5a

t=5320.0 ps, M[000000d4] = 0x2468ace0

t=5330.0 ps, M[000000d8] = 0x13579bdf

t=5340.0 ps, M[000000dc] = 0x0f0f0f0f

t=5350.0 ps, M[000000e0] = 0xf0f0f0f0

t=5360.0 ps, M[000000e4] = 0x00000009

t=5370.0 ps, M[000000e8] = 0x0000000a

t=5380.0 ps, M[000000ec] = 0x0000000b

t=5390.0 ps, M[000000f0] = 0x0000000c

t=5400.0 ps, M[000000f4] = 0x0000000d

t=5410.0 ps, M[000000f8] = 0xfffffff8

t=5420.0 ps, M[000000fc] = 0x000075cc

t=5430.0 ps, PC = 0x00000054,   IR = 0x0000000d

Instruction Memory Module 8

Testing Instructions: MULT, MFLO, MFHI

@0

3c 0f 10 01  // main:      lui  $15, 0x1001

35 ef 00 00  //            ori  $15, 0x0000      # $r15 <-- 0x10010000 
                                                          (src pointer)

8d e1 00 00  //            lw   $01, 00($15)     # $r01 <--     25

8d e2 00 04  //            lw   $02, 04($15)     # $r02 <--   1000

8d e3 00 08  //            lw   $03, 08($15)     # $r03 <--    -25

8d e4 00 0c  //            lw   $04, 12($15)     # $r04 <--  -1000

8d e5 00 10  //            lw   $05, 16($15)     # $r05 <--  25000

8d e6 00 14  //            lw   $06, 20($15)     # $r06 <-- -25000

8d e7 00 18  //            lw   $07, 24($15)     # $r07 <--     -1

00 22 00 18  //            mult $01, $02

00 00 40 12  //            mflo $08              # rs=pos rt=pos rd=pos

14 a8 00 10  //            bne  $05, $08, fail1

00 62 00 18  //            mult $03, $02

00 00 48 12  //            mflo $09              # rs=neg rt=pos rd=neg

00 00 50 10  //            mfhi $10

14 c9 00 0f  //            bne  $06, $09, fail2L

14 ea 00 11  //            bne  $07, $10, fail2H

00 24 00 18  //            mult $01, $04

00 00 58 12  //            mflo $11              # rs=pos rt=neg rd=neg

00 00 60 10  //            mfhi $12

14 cb 00 10  //            bne  $06, $11, fail3L

14 ec 00 12  //            bne  $07, $12, fail3H

00 64 00 18  //            mult $03, $04

00 00 68 12  //            mflo $13              # rs=neg rt=neg rd=pos

14 ad 00 12  //            bne  $05, $13, fail4

3c 0e 00 00  // pass:      lui  $14, 0x0000

35 ce 00 00  //            ori  $14, 0x0000      # $r14 <-- 0x00000000

00 00 00 0d  //            break		    # (Pass flag)

/* Rest of ‘fail flags’ on the next page */

BREAK INSTRUCTION FETCHED 1190.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=1200.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=1210.0 ps, $r1  = 0x00000019  ||   $r17 = 0xxxxxxxxx

t=1220.0 ps, $r2  = 0x000003e8  ||   $r18 = 0xxxxxxxxx

t=1230.0 ps, $r3  = 0xffffffe7  ||   $r19 = 0xxxxxxxxx

t=1240.0 ps, $r4  = 0xfffffc18  ||   $r20 = 0xxxxxxxxx

t=1250.0 ps, $r5  = 0x000061a8  ||   $r21 = 0xxxxxxxxx

t=1260.0 ps, $r6  = 0xffff9e58  ||   $r22 = 0xxxxxxxxx

t=1270.0 ps, $r7  = 0xffffffff  ||   $r23 = 0xxxxxxxxx

t=1280.0 ps, $r8  = 0x000061a8  ||   $r24 = 0xxxxxxxxx

t=1290.0 ps, $r9  = 0xffff9e58  ||   $r25 = 0xxxxxxxxx

t=1300.0 ps, $r10 = 0xffffffff  ||   $r26 = 0xxxxxxxxx

t=1310.0 ps, $r11 = 0xffff9e58  ||   $r27 = 0xxxxxxxxx

t=1320.0 ps, $r12 = 0xffffffff  ||   $r28 = 0xxxxxxxxx

t=1330.0 ps, $r13 = 0x000061a8  ||   $r29 = 0x000003fc

t=1340.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t=1350.0 ps, $r15 = 0x10010000  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=1360.0 ps, M[000000c0] = 0xxxxxxxxx

t=1370.0 ps, M[000000c4] = 0xxxxxxxxx

t=1380.0 ps, M[000000c8] = 0xxxxxxxxx

t=1390.0 ps, M[000000cc] = 0xxxxxxxxx

t=1400.0 ps, M[000000d0] = 0xxxxxxxxx

t=1410.0 ps, M[000000d4] = 0xxxxxxxxx

t=1420.0 ps, M[000000d8] = 0xxxxxxxxx

t=1430.0 ps, M[000000dc] = 0xxxxxxxxx

t=1440.0 ps, M[000000e0] = 0xxxxxxxxx

t=1450.0 ps, M[000000e4] = 0xxxxxxxxx

t=1460.0 ps, M[000000e8] = 0xxxxxxxxx

t=1470.0 ps, M[000000ec] = 0xxxxxxxxx

t=1480.0 ps, M[000000f0] = 0xxxxxxxxx

t=1490.0 ps, M[000000f4] = 0xxxxxxxxx

t=1500.0 ps, M[000000f8] = 0xxxxxxxxx

t=1510.0 ps, M[000000fc] = 0xxxxxxxxx

t=1520.0 ps, PC = 0x00000070,   IR = 0x0000000d

Instruction Memory Module 8 (page 2)

Testing Instructions: MULT, MFLO, MFHI

3c 0e ff ff  // fail1:     lui  $14, 0xFFFF

35 ce ff ff  //            ori  $14, 0xFFFF      # $r14 <-- 0xFFFFFFFF

00 00 00 0d  //            break		    # (Fail flag 1)

3c 0e ff ff  // fail2L:    lui  $14, 0xFFFF

35 ce ff fe  //            ori  $14, 0xFFFE      # $r14 <-- 0xFFFFFFFE

00 00 00 0d  //            break                 # (Fail flag 2L)

3c 0e ff ff  // fail2H:    lui  $14, 0xFFFF

35 ce ff fd  //            ori  $14, 0xFFFD      # $r14 <--0xFFFFFFFD  
00 00 00 0d  //            break 		    # (Fail flag 2H)

3c 0e ff ff  // fail3L:    lui  $14, 0xFFFF

35 ce ff fc  //            ori  $14, 0xFFFC      # $r14 <-- 0xFFFFFFFC

00 00 00 0d  //            break		    # (Fail flag 3L)

3c 0e ff ff  // fail3H:    lui  $14, 0xFFFF

35 ce ff fb  //            ori  $14, 0xFFFB      # $r14 <-- 0xFFFFFFFB

00 00 00 0d  //            break		    # (Fail flag 3H)

3c 0e ff ff  // fail4:     lui  $14, 0xFFFF

35 ce ff fa  //            ori  $14, 0xFFFA      # $r14 <-- 0xFFFFFFFA

00 00 00 0d  //            break		    # (Fail flag 4)

- Using LW registers $r1 to $r7 get their respective values from the Data Memory
- The first use of the MULT instruction multiplies the value from $r1 and $r2 together and 
   stores the lower 32-bit value of the product into register $r8 using the MFLO instruction.
   The BNE instruction checks the product if it has the same value in register $r5 and branches 
   to the fail flag if they are not equal.
- The second use of the MULT instruction multiplies the value from $r3 and $r2 together and
   stores the lower 32-bit value of the product into register $r9 using the MFLO instruction, 
   and stores the upper 32-bit value of the product into register $r10 using the MFHI 
   instruction. The BNE instruction checks if $r9 is equal to $r6 and if $r10 is equal to $r7 and
   branches to their respective fail flags if not.
- Since register $r14 has 0x00000000, this proves that the all the instructions are working as

intended

Instruction Memory Module 9

Testing Instructions: XOR, AND, OR, NOR, SLTU

@0

3c 0f 10 01  // main:      lui  $15, 0x1001

35 ef 00 c0  //            ori  $15, 0x00C0    # $r15 <-- 0x100100C0 
                                               # (destptr)

20 01 ff 8a  //            addi $01, $00, -118 # $r01 <-- 0xFFFFFF8A

20 02 00 8a  //            addi $02  $00,  138 # $r02 <-- 0x0000008A

0c 10 00 22  //            jal  slt_tests

3c 0d 77 88  //            lui  $13, 0x7788

35 ad 77 88  //            ori  $13, 0x7788    # $r13 <-- 0x77887788  
                                               # (pattern1)

3c 0c 88 77  //            lui  $12, 0x8877

35 8c 88 77  //            ori  $12, 0x8877    # $r12 <-- 0x88778877  
                                               # (pattern2)

3c 0b ff ff  //            lui  $11, 0xFFFF

35 6b ff ff  //            ori  $11, 0xFFFF    # $r11 <-- 0xFFFFFFFF  
                                               # (pattern3)

01 ac 50 26  //            xor  $10, $13, $12  # $r10 <-- 0xFFFFFFFF

11 4b 00 02  //            beq  $10, $11, xor_pass

20 0e ff fb  //            addi $14, $00, -5   # r14 <-- 0xFFFF_FFFB

00 00 00 0d  //            break               # ^(fail flag 5)

01 ac 48 24  // xor_pass:  and  $09, $13, $12  # $r09 <-- 0x00000000

11 20 00 02  //            beq  $09, $00, and_pass

20 0e ff fa  //            addi $14, $00, -6   # r14 <-- 0xFFFF_FFFA

00 00 00 0d  //            break               # ^(fail flag 6)

01 e2 48 25  // and_pass:  or   $09, $15, $02  # $r09 <-- 0x100100CA

3c 08 10 01  //            lui  $08, 0x1001

35 08 00 ca  //            ori  $08, 0x00CA    # $r08 <-- 0x100100CA

11 09 00 02  //            beq  $08, $09, or_pass

20 0e ff f9  //            addi $14, $00, -7   # r14 <-- 0xFFFF_FFF9

00 00 00 0d  //            break               # ^(fail flag 7)

01 e2 48 27  // or_pass:   nor  $09, $15, $02  # $r09 <-- 0xEFFEFF35

3c 08 ef fe  //            lui  $08, 0xEFFE

35 08 ff 35  //            ori  $08, 0xFF35    # $r08 <-- 0xEFFEFF35

11 09 00 02  //            beq  $08, $09, nor_pass

20 0e ff f8  //            addi $14, $00, -8   # r14 <-- 0xFFFF_FFF8

00 00 00 0d  //            break               # ^(fail flag 8)

ad e8 00 10  // nor_pass:  sw   $08, 0x10($15) # M[D0] <-- 0xEFFEFF35

00 00 70 20  //            add  $14, $00, $00  # clear r14 "passed all"

00 00 00 0d  //            break               # should stop here,

//                                #  having completed all 
             //                                #  the tests

BREAK INSTRUCTION FETCHED 1820.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=1830.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=1840.0 ps, $r1  = 0xffffff8a  ||   $r17 = 0xxxxxxxxx

t=1850.0 ps, $r2  = 0x0000008a  ||   $r18 = 0xxxxxxxxx

t=1860.0 ps, $r3  = 0x00000000  ||   $r19 = 0xxxxxxxxx

t=1870.0 ps, $r4  = 0x000000c0  ||   $r20 = 0xxxxxxxxx

t=1880.0 ps, $r5  = 0x000000c4  ||   $r21 = 0xxxxxxxxx

t=1890.0 ps, $r6  = 0x000000c8  ||   $r22 = 0xxxxxxxxx

t=1900.0 ps, $r7  = 0x000000cc  ||   $r23 = 0xxxxxxxxx

t=1910.0 ps, $r8  = 0xeffeff35  ||   $r24 = 0xxxxxxxxx

t=1920.0 ps, $r9  = 0xeffeff35  ||   $r25 = 0xxxxxxxxx

t=1930.0 ps, $r10 = 0xffffffff  ||   $r26 = 0xxxxxxxxx

t=1940.0 ps, $r11 = 0xffffffff  ||   $r27 = 0xxxxxxxxx

t=1950.0 ps, $r12 = 0x88778877  ||   $r28 = 0xxxxxxxxx

t=1960.0 ps, $r13 = 0x77887788  ||   $r29 = 0x000003fc

t=1970.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t=1980.0 ps, $r15 = 0x100100c0  ||   $r31 = 0x00000014

Memory Locations 0xC0 to 0xFF

t=1990.0 ps, M[000000c0] = 0x000000c0

t=2000.0 ps, M[000000c4] = 0x000000c4

t=2010.0 ps, M[000000c8] = 0x000000c8

t=2020.0 ps, M[000000cc] = 0x000000cc

t=2030.0 ps, M[000000d0] = 0xeffeff35

t=2040.0 ps, M[000000d4] = 0xxxxxxxxx

t=2050.0 ps, M[000000d8] = 0xxxxxxxxx

t=2060.0 ps, M[000000dc] = 0xxxxxxxxx

t=2070.0 ps, M[000000e0] = 0xxxxxxxxx

t=2080.0 ps, M[000000e4] = 0xxxxxxxxx

t=2090.0 ps, M[000000e8] = 0xxxxxxxxx

t=2100.0 ps, M[000000ec] = 0xxxxxxxxx

t=2110.0 ps, M[000000f0] = 0xxxxxxxxx

t=2120.0 ps, M[000000f4] = 0xxxxxxxxx

t=2130.0 ps, M[000000f8] = 0xxxxxxxxx

t=2140.0 ps, M[000000fc] = 0xxxxxxxxx

t=2150.0 ps, PC = 0x00000088,   IR = 0x0000000d

Instruction Memory Module 9 (page 2)

Testing Instructions: XOR, AND, OR, NOR, SLTU

00 22 18 2a  // slt_tests: slt  $03, $01, $02  # for signed# r01 < r02

14 60 00 02  //            bne  $03, $00, slt1 #   thus, should branch

20 0e ff ff  //            addi $14, $00, -1   # r14 <-- FFFF_FFFF

00 00 00 0d  //            break		  # ^(fail flag 1)

20 04 00 c0  // slt1:      addi $04, $00, 0xC0 # pass flag1 M[C0]<-- C0

ad e4 00 00  //            sw   $04, 0x00($15)

00 41 18 2b  //            sltu $03, $02, $01  # for unsigned,r02 < r01

14 60 00 02  //            bne  $03, $00, slt2 #   thus, should branch

20 0e ff fe  //            addi $14, $00, -2   # r14 <-- FFFF_FFFE

00 00 00 0d  //            break               # ^(fail flag 2)

20 05 00 c4  // slt2:      addi $05, $00, 0xC4 # pass flag1 M[C4]<-- C4

ad e5 00 04  //            sw   $05, 0x04($15)

00 41 18 2a  //            slt  $03, $02, $01  # for signed# r02 !< r01

10 60 00 02  //            beq  $03, $00, slt3 #   thus, should branch

20 0e ff fd  //            addi $14, $00, -3   # r14 <-- FFFF_FFFD

00 00 00 0d  //            break		  # ^(fail flag 3)

20 06 00 c8  // slt3:      addi $06, $00, 0xC8 # pass flag3 M[C8]<-- C8

ad e6 00 08  //            sw   $06, 0x08($15)

00 22 18 2b  //            sltu $03, $01, $02  # for unsigned,r01!< r02

10 60 00 02  //            beq  $03, $00, slt4 #   thus, should branch

20 0e ff fc  //            addi $14, $00, -4   # r14 <-- FFFF_FFFC

00 00 00 0d  //            break		  # ^(fail flag 4)

20 07 00 cc  // slt4:      addi $07, $00, 0xCC # pass flag4 M[CC]<-- CC

ad e7 00 0c  //            sw   $07, 0x0C($15)

03 e0 00 08  //            jr   $31            # return from subroutine

- The SLT and SLTU instruction are checked in their own subroutine reached by the JAL 
   instruction and confirmed to be working correctly according to the pass flags in memory 
   locations 0xC0 to 0xCC

- The XOR, AND, OR and NOR instructions are verified with memory location 0xD0 having 
   the value 0xEFFEFF35 and  the register $r14 having the value 0x00000000

Instruction Memory Module 10

Testing Instructions: DIV

@0

3c 0f 10 01  // main: lui  $15, 0x1001

35 ef 00 00  //       ori  $15, 0x0000  # $r15 <-- 0x10010000(src)

8d e1 00 00  //       lw   $01, 00($15) # $r01 <--  264465

8d e2 00 04  //       lw   $02, 04($15) # $r02 <--    1000

8d e3 00 08  //       lw   $03, 08($15) # $r03 <-- -264465

8d e4 00 0c  //       lw   $04, 12($15) # $r04 <--   -1000

8d e5 00 10  //       lw   $05, 16($15) # $r05 <--     264

8d e6 00 14  //       lw   $06, 20($15) # $r06 <--     465

8d e7 00 18  //       lw   $07, 24($15) # $r07 <--    -264

8d e8 00 1c  //       lw   $08, 28($15) # $r08 <--    -465

00 22 00 1a  //       div  $01, $02

00 00 48 12  //       mflo $09       	  # $r09 <--     264   
00 00 50 10  //       mfhi $10	  # $r10 <--     465

15 25 00 16  //       bne  $09, $05, fail1Q

15 46 00 18  //       bne  $10, $06, fail1R

00 62 00 1a  //       div  $03, $02

00 00 48 12  //       mflo $09	  # $r09 <--    -264

00 00 50 10  //       mfhi $10	  # $r10 <--    -465

15 27 00 17  //       bne  $09, $07, fail2Q

15 48 00 19  //       bne  $10, $08, fail2R

00 24 00 1a  //       div  $01, $04

00 00 48 12  //       mflo $09	  # $r09 <--    -264

00 00 50 10  //       mfhi $10	  # $r10 <--     465

15 27 00 18  //       bne  $09, $07, fail3Q

15 46 00 1a  //       bne  $10, $06, fail3R

00 64 00 1a  //       div  $03, $04

00 00 48 12  //       mflo $09	  # $r09 <--     264

00 00 50 10  //       mfhi $10	  # $r10 <--    -465

15 25 00 19  //       bne  $09, $05, fail4Q

15 48 00 1b  //       bne  $10, $08, fail4R

3c 0b 00 00  // pass: lui  $11, 0x0000

35 6b 00 00  //       ori  $11, 0x0000   # $r11 <-- 0x00000000 (Pass)

00 0b 60 20  //       add  $12, $00, $11 # $r12 <-- Pass

00 0b 68 20  //       add  $13, $00, $11 # $r13 <-- Pass

00 0b 70 20  //       add  $14, $00, $11 # $r14 <-- Pass

00 00 00 0d  //       break

/* Rest of ‘fail flags’ on the next page */

BREAK INSTRUCTION FETCHED 1500.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=1510.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=1520.0 ps, $r1  = 0x00040911  ||   $r17 = 0xxxxxxxxx

t=1530.0 ps, $r2  = 0x000003e8  ||   $r18 = 0xxxxxxxxx

t=1540.0 ps, $r3  = 0xfffbf6ef  ||   $r19 = 0xxxxxxxxx

t=1550.0 ps, $r4  = 0xfffffc18  ||   $r20 = 0xxxxxxxxx

t=1560.0 ps, $r5  = 0x00000108  ||   $r21 = 0xxxxxxxxx

t=1570.0 ps, $r6  = 0x000001d1  ||   $r22 = 0xxxxxxxxx

t=1580.0 ps, $r7  = 0xfffffef8  ||   $r23 = 0xxxxxxxxx

t=1590.0 ps, $r8  = 0xfffffe2f  ||   $r24 = 0xxxxxxxxx

t=1600.0 ps, $r9  = 0x00000108  ||   $r25 = 0xxxxxxxxx

t=1610.0 ps, $r10 = 0xfffffe2f  ||   $r26 = 0xxxxxxxxx

t=1620.0 ps, $r11 = 0x00000000  ||   $r27 = 0xxxxxxxxx

t=1630.0 ps, $r12 = 0x00000000  ||   $r28 = 0xxxxxxxxx

t=1640.0 ps, $r13 = 0x00000000  ||   $r29 = 0x000003fc

t=1650.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t=1660.0 ps, $r15 = 0x10010000  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=1670.0 ps, M[000000c0] = 0xxxxxxxxx

t=1680.0 ps, M[000000c4] = 0xxxxxxxxx

t=1690.0 ps, M[000000c8] = 0xxxxxxxxx

t=1700.0 ps, M[000000cc] = 0xxxxxxxxx

t=1710.0 ps, M[000000d0] = 0xxxxxxxxx

t=1720.0 ps, M[000000d4] = 0xxxxxxxxx

t=1730.0 ps, M[000000d8] = 0xxxxxxxxx

t=1740.0 ps, M[000000dc] = 0xxxxxxxxx

t=1750.0 ps, M[000000e0] = 0xxxxxxxxx

t=1760.0 ps, M[000000e4] = 0xxxxxxxxx

t=1770.0 ps, M[000000e8] = 0xxxxxxxxx

t=1780.0 ps, M[000000ec] = 0xxxxxxxxx

t=1790.0 ps, M[000000f0] = 0xxxxxxxxx

t=1800.0 ps, M[000000f4] = 0xxxxxxxxx

t=1810.0 ps, M[000000f8] = 0xxxxxxxxx

t=1820.0 ps, M[000000fc] = 0xxxxxxxxx

t=1830.0 ps, PC = 0x00000090,   IR = 0x0000000d

Instruction Memory Module 10 (page 2)

Testing Instructions: DIV

3c 0e ff ff  // fail1Q: lui  $14, 0xFFFF

35 ce ff ff  //         ori  $14, 0xFFFF   	# $r14 <-- 0xFFFFFFFF

00 00 00 0d  //         break		      	# ^(Fail flag 1 Quot)

3c 0e ff ff  // fail1R: lui  $14, 0xFFFF

35 ce ff fe  //         ori  $14, 0xFFFE 	# $r14 <-- 0xFFFFFFFE

00 00 00 0d  //         break			# ^(Fail flag 1 Rem)

3c 0e ff ff  // fail2Q: lui  $14, 0xFFFF

35 ce ff fd  //         ori  $14, 0xFFFD 	# $r14 <-- 0xFFFFFFFD

00 00 00 0d  //         break			# ^(Fail flag 2 Quot)

3c 0e ff ff  // fail2R: lui  $14, 0xFFFF

35 ce ff fc  //         ori  $14, 0xFFFC 	# $r14 <-- 0xFFFFFFFC

00 00 00 0d  //         break			# ^(Fail flag 2 Rem)

3c 0e ff ff  // fail3Q: lui  $14, 0xFFFF

35 ce ff fb  //         ori  $14, 0xFFFB 	# $r14 <-- 0xFFFFFFFB

00 00 00 0d  //         break			# ^(Fail flag 3 Quot)

3c 0e ff ff  // fail3R: lui  $14, 0xFFFF

35 ce ff fa  //         ori  $14, 0xFFFA 	# $r14 <-- 0xFFFFFFFA

00 00 00 0d  //         break			# ^(Fail flag 3 Rem)

3c 0e ff ff  // fail4Q: lui  $14, 0xFFFF

35 ce ff f9  //         ori  $14, 0xFFF9 	# $r14 <-- 0xFFFFFFF9

00 00 00 0d  //         break			# ^(Fail flag 4 Quot)

3c 0e ff ff  // fail4R: lui  $14, 0xFFFF

35 ce ff f8  //         ori  $14, 0xFFF8 	# $r14 <-- 0xFFFFFFF8

00 00 00 0d  //         break			# (Fail flag 4 Rem)

- Using LW registers $r1 to $r8 get their respective values from the Data Memory
- The first use of the DIV instruction divides the value from $r1 to $r2 and 
   stores the lower 32-bit value of the quotient into register $r9 using the MFLO instruction and 
   stores the upper 32-bit value of the quotient into register $r10 using the MFHI instruction. 
   The BNE instruction checks if $r9 is equal to $r5 and if $r10 is equal to $r6 and
   branches to their respective fail flags if not.

- The DIV instruction divides $r3 and $r4 and stores the lower 32-bit on register $r9 and stores 
   the upper 32-bit value into register $r10. The BNE instruction then checks if $r9 is equal 
   to $r5 and if $r10 is equal to $r8 and branches to their respective fail flags if not.
- Since registers $r11 to $r14 all have the value 0x00000000, this proves that the all the 
   instructions are working as intended and avoided all the previous fail flags.

Instruction Memory Module 11

Testing Instructions: XORI, SUB, ANDI, SLTIU

@0

3c 0f 10 01  // main:   lui  $15, 0x1001

35 ef 00 c0  //         ori  $15, 0x00C0      # $r15 <--0x100100C0(src)

20 01 ff 8a  //         addi $01, $00, -118   # $r01 <-- 0xFFFFFF8A

20 02 00 8a  //         addi $02  $00,  138   # $r02 <-- 0x0000008A

0c 10 00 1a  //         jal  sltiu_tests

3c 0d ff ff  //         lui  $13, 0xFFFF	 # $r13 <-- 0xFFFF5555

35 ad 55 55  //         ori  $13, 0x5555      #          ^(pattern1)

3c 0c ff ff  //         lui  $12, 0xFFFF      # $r12 <-- 0xFFFFFAF5

35 8c fa f5  //         ori  $12, 0xFAF5      #          ^(pattern2)

3c 0b ff ff  //         lui  $11, 0xFFFF      # $r11 <-- 0xFFFFFFFF

35 6b ff ff  //         ori  $11, 0xFFFF      #          ^(pattern3)

3c 0a 00 00  //         lui  $10, 0x0000      # $r10 <-- 0x0000F0F0

35 4a f0 f0  //         ori  $10, 0xF0F0      #          ^(pattern4)

39 a9 aa aa  //         xori $09, $13, 0xAAAA # $r09 <-- 0xFFFFFFFF

01 2b 40 22  //         sub  $08, $09, $11    # $r08 <-- 0

11 00 00 02  //         beq  $08, $00, xor_p1 # should branch

20 0e ff f9  //         addi $14, $00, -7     # $r14 <-- FFFF_FFF9

00 00 00 0d  //         break                 #          ^(fail flag7)

31 87 f5 fa  // xor_p1: andi $07, $12, 0xF5FA # $r07 <-- 0x0000F0F0

00 ea 40 22  //         sub  $08, $07, $10

11 00 00 02  //         beq  $08, $00, xor_p2 # should branch

20 0e ff f8  //         addi $14, $00, -8     # $r14 <-- FFFF_FFF8

00 00 00 0d  //         break                 #          ^(fail flag8)

ad e1 00 18  // xor_p2: sw   $01, 0x18($15)   # M[D8] <-- FFFFFF8A

00 00 00 0d  //         break                 # should stop here,

#  having completed all 
 	 #  the tests

// sltiu_tests:

2c 23 ff 8b  //   sltiu  $03, $01, -117       # if r1 < se(0xFF8B) uns

14 60 00 02  //   bne    $03, $00, slt1_p1    #  should branch

20 0e ff ff  //   addi   $14, $00, -1         # $r14 <-- FFFF_FFFF

00 00 00 0d  //   break		      	 #          ^(fail flag1)

20 04 00 c0  // slt1_p1: addi   $04, $00,0xC0 # pass flag1 M[C0]<--C0

ad e4 00 00  //          sw     $04, 0x00($15)

BREAK INSTRUCTION FETCHED 1980.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=1990.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=2000.0 ps, $r1  = 0xffffff8a  ||   $r17 = 0xxxxxxxxx

t=2010.0 ps, $r2  = 0x0000008a  ||   $r18 = 0xxxxxxxxx

t=2020.0 ps, $r3  = 0x00000000  ||   $r19 = 0xxxxxxxxx

t=2030.0 ps, $r4  = 0x000000c0  ||   $r20 = 0xxxxxxxxx

t=2040.0 ps, $r5  = 0x000000c4  ||   $r21 = 0xxxxxxxxx

t=2050.0 ps, $r6  = 0x000000d4  ||   $r22 = 0xxxxxxxxx

t=2060.0 ps, $r7  = 0x0000f0f0  ||   $r23 = 0xxxxxxxxx

t=2070.0 ps, $r8  = 0x00000000  ||   $r24 = 0xxxxxxxxx

t=2080.0 ps, $r9  = 0xffffffff  ||   $r25 = 0xxxxxxxxx

t=2090.0 ps, $r10 = 0x0000f0f0  ||   $r26 = 0xxxxxxxxx

t=2100.0 ps, $r11 = 0xffffffff  ||   $r27 = 0xxxxxxxxx

t=2110.0 ps, $r12 = 0xfffffaf5  ||   $r28 = 0xxxxxxxxx

t=2120.0 ps, $r13 = 0xffff5555  ||   $r29 = 0x000003fc

t=2130.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t=2140.0 ps, $r15 = 0x100100c0  ||   $r31 = 0x00000014

Memory Locations 0xC0 to 0xFF

t=2150.0 ps, M[000000c0] = 0x000000c0

t=2160.0 ps, M[000000c4] = 0x000000c4

t=2170.0 ps, M[000000c8] = 0x000000c8

t=2180.0 ps, M[000000cc] = 0x000000cc

t=2190.0 ps, M[000000d0] = 0x000000d0

t=2200.0 ps, M[000000d4] = 0x000000d4

t=2210.0 ps, M[000000d8] = 0xffffff8a

t=2220.0 ps, M[000000dc] = 0xxxxxxxxx

t=2230.0 ps, M[000000e0] = 0xxxxxxxxx

t=2240.0 ps, M[000000e4] = 0xxxxxxxxx

t=2250.0 ps, M[000000e8] = 0xxxxxxxxx

t=2260.0 ps, M[000000ec] = 0xxxxxxxxx

t=2270.0 ps, M[000000f0] = 0xxxxxxxxx

t=2280.0 ps, M[000000f4] = 0xxxxxxxxx

t=2290.0 ps, M[000000f8] = 0xxxxxxxxx

t=2300.0 ps, M[000000fc] = 0xxxxxxxxx

t=2310.0 ps, PC = 0x00000064,   IR = 0x0000000d

Instruction Memory Module 11

Testing Instructions: XORI, SUB, ANDI, SLTIU

2c 23 ff 89  //   sltiu  $03, $01, -119 	 # if r1 !<se(0xFF89) uns

10 60 00 02  //   beq    $03, $00, slt_p2   	 #  should branch

20 0e ff fe  //   addi   $14, $00, -2       	 # $r14 <-- FFFF_FFFE

00 00 00 0d  //   break		 	 #          ^(fail flag2)

20 05 00 c4  // slt_p2: addi   $05, $00, 0xC4 # pass flag2 M[C4]<-- C4

ad e5 00 04  //         sw     $05, 0x04($15)

2c 23 ff 8a  //   sltiu  $03, $01, -118     	# if r1 !< se(0xFF8A) uns

10 60 00 02  //   beq    $03, $00, slt_p3   	#   should branch

20 0e ff fd  //   addi   $14, $00, -3       	# r14 <-- FFFF_FFFD

00 00 00 0d  //   break		  	#         ^(fail flag3)

20 06 00 c8  // slt_p3: addi   $06, $00, 0xC8# pass flag3 M[C8] <-- C8

ad e6 00 08  //         sw     $06, 0x08($15)

2c 43 00 8b  //   sltiu  $03, $02, 0x008B   	# if r2 < se(0x008B) uns

14 60 00 02  //   bne    $03, $00, slt1_p4  	#   should branch

20 0e ff fc  //   addi   $14, $00, -4       	# r14 <-- FFFF_FFFC

00 00 00 0d  //   break			# 	  ^(fail flag4)

20 07 00 cc  // slt1_p4: addi  $07, $00, 0xCC# pass flag4 M[CC] <-- CC

ad e7 00 0c  //          sw    $07, 0x0C($15)

2c 43 00 89  //   sltiu  $03, $02, 0x0089   	# if r2 !< se(0x0089) uns

10 60 00 02  //   beq    $03, $00, slt_p5   	#   should branch

20 0e ff fb  //   addi   $14, $00, -5       	# r14 <-- FFFF_FFFB

00 00 00 0d  //   break			# 	  ^(fail flag5)

20 08 00 d0  // slt_p5: addi   $08, $00, 0xD0# pass flag5 M[D0] <-- D0

ad e8 00 10  //         sw     $08 0x10($15)

2c 43 00 8a  //   sltiu  $03, $02, 0x008A   	# if r2 !< se(0x008A) uns

10 60 00 02  //   beq    $03, $00, slt_p6   	#   should branch

20 0e ff fa  //   addi   $14, $00, -6       	# r14 <-- FFFF_FFFA

00 00 00 0d  //   break			# 	  ^(fail flag6)

20 06 00 d4  // slt_p6: addi  $06, $00, 0xD4	# pass flag6 M[D4] <-- D4

ad e6 00 14  //         sw    $06, 0x14($15)

20 0e 00 00  //         addi   $14, $00, 0  	# set $r14 to 0000_0000

03 e0 00 08  //         jr   $31           	# return from subroutine

- The SLTIU instruction is checked in its own subroutine reached by the JAL 
   instruction and confirmed to be working correctly according to the pass flags in memory 
   locations 0xC0 to 0xD4

- The XORI, SUB and ANDI instructions are verified with memory location 0xD8 having 
   the value 0xFFFFFF8A and the register $r14 having the value 0x00000000

Instruction Memory Module 12

Testing Instructions: BLEZ, BGTZ

@0

3c 0f 10 01  // main:      lui  $15, 0x1001

35 ef 00 c0  //            ori  $15, 0x00C0    # $r15 <-- 0x100100C0  (dest pointer)

20 01 ff 8a  //            addi $01, $00, -118 # $r01 <-- 0xFFFFFF8A

20 02 00 8a  //            addi $02  $00,  138 # $r02 <-- 0x0000008A

0c 10 00 08  //            jal  blt_tests

ad e1 00 18  //            sw   $01, 0x18($15) # M[D8] <-- 0xFFFFFF8A

ad e2 00 1c  //            sw   $02, 0x1C($15) # M[DC] <-- 0x0000008A

00 00 00 0d  //            break

18 20 00 02  // blt_tests: blez $01, blez_p1   # this should branch

20 0e ff ff  //            addi $14, $00, -1   # r14 <-- FFFF_FFFF

00 00 00 0d  //            break		            ^(fail flag)

20 03 00 c0  // blez_p1:   addi $03, $00, 0xC0 # pass flag1 M[C0]<-- C0

ad e3 00 00  //            sw   $03, 0x00($15)

18 40 00 03  //            blez $02, blez_f2   # this should not branch

20 04 00 c4  //            addi $04, $00, 0xC4 # pass flag2 M[C4]<-- C4

ad e4 00 04  //            sw   $04, 0x04($15)

08 10 00 13  //            j    blez_p2

20 0e ff fe  // blez_f2:   addi $14, $00, -2   # r14 <-- FFFF_FFFE

00 00 00 0d  //            break			    ^(fail flag2)

18 00 00 02  // blez_p2:   blez $0, blez_p3    # this should branch

20 0e ff fd  //            addi $14, $00, -3   #failflag3 r14<-FFFFFFFD

00 00 00 0d  //            break

20 05 00 c8  // blez_p3:   addi $05, $00, 0xC8 # pass flag3 M[C8]<-- C8

ad e5 00 08  //            sw   $05, 0x08($15)

1c 40 00 02  //            bgtz $02, bgtz_p1   # this should pass

20 0e ff fc  //            addi $14, $00, -4   # r14 <-- FFFF_FFFC

00 00 00 0d  //            break			    ^(fail flag3)

20 06 00 cc  // bgtz_p1:   addi $06, $00, 0xCC # pass flag4 M[CC]<-- CC

ad e6 00 0c  //            sw   $06, 0x0C($15)

1c 20 00 03  //            bgtz $01, bgtz_f2   # this should not branch

20 07 00 d0  //            addi $07, $00, 0xD0 # pass flag5 M[D0]<-- D0

ad e7 00 10  //            sw   $07, 0x10($15)

08 10 00 23  //            j    bgtz_p2

20 0e ff fb  // bgtz_f2:   addi $14, $00, -5   # r14 <-- FFFF_FFFB

00 00 00 0d  //            break			    ^(fail flag5)

1c 20 00 03  // bgtz_p2:   bgtz $01, bgtz_f3   # this should not branch

20 08 00 d4  //            addi $08, $00, 0xD4 # pass flag6 M[D4]<-- D4

ad e8 00 14  //            sw   $08, 0x14($15)

08 10 00 29  //            j    bgtz_p3

20 0e ff fa  // bgtz_f3:   addi $14, $00, -6   # r14 <-- FFFF_FFFA

00 00 00 0d  //            break			    ^(fail flag6)

20 0e 00 00  // bgtz_p3:   addi $14, $00, 0    # set $r14 to 0000_0000

03 e0 00 08  //            jr   $31            # return from subroutine

BREAK INSTRUCTION FETCHED 1310.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=1320.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=1330.0 ps, $r1  = 0xffffff8a  ||   $r17 = 0xxxxxxxxx

t=1340.0 ps, $r2  = 0x0000008a  ||   $r18 = 0xxxxxxxxx

t=1350.0 ps, $r3  = 0x000000c0  ||   $r19 = 0xxxxxxxxx

t=1360.0 ps, $r4  = 0x000000c4  ||   $r20 = 0xxxxxxxxx

t=1370.0 ps, $r5  = 0x000000c8  ||   $r21 = 0xxxxxxxxx

t=1380.0 ps, $r6  = 0x000000cc  ||   $r22 = 0xxxxxxxxx

t=1390.0 ps, $r7  = 0x000000d0  ||   $r23 = 0xxxxxxxxx

t=1400.0 ps, $r8  = 0x000000d4  ||   $r24 = 0xxxxxxxxx

t=1410.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=1420.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=1430.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=1440.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=1450.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=1460.0 ps, $r14 = 0x00000000  ||   $r30 = 0xxxxxxxxx

t=1470.0 ps, $r15 = 0x100100c0  ||   $r31 = 0x00000014

Memory Locations 0xC0 to 0xFF

t=1480.0 ps, M[000000c0] = 0x000000c0

t=1490.0 ps, M[000000c4] = 0x000000c4

t=1500.0 ps, M[000000c8] = 0x000000c8

t=1510.0 ps, M[000000cc] = 0x000000cc

t=1520.0 ps, M[000000d0] = 0x000000d0

t=1530.0 ps, M[000000d4] = 0x000000d4

t=1540.0 ps, M[000000d8] = 0xffffff8a

t=1550.0 ps, M[000000dc] = 0x0000008a

t=1560.0 ps, M[000000e0] = 0xxxxxxxxx

t=1570.0 ps, M[000000e4] = 0xxxxxxxxx

t=1580.0 ps, M[000000e8] = 0xxxxxxxxx

t=1590.0 ps, M[000000ec] = 0xxxxxxxxx

t=1600.0 ps, M[000000f0] = 0xxxxxxxxx

t=1610.0 ps, M[000000f4] = 0xxxxxxxxx

t=1620.0 ps, M[000000f8] = 0xxxxxxxxx

t=1630.0 ps, M[000000fc] = 0xxxxxxxxx

t=1640.0 ps, PC = 0x00000020,   IR = 0x0000000d

- The BLEZ and BGTZ instructions are tested after the JAL instruction and are confirmed to 
  be working correctly according to the pass flags in memory locations 0xC0 to 0xD4. After 
  jumping back to the original PC before the JAL instruction, 0xD8 and 0xDC gets the values 
  0xFFFFFF8A and 0x0000008A, respectively, and further proves the functionality of the 
  instructions.

Instruction Memory Module 13

Testing Instructions: SETIE, INPUT, OUTPUT, SRA(Barrel Shift)

@0

00 00 00 1f  // main:      setie

3c 01 12 34  //            lui  $01, 0x1234

34 21 56 78  //            ori  $01, 0x5678    # LI  R01,  0x12345678

3c 02 87 65  //            lui  $02, 0x8765

34 42 43 21  //            ori  $02, 0x4321    # LI  R02,  0x87654321

3c 03 ab cd  //            lui  $03, 0xABCD

34 63 ef 01  //            ori  $03, 0xEF01    # LI  R03,  0xABCDEF01

3c 04 01 fe  //            lui  $04, 0x01FE

34 84 dc ba  //            ori  $04, 0xDCBA    # LI  R04,  0x01FEDCBA

3c 05 5a 5a  //            lui  $05, 0x5A5A

34 a5 5a 5a  //            ori  $05, 0x5A5A    # LI  R05,  0x5A5A5A5A

3c 06 ff ff  //            lui  $06, 0xFFFF

34 c6 ff ff  //            ori  $06, 0xFFFF    # LI  R06,  0xFFFFFFFF

3c 07 ff ff  //            lui  $07, 0xFFFF

34 e7 ff 00  //            ori  $07, 0xFF00    # LI  R07,  0xFFFFFF00

00 c7 40 20  //            add  $08, $06, $07

00 c8 48 20  //            add  $09, $06, $08

00 c9 50 20  //            add  $10, $06, $09

00 ca 58 20  //            add  $11, $06, $10

00 cb 60 20  //            add  $12, $06, $11

00 cc 68 20  //            add  $13, $06, $12

00 cd 70 20  //            add  $14, $06, $13

00 ce 78 20  //            add  $15, $06, $14

3c 07 10 01  //            lui  $07, 0x1001

34 e7 03 f0  //            ori  $07, 0x03F0    # LI   R07, 0x100103F0

ac ef 00 00  //            sw   $15, 0($07)    # ST  [R07], R15

00 00 00 0d  //            break

@200

//*******************************************

// In this ISR, we will implement writing

// some patterns to the IO space, and then

// reading them back.

// Note: this "ISR" expects the "return addr”

//  to have been saved in $ra (not the stack)

//*****************************************

3c 10 10 01  // isr:     lui  $16, 0x1001    #load destination IO addr

36 10 00 c0  //          ori  $16, 0x00C0    #  0x100100C0 into r16

3c 11 80 00  //          lui  $17, 0x8000    #initialize the pattern of

36 31 ff ff  //          ori  $17, 0xFFFF    #  0x8000FFFF into r17

20 12 00 10  //          addi $18, $0, 0x10  #loop counter set to 16

BREAK INSTRUCTION FETCHED 5030.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=5040.0 ps, $r0  = 0x00000000  ||   $r16 = 0x100100c0

t=5050.0 ps, $r1  = 0x12345678  ||   $r17 = 0xffffffff

t=5060.0 ps, $r2  = 0x87654321  ||   $r18 = 0x00000000

t=5070.0 ps, $r3  = 0xabcdef01  ||   $r19 = 0x8000ffff

t=5080.0 ps, $r4  = 0x01fedcba  ||   $r20 = 0xe0003fff

t=5090.0 ps, $r5  = 0x5a5a5a5a  ||   $r21 = 0xf8000fff

t=5100.0 ps, $r6  = 0xffffffff  ||   $r22 = 0xfe0003ff

t=5110.0 ps, $r7  = 0x100103f0  ||   $r23 = 0xff8000ff

t=5120.0 ps, $r8  = 0xfffffeff  ||   $r24 = 0xffe0003f

t=5130.0 ps, $r9  = 0xfffffefe  ||   $r25 = 0xxxxxxxxx

t=5140.0 ps, $r10 = 0xfffffefd  ||   $r26 = 0xxxxxxxxx

t=5150.0 ps, $r11 = 0xfffffefc  ||   $r27 = 0xxxxxxxxx

t=5160.0 ps, $r12 = 0xfffffefb  ||   $r28 = 0xxxxxxxxx

t=5170.0 ps, $r13 = 0xfffffefa  ||   $r29 = 0x000003fc

t=5180.0 ps, $r14 = 0xfffffef9  ||   $r30 = 0xxxxxxxxx

t=5190.0 ps, $r15 = 0xfffffef8  ||   $r31 = 0x00000020

Memory Location 0x3F0

t=5200.0 ps, M[000003f0] = 0xfffffef8

IO Memory Locations 0xC0 to 0xFF

t=5210.0 ps, IO[000000c0] = 0x8000ffff

t=5220.0 ps, IO[000000c4] = 0xe0003fff

t=5230.0 ps, IO[000000c8] = 0xf8000fff

t=5240.0 ps, IO[000000cc] = 0xfe0003ff

t=5250.0 ps, IO[000000d0] = 0xff8000ff

t=5260.0 ps, IO[000000d4] = 0xffe0003f

t=5270.0 ps, IO[000000d8] = 0xfff8000f

t=5280.0 ps, IO[000000dc] = 0xfffe0003

t=5290.0 ps, IO[000000e0] = 0xffff8000

t=5300.0 ps, IO[000000e4] = 0xffffe000

t=5310.0 ps, IO[000000e8] = 0xfffff800

t=5320.0 ps, IO[000000ec] = 0xfffffe00

t=5330.0 ps, IO[000000f0] = 0xffffff80

t=5340.0 ps, IO[000000f4] = 0xffffffe0

t=5350.0 ps, IO[000000f8] = 0xfffffff8

t=5360.0 ps, IO[000000fc] = 0xfffffffe

t=5370.0 ps, PC = 0x0000006c,   IR = 0x0000000d

Instruction Memory Module 13 (page 2)

Testing Instructions: SETIE, INPUT, OUTPUT, SRA(Barrel Shift)

76 11 00 00  // out_IO:  output $17, 0($16)  # output  [R16], R17

00 11 88 83  //          sra    $17, $17, 2  #  shift twice

22 10 00 04  //          addi   $16, $16, 4  # inc memory pointer by 4

22 52 ff ff  //          addi   $18, $18, -1     # dec the loop counter

16 40 ff fb  //          bne    $18, $00, out_IO # and jmp to top if 
             //                                     not done

3c 10 10 01  //          lui    $16, 0x1001  #load source IO addr

36 10 00 c0  //          ori    $16, 0x00C0  #  0x100100C0 into r16

72 13 00 00  //          input  $19,  0($16) #  and input from 6

72 14 00 04  //          input  $20,  4($16) #    the IO locations,

72 15 00 08  //          input  $21,  8($16) #    starting from 0xC0

72 16 00 0c  //          input  $22, 12($16)

72 17 00 10  //          input  $23, 16($16)

72 18 00 14  //          input  $24, 20($16)

03 e0 00 08  //          jr     $31          # return from interrupt 
             //                              # (v1, using $ra)

- The SETIE instruction is called as the very first instruction in the Instruction Memory.
- After 300ns, an interrupt happens and the processor jumps to the Interrupt Service Routine.
  In the ISR, the OUTPUT instruction is called 16 times and writes to IO memory locations
  0xC0 to 0xFF while doing the SRA(2) instruction in every iteration of the loop.

- After the loop, the INPUT instruction writes to registers $r19 to $r24 from IO memory 
  locations 0xC0 to 0xD4

- The ISR then calls the JR instruction to jump back to the main routine and at the end, the 
  value of register $r15, 0xFFFFFEF8 is stored into memory location 0x3F0.

Instruction Memory Module 14

Testing Instructions: RETI

@0

00 00 00 1f  // main: setie

3c 01 12 34  //       lui  $01, 0x1234

34 21 56 78  //       ori  $01, 0x5678      # LI  R01,  0x12345678

3c 02 87 65  //       lui  $02, 0x8765

34 42 43 21  //       ori  $02, 0x4321      # LI  R02,  0x87654321

3c 03 ab cd  //       lui  $03, 0xABCD

34 63 ef 01  //       ori  $03, 0xEF01      # LI  R03,  0xABCDEF01

3c 04 01 fe  //       lui  $04, 0x01FE

34 84 dc ba  //       ori  $04, 0xDCBA      # LI  R04,  0x01FEDCBA

3c 05 5a 5a  //       lui  $05, 0x5A5A

34 a5 5a 5a  //       ori  $05, 0x5A5A      # LI  R05,  0x5A5A5A5A

3c 06 ff ff  //       lui  $06, 0xFFFF

34 c6 ff ff  //       ori  $06, 0xFFFF      # LI  R06,  0xFFFFFFFF

3c 07 ff ff  //       lui  $07, 0xFFFF

34 e7 ff 00  //       ori  $07, 0xFF00      # LI  R07,  0xFFFFFF00

00 c7 40 20  //       add  $08, $06, $07

00 c8 48 20  //       add  $09, $06, $08

00 c9 50 20  //       add  $10, $06, $09

00 ca 58 20  //       add  $11, $06, $10

00 cb 60 20  //       add  $12, $06, $11

00 cc 68 20  //       add  $13, $06, $12

00 cd 70 20  //       add  $14, $06, $13

00 ce 78 20  //       add  $15, $06, $14

3c 07 10 01  //       lui  $07, 0x1001

34 e7 03 f0  //       ori  $07, 0x03F0      # LI   R07,  0x100103F0

ac ef 00 00  //       sw   $15, 0($07)      # ST  [R07], R15

00 00 00 0d  //       break

@200                         
		//************************************************

// In this ISR, we will implement writing

// some patterns to the IO space, and then

// reading them back.

// Note: this "ISR" expects the "return address"

//       to have been saved on the stack (not $ra)

//************************************************

3c 10 10 01  // isr:   lui  $16, 0x1001      #load dest IO address

36 10 00 c0  //        ori  $16, 0x00C0      #  0x100100C0 into r16

3c 11 80 00  //        lui  $17, 0x8000      #initialize the pattern of

36 31 ff ff  //        ori  $17, 0xFFFF      #  0x8000FFFF into r17

20 12 00 10  //        addi $18, $0, 0x10    #loop counter set to 16

76 11 00 00  // out_IO:output $17, 0($16)    #output  [R16], R17

00 11 88 83  //        sra    $17, $17, 2    #shift right by 2

22 10 00 04  //        addi   $16, $16, 4    # inc memory pointer by 4

22 52 ff ff  //        addi   $18, $18, -1   # dec the loop counter

16 40 ff fb  //        bne    $18, $00,out_IO#  jmp to top if not done

BREAK INSTRUCTION FETCHED 5100.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=5110.0 ps, $r0  = 0x00000000  ||   $r16 = 0x100100c0

t=5120.0 ps, $r1  = 0x12345678  ||   $r17 = 0xffffffff

t=5130.0 ps, $r2  = 0x87654321  ||   $r18 = 0x00000000

t=5140.0 ps, $r3  = 0xabcdef01  ||   $r19 = 0x8000ffff

t=5150.0 ps, $r4  = 0x01fedcba  ||   $r20 = 0xe0003fff

t=5160.0 ps, $r5  = 0x5a5a5a5a  ||   $r21 = 0xf8000fff

t=5170.0 ps, $r6  = 0xffffffff  ||   $r22 = 0xfe0003ff

t=5180.0 ps, $r7  = 0x100103f0  ||   $r23 = 0xff8000ff

t=5190.0 ps, $r8  = 0xfffffeff  ||   $r24 = 0xffe0003f

t=5200.0 ps, $r9  = 0xfffffefe  ||   $r25 = 0xxxxxxxxx

t=5210.0 ps, $r10 = 0xfffffefd  ||   $r26 = 0xxxxxxxxx

t=5220.0 ps, $r11 = 0xfffffefc  ||   $r27 = 0xxxxxxxxx

t=5230.0 ps, $r12 = 0xfffffefb  ||   $r28 = 0xxxxxxxxx

t=5240.0 ps, $r13 = 0xfffffefa  ||   $r29 = 0x000003fc

t=5250.0 ps, $r14 = 0xfffffef9  ||   $r30 = 0xxxxxxxxx

t=5260.0 ps, $r15 = 0xfffffef8  ||   $r31 = 0x00000000

Memory Location 0x3F0 to 0x404

t=5270.0 ps, M[000003f0] = 0xfffffef8	//SW $15, 0($07)

t=5280.0 ps, M[000003f4] = 0xxxxxxxxx

t=5290.0 ps, M[000003f8] = 0x00000000

t=5300.0 ps, M[000003fc] = 0x00000200	//ISR address

t=5310.0 ps, M[00000400] = 0x00000020	//PC

t=5320.0 ps, M[00000404] = 0x00000011	//Status Flags

IO Memory Locations 0xC0 to 0xFF

t=5330.0 ps, IO[000000c0] = 0x8000ffff

t=5340.0 ps, IO[000000c4] = 0xe0003fff

t=5350.0 ps, IO[000000c8] = 0xf8000fff

t=5360.0 ps, IO[000000cc] = 0xfe0003ff

t=5370.0 ps, IO[000000d0] = 0xff8000ff

t=5380.0 ps, IO[000000d4] = 0xffe0003f

t=5390.0 ps, IO[000000d8] = 0xfff8000f

t=5400.0 ps, IO[000000dc] = 0xfffe0003

t=5410.0 ps, IO[000000e0] = 0xffff8000

t=5420.0 ps, IO[000000e4] = 0xffffe000

t=5430.0 ps, IO[000000e8] = 0xfffff800

t=5440.0 ps, IO[000000ec] = 0xfffffe00

t=5450.0 ps, IO[000000f0] = 0xffffff80

t=5460.0 ps, IO[000000f4] = 0xffffffe0

t=5470.0 ps, IO[000000f8] = 0xfffffff8

t=5480.0 ps, IO[000000fc] = 0xfffffffe

t=5490.0 ps, PC = 0x0000006c,   IR = 0x0000000d

Instruction Memory Module 14 (page 2)

Testing Instructions: RETI

3c 10 10 01  //        lui    $16, 0x1001    #load source IO address

36 10 00 c0  //        ori    $16, 0x00C0    #  0x100100C0 into r16

72 13 00 00  //        input  $19,  0($16)   #  and input from 6

72 14 00 04  //        input  $20,  4($16)   #    the IO locations,

72 15 00 08  //        input  $21,  8($16)   #    starting from 0xC0

72 16 00 0c  //        input  $22, 12($16)

72 17 00 10  //        input  $23, 16($16)

72 18 00 14  //        input  $24, 20($16)

7B A0 00 00  //        reti   # return from interrupt (v2, using M[sp]
                              # as saved PC)

# Note opcode=0x1E and $rs=0x1D, i.e. $sp

- The SETIE instruction is called as the very first instruction in the Instruction Memory.
- After 300ns, an interrupt happens and the processor jumps to the Interrupt Service Routine.
  The INTR instruction from the Control Unit then writes the PC to memory location 
  0x400and current status flags to memory location 0x404 before jumping to the ISR

- In the ISR, the OUTPUT instruction is called 16 times and writes to IO memory locations
  0xC0 to 0xFF while doing the SRA(2) instruction in every iteration of the loop.

- After the loop, the INPUT instruction writes to registers $r19 to $r24 from IO memory 
  locations 0xC0 to 0xD4

- The ISR then calls the RETI instruction to get the old PC from memory location 0x400 and 
  jump back to it (0x00000020) and resets the stack pointer to 0x3FC on $sp

Instruction Memory Enhanced Instruction Module 1

Testing Instructions: ROTLEFT

@0

3c 01 ff fF  // main: lui  $01, 0xFFFF

34 21 00 00  //       ori  $01, 0x0000     # LI   R01,  0xFFFF0000

20 02 00 10  //       addi $02, $00, 0x10  # LI   R02,  0x10

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0     # LI   R15,  0x100100C0

00 01 08 ac  // top:  rotleft $01, $01, 2  # rotate left 2 bits

ad e1 00 00  //       sw   $01, 0($15)     # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4     # inc memory pointer by 4

20 42 ff ff  //       addi $02, $02, -1    # decrement the loop counter

14 40 ff fb  //       bne  $02, $00, top   # and jmp to top if not done

08 10 00 0c  //       j    exit            # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C     # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

$r2 is decremented by 1, sixteen times until it reaches 0 and JUMPS to the exit.

$r15 from 0x100100C0 incremented by 4, sixteen times and resulted into 0x10010100
- Register $r2 having the value 0x00000000 proves that the ADDI function works correctly.

- Since register $r14 has 0x5A5A3C3C, it shows that all the instructions are working as 
  intended

- Values inside the Memory Locations 0xC0 to 0xFF shows the original value of $r1 of 
  0xFFFF0000 rotating 2 bits to the left using the instruction ROTLEFT each loop and after 
  16 loops, goes back to its original value.

BREAK INSTRUCTION FETCHED 3650.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=3660.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=3670.0 ps, $r1  = 0xffff0000  ||   $r17 = 0xxxxxxxxx

t=3680.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=3690.0 ps, $r3  = 0xxxxxxxxx  ||   $r19 = 0xxxxxxxxx

t=3700.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=3710.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=3720.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=3730.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=3740.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=3750.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=3760.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=3770.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=3780.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=3790.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=3800.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=3810.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=3820.0 ps, M[000000c0] = 0xfffc0003

t=3830.0 ps, M[000000c4] = 0xfff0000f

t=3840.0 ps, M[000000c8] = 0xffc0003f

t=3850.0 ps, M[000000cc] = 0xff0000ff

t=3860.0 ps, M[000000d0] = 0xfc0003ff

t=3870.0 ps, M[000000d4] = 0xf0000fff

t=3880.0 ps, M[000000d8] = 0xc0003fff

t=3890.0 ps, M[000000dc] = 0x0000ffff

t=3900.0 ps, M[000000e0] = 0x0003fffc

t=3910.0 ps, M[000000e4] = 0x000ffff0

t=3920.0 ps, M[000000e8] = 0x003fffc0

t=3930.0 ps, M[000000ec] = 0x00ffff00

t=3940.0 ps, M[000000f0] = 0x03fffc00

t=3950.0 ps, M[000000f4] = 0x0ffff000

t=3960.0 ps, M[000000f8] = 0x3fffc000

t=3970.0 ps, M[000000fc] = 0xffff0000

t=3980.0 ps, PC = 0x0040003c,   IR = 0x0000000d

Instruction Memory Enhanced Instruction Module 2

Testing Instructions: ROTRIGHT

@0

3c 01 ff fF  // main: lui  $01, 0xFFFF

34 21 00 00  //       ori  $01, 0x0000     # LI   R01,  0xFFFF0000

20 02 00 10  //       addi $02, $00, 0x10  # LI   R02,  0x10

3c 0f 10 01  //       lui  $15, 0x1001

35 ef 00 c0  //       ori  $15, 0x00C0     # LI   R15,  0x100100C0

00 01 08 ad  // top: rotright $01, $01, 2  # rotate right 2 bits

ad e1 00 00  //       sw   $01, 0($15)     # ST  [R15], R01

21 ef 00 04  //       addi $15, $15, 4     # inc memory pointer by 4

20 42 ff ff  //       addi $02, $02, -1    # decrement the loop counter

14 40 ff fb  //       bne  $02, $00, top   # and jmp to top if not done

08 10 00 0c  //       j    exit            # jump around a halt instr

00 00 00 0d  //       break

3c 0e 5a 5a  // exit: lui  $14, 0x5A5A

35 ce 3c 3c  //       ori  $14, 0x3C3C     # LI   R14,  0x5A5A3C3C

00 00 00 0d  //       break

$r2 is decremented by 1, sixteen times until it reaches 0 and JUMPS to the exit.

$r15 from 0x100100C0 incremented by 4, sixteen times and resulted into 0x10010100
- Register $r2 having the value 0x00000000 proves that the ADDI function works correctly.

- Since register $r14 has 0x5A5A3C3C, it shows that all the instructions are working as 
  intended

- Values inside the Memory Locations 0xC0 to 0xFF shows the original value of $r1 of 
  0xFFFF0000 rotating 2 bits to the right using the instruction ROTRIGHT each loop and 
  after 16 loops, goes back to its original value.

BREAK INSTRUCTION FETCHED 3650.0 ps

R E G I S T E R ' S   A F T E R   B R E A K

t=3660.0 ps, $r0  = 0x00000000  ||   $r16 = 0xxxxxxxxx

t=3670.0 ps, $r1  = 0xffff0000  ||   $r17 = 0xxxxxxxxx

t=3680.0 ps, $r2  = 0x00000000  ||   $r18 = 0xxxxxxxxx

t=3690.0 ps, $r3  = 0xxxxxxxxx  ||   $r19 = 0xxxxxxxxx

t=3700.0 ps, $r4  = 0xxxxxxxxx  ||   $r20 = 0xxxxxxxxx

t=3710.0 ps, $r5  = 0xxxxxxxxx  ||   $r21 = 0xxxxxxxxx

t=3720.0 ps, $r6  = 0xxxxxxxxx  ||   $r22 = 0xxxxxxxxx

t=3730.0 ps, $r7  = 0xxxxxxxxx  ||   $r23 = 0xxxxxxxxx

t=3740.0 ps, $r8  = 0xxxxxxxxx  ||   $r24 = 0xxxxxxxxx

t=3750.0 ps, $r9  = 0xxxxxxxxx  ||   $r25 = 0xxxxxxxxx

t=3760.0 ps, $r10 = 0xxxxxxxxx  ||   $r26 = 0xxxxxxxxx

t=3770.0 ps, $r11 = 0xxxxxxxxx  ||   $r27 = 0xxxxxxxxx

t=3780.0 ps, $r12 = 0xxxxxxxxx  ||   $r28 = 0xxxxxxxxx

t=3790.0 ps, $r13 = 0xxxxxxxxx  ||   $r29 = 0x000003fc

t=3800.0 ps, $r14 = 0x5a5a3c3c  ||   $r30 = 0xxxxxxxxx

t=3810.0 ps, $r15 = 0x10010100  ||   $r31 = 0x00000000

Memory Locations 0xC0 to 0xFF

t=3820.0 ps, M[000000c0] = 0x3fffc000

t=3830.0 ps, M[000000c4] = 0x0ffff000

t=3840.0 ps, M[000000c8] = 0x03fffc00

t=3850.0 ps, M[000000cc] = 0x00ffff00

t=3860.0 ps, M[000000d0] = 0x003fffc0

t=3870.0 ps, M[000000d4] = 0x000ffff0

t=3880.0 ps, M[000000d8] = 0x0003fffc

t=3890.0 ps, M[000000dc] = 0x0000ffff

t=3900.0 ps, M[000000e0] = 0xc0003fff

t=3910.0 ps, M[000000e4] = 0xf0000fff

t=3920.0 ps, M[000000e8] = 0xfc0003ff

t=3930.0 ps, M[000000ec] = 0xff0000ff

t=3940.0 ps, M[000000f0] = 0xffc0003f

t=3950.0 ps, M[000000f4] = 0xfff0000f

t=3960.0 ps, M[000000f8] = 0xfffc0003

t=3970.0 ps, M[000000fc] = 0xffff0000

t=3980.0 ps, PC = 0x0040003c,   IR = 0x0000000d